3GPP TSG-RAN4 Meeting #114bis	R4-2503256

# Wuhan, China 7th – 11th April, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM requirements for low band carrier aggregation via switching

Agenda item:	7.7.3

Document for:	Discussion

# Introduction

3GPP introduced a new work item on low band carrier aggregation via switching in RAN #106 [1]. We discuss some relevant RRM requirements for this topic.

# Past Agreements

RAN4 RRM session discussed this topic during the past meeting. The agreements and open issues are shown below:

- Switch delay requirements Agreement: - The switching delay between operating the FDD and SDL carrier is to be discussed in the RF session. RAN4 RRM not to define additional switch delay in RRM. - Synchronization requirements for FDD and synchronization accuracy at BS antenna connectors or radiated interface boundaries Agreement: - No need to define new cell phase synchronization requirement in RRM spec for R19 LB CA via switching. - SDL SCC related RRM requirement - activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL activated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used. - de-activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL deactivated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL deactivated SCell L3 measurement requirement if SSB based SCell operation is used. - other SDL SCell requirements Agreement: - FFS: Whether and how - RAN4 to define a new SDL SCell activation/deactivation delay and related interruption for R19 LB CA - the existing SCell activation/deactivation delay and related interruption requirement can be used as baseline for R19 LB CA - FFS: Whether and how - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA Agreement: - Not to change definition of intra-frequency/inter-frequency measurement with/without gap measurement - FFS on the use case for MG application. - Potential impacts to the PCell related requirement - FFS - FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Agreement: - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting. - (2) FFS: MRTD between the two low bands is CP? Or wait for TAE conclusion from RF session? Agreement: As generic assumption for discussing the RRM requirement, the received timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session. - Option 1: 4.67us - Option 2: 3 us

# Discussion

According to [2], low band spectrum carries significant traffic volume in urban indoor and rural areas. However, SDL remain under-utilized since they cannot be paired with UL. That’s why, [2] proposed to introduce a network configured switching pattern in low band carrier aggregation via switching

Figure 1: RRC configured switching pattern in low band carrier aggregation via switching [2]

Figure 1 shows how network could configure this switching pattern to enable this low band carrier aggregation. This switching pattern would instruct UE to alternate between FDD DL+UL band and SDL band in a periodic manner. We discuss different RRM requirements of SCells and PCells in this scenario.

## SCell Activation Delay and Deactivated SCell Measurement Requirement

RAN4 can consider any or multiple of the following three approaches regarding SCell addition and measurement of deactivated SCells.

- Approach 1: Network adds SCell of the SDL band prior to activation. Spec requires deactivated UE to measure SCell.

- Approach 2: Spec does not allow network to add SCell of the SDL band prior to activation. SCell is added and activated at the same time. SCell is also deactivated and released at the same time.

- Approach 3: Spec allows network to add SCell prior to activation and deactivate SCell prior to releasing it. However, spec does not require deactivated UE to measure the SCell in the SDL band.

In approach 1, spec allows network the flexibility to add SCell prior to activation. UE is required to measure the added but deactivated SCell. Since UE measures deactivated SCell, the SCell is expected to be “known” when network plans to activate it. However, since UE cannot simultaneously operate in FDD and SDL band in this scenario, UE can periodically measure the deactivated SCell of SDL band by switching away from the FDD band.  This can be achieved by generating a network configured switching pattern. This periodic switch to measure deactivated SCell would lead to throughput loss during the time when SCell is added but deactivated.

Observation 1: Measuring added but deactivated SCell increases the probability of the SCell to be known during next SCell activation. This, in turn, potentially reduces the SCell activation time.

Observation 2: Measurement of deactivated SCell in LB-LB CA via switching scenario leads to throughput loss since UE cannot simultaneously communicate in the FDD and SDL band.

In legacy scenarios, network adds SCell before activating it because network wants to activate a SCell quickly based on dynamic traffic pattern. In LB-LB CA via switching scenario, UE’s individual data rate does not increase due to the addition of SCell since UE cannot operate simultaneously in SDL and FDD band. The SCell in the SDL band is enabled to share network DL load among different users of the cell. The switching between FDD and SDL band is enabled to ensure uplink communication of each UE. Hence, the need to add SCell prior to activation is not as essential as legacy scenario and that’s what is captured in approach 2. However, it might be good to allow SCell addition prior to activation and release after deactivation to give flexibility to network.

Figure 2: UE following switching pattern to measure SSB and CSI-RS of SCell during SCell activation

Observation 3: In legacy scenarios, network adds SCell before activating it because network wants to activate a SCell quickly based on dynamic traffic pattern.

Observation 4: LB-LB CA via switching was not introduced to increase individual UE’s throughput. Rather, it was introduced to balance DL load properly among different UEs in the cell while ensuring that each UE gets the chance to communicate in UL.

Observation 5: Allowing SCell addition prior to activation and release after deactivation allows network flexibility.

That’s why, approach 3 allows network to add SCell prior to activation and deactivate SCell prior to release. However, approach 3 does not allow deactivated SCell measurement to reduce throughput loss.

Proposal 1: RAN4 does not define any requirement for deactivated SCell measurement in LB-LB CA scenario to reduce throughput loss.

As mentioned before, the absence of deactivated SCell measurement can potentially increase the SCell activation timeline since SCell could be “unknown” during activation timeline. During legacy “unknown” FR1 SCell activation, UE needs to measure multiple SSB samples of SCell for following purposes:

- Coarse AGC acquisition

- Fine AGC acquisition

- SSB index detection

- Fine time/frequency acquisition

Since FDD and SDL cells are co-located and synchronous in LB-LB CA via switching scenario, UE can use some properties of PCell measurement to estimate the corresponding properties of SCell during SCell activation. This would be possible if MRTD and reception power difference between the cells remain small.

RAN4 has already started discussing MRTD between FDD and SDL cells. reception power difference between the cells needs to be contained within a threshold, too. Note that, the guarantee of small MRTD and reception power difference should not tie this feature with SSB-less SCell. In other words, a UE that does not support SSB-less SCell feature should still be allowed to support LB-LB CA via Switching feature.

Proposal 2: Spec guarantees small MRTD and reception power difference between FDD and SDL cells in LB-LB CA via switching scenario so that UE can use the measurement of FDD PCell to reduce the activation timeline of SDL SCell that it has not measured recently.

- Note: Support of LB-LB CA via switching feature does not depend on “SSB-less SCell” feature. Above spec guarantee for small MRTD and RPD difference are needed just to reduce SCell activation time for UEs.

Figure 3: UE switching to SCell at pre-defined “essential” locations to measure SSB+CSI-RS of SCell during SCell activation

Proposal 3: RAN4 confirms that the reception power difference between the FDD PCell and SDL SCell at the UE is smaller than or equal to X dB.

- X is FFS.

UE can measure SCell’s SSB during SCell activation using different methods. The methods are mentioned below:

- Method 1:

  - UE follows the network configured switching pattern to measure SCell’s SSBs and CSI-RS resources during SCell activation.

- Method 2:

  - During SCell activation, UE switches to SCell only when it is necessary to measure SCell’s SSBs and CSI-RS resources.

  - Spec allows interruption and scheduling restriction in FDD PCell based on SSB/CSI-RS locations in the time domain.

  - UE follows the network configured switching pattern between FDD PCell and SDL SCell after sending CSI report corresponding to the SCell.

Figure 2 and Figure 3 show the procedures of method 1 and method 2.

Method 1 requires UE to use network configured switching pattern to stay in SCell before sending CSI report corresponding to the SCell although UE won’t be able to communicate via SCell in that scenario. Unnecessary throughput loss occurs since UE stays in SCell to follow the switching pattern during SCell activation timeline.

Method 2 requires UE to stay in SCell only at “essential” time locations. Hence, higher throughput can be achieved with method 2 during SCell activation timeline.

Observation 6: UE suffers from throughput loss if it follows network configured switching pattern to measure SSB and CSI-RS of SDL SCell during SCell activation.

Observation 7: UE cannot communicate PDSCH/PUSCH via SDL SCell until it sends CSI report corresponding to that SCell.

Observation 8: Minimum throughput loss would be observed if UE switches to SDL SCell only to measure SSB and CSI-RS instead of following the switching pattern during SCell activation of LB-LB CA via switching scenario.

Figure 4: A reference switching pattern {6, 8, 6, 6, 8, 6}

Proposal 4:

  - During SCell activation, UE switches to SCell only when it is necessary to measure SCell’s SSBs and CSI-RS resources.

  - Spec allows interruption and scheduling restriction in FDD PCell based on SSB/CSI-RS locations in the time domain.

  - UE follows the network configured switching pattern between FDD PCell and SDL SCell after sending CSI report corresponding to the SCell.

## L1 and L3 Measurement of Activated SCells

During the last meeting, several companies proposed to define L1 and L3 measurement requirements of SCells.

Network adds these SCells to balance DL load among different users in the cell. The WID guarantees that the SDL SCells are co-located and synchronous with the FDD PCells. Hence, measuring SDL SCell of NCell is not necessary. Network can hand over UE to neighbor cell based on FDD PCell measurements and then add NCell’s SDL SCell to the UE.

Observation 9: SDL SCells are co-located and synchronous with the FDD PCells. Hence, network can hand over UE to a neighbor cell simply based on FDD PCell measurements and then add NCell’s SDL SCell to the UE.

Proposal 5: RAN4 does not define L3 measurement requirements for SDL SCells in LB-LB CA via switching scenario.

RAN4 legacy spec defines requirements for following two types of L1 measurements.

- L1-RSRP measurement

- Beam failure detection (BFD) and candidate beam detection (CBD) measurements

From a theoretical perspective, BFD and CBD related measurements are allowed in SDL SCell in LB-LB CA via switching scenario. On the other hand, both FDD PCell and SDL SCells are guaranteed to operate in less than 1 GHz band. In practical deployments, BFD and CBD get configured only in the higher frequency region ( >3 GHz) of FR1. Hence, it is not essential to define BFD and CBD requirements for SCells and RAN4 can save time by not defining these requirements.

Observation 10: Both FDD PCell and SDL SCells are guaranteed to operate in less than 1 GHz band. In practical deployments, BFD and CBD get configured only in the higher frequency region ( >3 GHz) of FR1.

Proposal 6: RAN4 does not define BFD and CBD measurement requirements for SDL SCells in LB-LB CA via switching scenario.

Observation 11: RAN4 can consider extending the L1-RSRP measurement periods of SCell to accommodate the switching pattern.

## Measurement of PCells

PCell measurement opportunities would be impacted due to the switching pattern. Figure 4 shows a reference switching pattern. Assuming that the SSBs of PCell and SCell overlap in time and are configured with 20ms periodicity, UE will miss every other SSB of PCell and SCell due to the reference switching pattern shown in Figure 4. Hence, PCell measurement periods need to be extended to accommodate the switching pattern at the UE.

Proposal 7: RAN4 extends all relevant measurement periods of PCell to accommodate switching pattern in LB-LB CA via switching.

# Conclusions

Observation 1: Measuring added but deactivated SCell increases the probability of the SCell to be known during next SCell activation. This, in turn, potentially reduces the SCell activation time.

Observation 2: Measurement of deactivated SCell in LB-LB CA via switching scenario leads to throughput loss since UE cannot simultaneously communicate in the FDD and SDL band.

Observation 3: In legacy scenarios, network adds SCell before activating it because network wants to activate a SCell quickly based on dynamic traffic pattern.

Observation 4: LB-LB CA via switching was not introduced to increase individual UE’s throughput. Rather, it was introduced to balance DL load properly among different UEs in the cell while ensuring that each UE gets the chance to communicate in UL.

Observation 5: Allowing SCell addition prior to activation and release after deactivation allows network flexibility.

Observation 6: UE suffers from throughput loss if it follows network configured switching pattern to measure SSB and CSI-RS of SDL SCell during SCell activation.

Observation 7: UE cannot communicate PDSCH/PUSCH via SDL SCell until it sends CSI report corresponding to that SCell.

Observation 8: Minimum throughput loss would be observed if UE switches to SDL SCell only to measure SSB and CSI-RS instead of following the switching pattern during SCell activation of LB-LB CA via switching scenario.

Observation 9: SDL SCells are co-located and synchronous with the FDD PCells. Hence, network can hand over UE to a neighbor cell simply based on FDD PCell measurements and then add NCell’s SDL SCell to the UE.

Observation 10: Both FDD PCell and SDL SCells are guaranteed to operate in less than 1 GHz band. In practical deployments, BFD and CBD get configured only in the higher frequency region ( >3 GHz) of FR1.

Observation 11: RAN4 can consider extending the L1-RSRP measurement periods of SCell to accommodate the switching pattern.

Proposal 1: RAN4 does not define any requirement for deactivated SCell measurement in LB-LB CA scenario to reduce throughput loss.

Proposal 2: Spec guarantees small MRTD and reception power difference between FDD and SDL cells in LB-LB CA via switching scenario so that UE can use the measurement of FDD PCell to reduce the activation timeline of SDL SCell that it has not measured recently.

- Note: Support of LB-LB CA via switching feature does not depend on “SSB-less SCell” feature. Above spec guarantee for small MRTD and RPD differences are needed just to reduce SCell activation time for UEs.

Proposal 3: RAN4 confirms that the reception power difference between the FDD PCell and SDL SCell at the UE is smaller than or equal to X dB.

- X is FFS.

Proposal 4:

  - During SCell activation, UE switches to SCell only when it is necessary to measure SCell’s SSBs and CSI-RS resources.

  - Spec allows interruption and scheduling restriction in FDD PCell based on SSB/CSI-RS locations in the time domain.

  - UE follows the network configured switching pattern between FDD PCell and SDL SCell after sending CSI report corresponding to the SCell.

Proposal 5: RAN4 does not define L3 measurement requirements for SDL SCells in LB-LB CA via switching scenario.

Proposal 6: RAN4 does not define BFD and CBD measurement requirements for SDL SCells in LB-LB CA via switching scenario.

Proposal 7: RAN4 extends all relevant measurement periods of PCell to accommodate switching pattern in LB-LB CA via switching.

# References

[1] RP-243317, New WID on low band carrier aggregation via switching, 3GPP TSG RAN #106.

[2] RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19, 3GPP TSG RAN #106.

[3] R4-2502588, WF on RRM requirements for NR_LBCA_Sw, 3GPP TSG-RAN WG4 Meeting#114.