3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2503257

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.19.3

Source: 	National Taiwan University

Title: 	On DL Tx Hierarchy Beamforming Codebook Design and the Prediction Impact

Document for:	Approval

Introduction

We present our view on partially specified decoder and reference decoder derivation procedure in this contribution.

Discussion

Discussion on Tx Beamforming Codebook

Two sub-options are listed in the previous meeting WF:

In simulation assumptions: Option 1a [corresponding to BS antenna configurations option 1]: 32 beams with grid of 4 elevation angles from [-25 to 25] and 8 azimuth angles from [-60 to 60] Option 1b [corresponding to BS antenna configurations option 1]: The codebook construction can be clarified by companies. In Testing for one sided models: Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

We can observe that the simulation assumptions for Set B beams are missing. In most of the commercial mmWave BS beamforming codebook design, hierarchical codebook with wide and narrow beams are used. From beam prediction perspective, with the same number of beams in Set B, wider spatial domain coverage is better for prediction accuracy, especially in fading scenarios, otherwise UE may fail to predict best beams if all least fading angle is out of Set B beam coverage, and the stronger angles from the dataset are in deep fading. Therefore, transmitting wide beams in Set B and leave most of the narrow beams in Set A is a better design for beam prediction than all beams with uniform beam width, i.e., non-hierarchical codebook. Unfortunately, both option 1a and 1b are codebook with uniform beam-width.

Observation 1: Simulation assumptions for Set B beams are missing from the previous agreement

Therefore, we propose the hierarchical codebook based on the simulation setup in [1]:

Wide beams: (M, N)=(2,4) for BS antenna configuration, 8 beams with grid of 2 elevation angles from [-25 to 25] and 4 azimuth angles from [-60 to 60].

Narrow beams:  (M, N)=(4,8) for BS antenna configuration, 32 beams with grid of 4 elevation angles from [-25 to 25] and 8 azimuth angles from [-60 to 60].

Proposal 1: Use the following BS antenna configurations and beamforming patterns:

| BS antenna configurations | - Set A beams - One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline. - Number of Tx beams is 32 - Set B beams - One panel: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline. - Number of Tx beams is 8 |
| --- | --- |
| Beamforming characteristic of the BS pattern | Set A beams: 32 beams with grid of 4 elevation angles from [-25 to 25] and 8 azimuth angles from [-60 to 60] Set B beams: 8 beams with grid of 2 elevation angles from [-25 to 25] and 4 azimuth angles from [-60 to 60] |

Conclusion

Observation 1: Simulation assumptions for Set B beams are missing from the previous agreement.

Proposal 1: Use the following BS antenna configurations and beamforming patterns:

| BS antenna configurations | - Set A beams - One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline. - Number of Tx beams is 32 - Set B beams - One panel: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline. - Number of Tx beams is 8 |
| --- | --- |
| Beamforming characteristic of the BS pattern | Set A beams: 32 beams with grid of 4 elevation angles from [-25 to 25] and 8 azimuth angles from [-60 to 60] Set B beams: 8 beams with grid of 2 elevation angles from [-25 to 25] and 4 azimuth angles from [-60 to 60] |

Reference

[1] C.-H. Huang, Y. Kim, and S.-C. Lin, “Universal Beam Prediction for Mismatched Beam Codebooks,” to be submitted to IEEE Globecom 2025.