3GPP TSG-RAN WG4 Meeting #114bis	R4-2503262

Wuhan, Hubei, China, 07th – 11th April 2025

Source:	China Telecom

Title:	Discussion on initial BWP size restriction to 5 MHz for 7MHz Channel Bandwidth

Agenda Item:	7.8.2

Document for:	Discussion

# Introduction

In RAN4#114, open issues on 7 MHz Channel Bandwidth have been discussed first time and the WF has been approved[1]. This contribution will provide our consideration on the release independence issue.

# Discussion

Last meeting, it was agreed that the support of 7 MHz channel bandwidth could be released independently from Rel-15 and would remain optional in Rel-19 as well as in previous releases. However, concerns have been raised regarding the initial BWP size. For instance, if the gNB broadcasts an initial BWP size of 7 MHz, legacy UEs supporting only 5 MHz would not be able to access the network due to their lack of support for 7 MHz.

It is important to highlight that the support for 7 MHz is optional in prior releases. This implies that the UE must explicitly indicate its support for 7 MHz through UE capability signalling. The initial BWP is utilized during the initial access procedure. At this stage, the network cannot determine whether the UE supports 7 MHz. Therefore, it is logical that before UE enters the RRC Connected mode, the network will only employ mandatory features to ensure all UEs can successfully access the network.

Observation 1: The support of 7MHz is optional for Rel-19 and the previous releases, it’s reasonable that the network shall use the channel bandwidth which is mandatory supported in deployment.

Proposal 1: From a specification perspective, there should be no restriction on the initial BWP size. This is more of a deployment-related issue.

As agreed upon by RAN4, the channel bandwidth indicated by SIB1 can encompass any number of PRBs. Given that the initial BWP must be contained within the channel bandwidth specified in SIB1, it follows that there should also be no restriction on the initial BWP size. Additionally, from a signalling perspective, the only limitation on BWP size is that it should not exceed the maximum allowable value.

Observation 2: The channel bandwidth indicated in SIB1 can take any value, and consequently, so can the initial BWP size. 
Proposal 2: The initial BWP size can assume any value between 0 and the maximum allowable value.

# Conclusion

Observation 1: The support of 7MHz is optional for Rel-19 and the previous releases, it’s reasonable that the network shall use the channel bandwidth which is mandatory supported in deployment.

Proposal 1: From a specification perspective, there should be no restriction on the initial BWP size. This is more of a deployment-related issue.

Observation 2: The channel bandwidth indicated in SIB1 can take any value, and consequently, so can the initial BWP size. 
Proposal 2: The initial BWP size can assume any value between 0 and the maximum allowable value.

# References

R4-2502870, WF on requirements for 7MHz channel bandwidth, Ericsson, RAN4#114, Feb 2025