3GPP TSG-RAN WG4 # 114-bis	R4-2503267

Wuhan, China, 7th Apr 2025 – 11th Apr 2025

Agenda item:	7.16.2

Source:	MediaTek Inc.

Title:	Discussion on Spatial Channel Modelling

Document for:	Discussion

# Introduction

The Rel-19 study item [1] aims to investigate practical spatial channel modelling methodologies for both SU-MIMO and MU-MIMO demodulation and CSI reporting requirements. It has been identified that the current TDL-based models and spatial correlation matrices poorly match with realistic propagation conditions that have been used as the design assumption for the NR downlink precoder (PMI) codebooks. We also believe that the test scenarios and channel models should match with realistic transmit strategies and channel conditions in order to correctly support UE algorithm design. When RAN4 test cases are unrealistic, the UE algorithm designers may need to compromise between real-world behavior/performance and RAN4 tests.

In our view, the problem with the existing RAN4 spatial correlation models is that they amplify the signal only in the broadside direction (AOD/AOA) of the TX and RX antenna arrays. In order to support a higher number of layers, multiple separate spatial signal directions should exist. This can be achieved by introducing spatial clusters, where each cluster has its own separable mean AOD/AOA.

The current RAN4 MIMO tests employ TDL (Tapped Delay Line) channel models with specific spatial correlation matrices. For spatial channel model development, both CDL (Clustered Delay Line) and TDL models can be considered as baseline.

# Discussion

# 2.1 Discussion of common issues for all methodologies

In this section, we share our views of common issues for all methodologies.

We have noticed that implementing the CDL model is not trivial as the many details of the model are scattered between multiple TRs and their many sections. Furthermore, the discussion in RAN4 (during the past seven months) has shown that the text in both TR 38.901 and TR 38.827 is ambiguous, leaving room for different interpretations. Therefore, we should improve the documentation quality and provide a more focused and self-contained description of the proposed link-level model.

Proposal #1: The upcoming TR (TR 38.753) should provide self-contained, unambiguous, exact, and focused descriptions of the agreed channel models.

Metrics for comparison

Proposals:

- Option 1: RAN4 to discuss test metrics to compare the SCM methodologies.

- Option 2: The metric of PDSCH cases could be throughput or condition number, i.e, ratio of the absolute value of the maximum eigenvalue to the absolute value of the minimum eigenvalue of the channel.

It has turned out that result alignment in CDL channels between companies (in terms of tput performance) with different AAV options is very difficult. Therefore, we propose to carry out initial alignment via simpler alternative metrics that omit all uncertainty resulting from different receiver algorithms implementations. These alternative metrics should be based on measured channel statistics only. For example, the following measures can be recorded and cross-checked between companies:

- Spatial domain / multiplexing: Average eigenmode powers, sorted from largest to smallest

- Spatial domain / directionality: Power angular distribution (PAD) of the channel in TX and RX dimensions

- Time domain: Doppler spectrum, or more simply time coherence function (measured as channel correlation between symbols or slots)

- Frequency domain: Frequency coherence function (measured as channel correlation between subcarriers or PRBs)

Note that we already share measured average eigenmode powers for the different AAV options in simulation result document [3].

Proposal #2: We propose following measures to be recorded and cross-checked between interested companies

- Spatial domain/multiplexing: Average eigenmode powers, sorted from largest to smallest

- Spatial domain/directionality: Power angular distribution (PAD) of the channel in TX and RX dimensions

- Time domain: Doppler spectrum, or more simply time coherence function

- Frequency domain: Frequency coherence function

PDSCH PMI Selection

Way Forward:

Random PMI already agreed. Regarding potential fixed PMI:

- Option 1: Random PMI from sub-set

- Option 2: Collectively chosen fixed PMI

- Option 3: Independently chosen fixed PMI

Note: Companies to propose detailed method description for interested options for RAN4#114-bis.

We believe that it is impossible to align fixed PMI simulation results if all companies choose different PMIs. Therefore, we propose that collective chosen fixed PMI is applied.

Proposal #3: We propose that collective chosen fixed PMI is applied for fixed PMI testing.

# 2.2 PDSCH demodulation and PMI follow testing in CDL channels

We have simulated PDSCH demodulation performance with random and fixed PMI. Results are available in separate simulation result document [3]. We can make the following observations.

Observation #1: CDL-C UMa with both AAV options are feasible with 4T4R and random precoding.

Observation #2: CDL-C UMa with both AAV options are feasible with 8T8R and best fixed Type1 precoding.

Observation #3: CDL-C UMa with both AAV options is challenging with 8T8R and random precoding.

Proposal #4: We propose to focus on checking simulation results alignment in the next meeting.

# 2.3 Discussion of CDL-C UMa FR1 channel model

In this section we discuss CDL channel model related issues.

Angles of Departure and Angles of Arrival

Way Forward:

For all 4 direction combinations (AoA, AoD, azimuth, zenith) the following parameters need to be configured.

Angle spread scaling: ASdesired

Angle rotation: μϕ,desired

as a starting point.

For CDL Option A1: Based on 38.827

For CDL Option A2, for each (AoA, AoD) and θ,ϕ, the parameters in the formula are the following:

ASmodel is computed according to TR 38.901, Annex A.1, Equation A-1

μϕ,model  is computed according to TR 38.901, Annex A.2, Equation A-2

For the purposes of simulation alignment:

μϕ,desired =  μϕ,model

ASdesired

|  | ASdesired, ASD | ASdesired, ASA | ASdesired, ZSD | ASdesired, ZSA |
| --- | --- | --- | --- | --- |
| CDL Option A1 | 25.7620 | 74.1138 | 4.8978 | 18.2050 |
| CDL Option A2 | 25.7620 | 74.1138 | 4.8978 | 18.2050 |
| CDL Option A3 | 25.7620 | 74.1138 | 4.8978 | 18.2050 |

As a starting point: cASA,ASD,ZSD,ZSA according to the table below

| Per-Cluster Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value | 2 | 15 | 3 | 7 | 7 |

We have identified certain issues in the angle scaling procedure agreed during/after RAN4#114 that we wish to point out.

One of our observations is that the AS scaling procedure defined in WF [2] is different from the procedure specified in TR 38.827 Section 7.2. More specifically, the definition of angular spread (AS) is different. TR 38.827 applies equations as shown in Figure 1, while WF proposed equation A-1 from TR 38.901 shown in Figure 2. Due to the different definitions of AS, the channel models obtained via angle scaling will be different.

Observation #4: Angle scaling procedure proposed/agreed in WF differs from what TR 38.827 applies. The resulting scaled channel models therefore differ as well.

Figure 1: AS definition applied in TR 38.827

Figure 2: AS definition proposed in WF [2].

Another observation is that the AS scaling procedure agreed/proposed in WF [2] approximately achieves the desired AS. This is due to the approximative nature of the AS definition itself. In Table 1, we list the original AS (ASmodel), the target AS (ASdesired), and the achieved AS (ASachieved) to analyse the scaling from CDL-C model 901 to A2. Note that we applied here the unquantized angles of A2 for the comparison.

Observation #5: Angle scaling procedure proposed/agreed in WF is approximative. It approximately (but not exactly) achieves the desired AS.

Table 1: AS scaling from 901 to A2

|  | ASmodel | ASdesired | ASachieved |
| --- | --- | --- | --- |
| ASD | 37.4036 | 25.7620 | 26.4161 |
| ASA | 71.4536 | 74.1138 | 74.0806 |
| ZSD | 4.0665 | 4.8978 | 4.8977 |
| ZSA | 10.4183 | 18.2050 | 18.1818 |

One more observation about the AS scaling procedure is that it is affected by how angle wrapping to keep angles between -180 and +180 degrees is applied. This holds especially for AOA where the AS is so large that angle wrapping has effect. Furthermore, any method for angle wrapping is not specified in the currently agreed equation:

.

Observation #6: The CDL angle scaling result is affected by how angles are wrapped (or not wrapped) to stay between -180 and +180. Wrapping rules are not specified in the currently agreed equations.

As a side note, we would like to point out that for large angular spreads, when the clusters are located in all physical directions between -180 and +180, the definition of mean angle becomes less meaningful. Furthermore, angle scaling itself becomes ambiguous, and the AS after scaling might actually move into undesired direction due to angle wrapping.

We propose to explicitly specify how to apply wrapping to avoid ambiguity and different interpretations. Furthermore, we propose to apply the scaling not to the ray-angles, but to the cluster angles that are tabulated. This is because when scaling/wrapping is applied per ray, the rays of one cluster may start deviating from each other.

Proposal #5: For CDL angle scaling, we propose to explicitly specify angle wrapping rules. Furthermore, we propose to apply scaling to the cluster-angles instead of the ray-angles as shown in equations (1) and (2) below.

ϕn,AOA,scaled=wrapASdesiredASmodel∙wrapϕn,AOA-μϕ,model+μϕ,desired	(1)

CASA,scaled=ASdesiredASmodel∙CASA,scaled						(2)

Here, function y = wrap(x) makes y to fall between -180 and +180 degrees so that y = (x+180) mod 360 - 180. Note that this is the scaling/wrapping rule we used to exactly obtain a match with the recently agreed A2 model.

CDL Simplification

Proposals:

- Option 1 RAN4 to study if the CDL channel models can be simplified to smaller number of clusters.

- Option 2: RAN4 to discuss whether the existing simplification procedure in 38.901 Annex B.2.1 is applicable to SCM, and if not how to extend the existing simplification methodology to reduce the number of channel taps for CDL channels based on 38.901.

Proposal #6: We support CDL simplification procedure as depicted in Figure 3.

Figure 3: Proposed angle scaling and cluster reduction procedure.

SNR Definition with CDL

Way Forward:

Options:

- Option 1: Interested companies to provide information on long-term pre-EQ power measurements, for discussion whether a formula, or value, is required during RAN4#114-bis.

- Option 2: Define a normalisation method

  - Option 2a:

RAN4 to use following as normalization factor: C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2} Where H(k,t)∈CNR×NT is channel coefficient at subcarrier k, OFDM symbol l. NRx is number of Rx and NTx is number of Tx ports. Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation.

  - Option 2b:

Let normalized AAV beamformer vector of length Ms∙Ns (i.e. sub-array size) for one antenna port be α1,1⋯αMs,Ns=1GBF1Ms∙Nsexp-j2π∙θ1,1 ⋯ exp⁡(-j2π∙θMs,Ns) Here, amplitude normalization is separated into two factors - 1Ms∙Ns : split transmit power of each AP evenly over Ms∙Ns transmit antenna elements - 1GBF : normalize with measured transmit beamforming (power) gain GBF

The channel power measurement result depends on the transmit power of the AAV beamformer, which has not been specified by RAN4. We propose to initially set the AAV beamformer power so that the total power over all antenna elements and AAV beams becomes one as in Option 2b. The further SNR adjustment can be done based on the measurement defined as in Option 2a.

Observation #7: Related to SNR definition, Option 2a and Option 2b are connected so that TX amplitude normalization factor C = 1/sqrt(GBF), where C and GBF are scalar values.

Proposal #7: We propose to formally define the AAV beamformer to facilitate SNR alignment as in Option 2b.

Proposal #8: We propose to normalize total average channel power to unity in simulations and document the normalization factor C with different channel model and AAV options.

Reduction of Randomness

Proposals:

- Option 1: All CDL based approaches to use the 827 randomness reduction framework, i.e., small scale fading changes, fixed subpaths, weak clusters, etc.

We agree that TR 38.827 randomness reduction framework is needed in TR 38.753 to enable result alignment.

Proposal #9: We support TR 38.827 randomness reduction framework to be included in CDL models of TR 38.753.

# 2.3.1 Ray-splitting

One feature of the CDL models is so-called sub-clusters that involve cluster-splitting and ray-splitting as specified in Table 7.5-5 of TR 38.901. We would like to define these two terms just to clarify our meaning:

- Cluster-splitting: splitting one strong cluster into three sub-clusters with different delays and powers.

- Ray-splitting: splitting the 20 rays between the three sub-clusters so that the first gets 10, the second 6, and the third gets 4 rays.

The motivation for ray-splitting is to avoid fading correlation between the sub-clusters. According to the recent email discussion, there is some disagreement between different companies whether ray-splitting needs to be done in the context of link-level CDL models or not. In our view, this detail cannot be left open for interpretation.

The issue of sub-cluster ray-splitting was already discussed and agreed in RAN4 #113 in November 2024 as documented in WF (R4-2419782) and shown below. Furthermore, the motivation and necessity of ray-splitting was explained in R4-2418042 by Keysight.

We agree that TR 38.901 might be ambiguous related to sub-clusters as shown by the text in Section 7.7.1 Step 4:

Our interpretation is that this text recognizes that the link-level CDL model tables already list all clusters, and further cluster-splitting should not be done. However, the issue of ray-splitting is not mentioned. In our view, the cluster correlation problem does not disappear from link-level models, and it thus needs to be addressed.

Ray-splitting is specified in TR 38.901 and further enforced in TR 38.827 Section 7.2 where it says that “the problematic clusters are treated as midpaths”. Therefore, we can only assume that ray-splitting is meaningful, and the problem is the same in both system-level and link-level models.

Proposal #10: We propose to specify and apply ray-splitting for sub-clusters of CDL models both in this study item and in TR 38.753.

We recognize that ray-splitting conceptually complicates the implementation of CDL channel models. Therefore, we are open to consider alternative means to avoid sub-cluster correlation. One simple method is to add small angular offsets to the AOA/AOD of the sub-clusters. This will (similarly to ray-splitting) ensure that the set of Doppler shifts will not be identical for all three sub-clusters.

Proposal #11: We propose RAN4 to consider simpler alternatives to ray-splitting to avoid sub-cluster correlation, for example small sub-cluster angular offsets.

# 2.4 Discussion of TDL channels

In this section we discuss general issues of TDL channels.

TDL Model Extensions

Way Forward:

Companies to use following two enhanced TDL channel for study.

- Option 1: Multi-cluster TDL channel with implementation details in R4-2500337:

  - For proposed simulation configurations see Annex C, Table C.1 and Table C.2.

  - For cluster powers settings there are several examples simulated in R4-2500335. We suggest trying 2 different setups

    - Un-even layer powers and even CW-powers.

    - Even layer powers and un-even CW-powers.

- Option 2: Multi-beams TDL channel with implementation details in R4-2501738:

  - 4T4R,4Layers: Enhanced TDLC300-100, XPL, high with 2 Tx-Rx beams as defined in Table C.3

  - PMI test: Enhanced TDLC300-100, XPL, high with 2 Tx-Rx beams as defined in Table C.3

  - 8T8R, 8Layers: Enhanced TDLC300-100, XPL, high with 4 Tx-Rx beams as defined in Table C.4 and 6 Tx-Rx beams as defined in Table C.5

We have simulated both TDL extensions as shown in our simulation results [3].

For a detailed harmonized description of both multi-cluster TDL models see [4].

Proposal #12: We propose interested companies to contribute to multi-cluster TDL simulation alignment.

# Conclusion

In this paper we provided the view on the spatial channel modelling. The following observations and proposals are made:

Proposal #1: The upcoming TR (TR 38.753) should provide self-contained, unambiguous, exact, and focused descriptions of the agreed channel models.

Proposal #2: We propose following measures to be recorded and cross-checked between interested companies

- Spatial domain/multiplexing: Average eigenmode powers, sorted from largest to smallest

- Spatial domain/directionality: Power angular distribution (PAD) of the channel in TX and RX dimensions

- Time domain: Doppler spectrum, or more simply time coherence function

- Frequency domain: Frequency coherence function

Proposal #3: We propose that collective chosen fixed PMI is applied for fixed PMI testing.

Observation #1: CDL-C UMa with both AAV options are feasible with 4T4R and random precoding.

Observation #2: CDL-C UMa with both AAV options are feasible with 8T8R and best fixed Type1 precoding.

Observation #3: CDL-C UMa with both AAV options is challenging with 8T8R and random precoding.

Proposal #4: We propose to focus on checking simulation results alignment in the next meeting.

Observation #4: Angle scaling procedure proposed/agreed in WF differs from what TR 38.827 applies. The resulting scaled channel models therefore differ as well.

Observation #5: Angle scaling procedure proposed/agreed in WF is approximative. It approximately (but not exactly) achieves the desired AS.

Observation #6: The CDL angle scaling result is affected by how angles are wrapped (or not wrapped) to stay between -180 and +180. Wrapping rules are not specified in the currently agreed equations.

Proposal #5: For CDL angle scaling, we propose to explicitly specify angle wrapping rules. Furthermore, we propose to apply scaling to the cluster-angles instead of the ray-angles as shown in equations (1) and (2) below.

Proposal #6: We support CDL simplification procedure as depicted in Figure 3.

Observation #7: Related to SNR definition, Option 2a and Option 2b are connected so that TX amplitude normalization factor C = 1/sqrt(GBF), where C and GBF are scalar values.

Proposal #7: We propose to formally define the AAV beamformer to facilitate SNR alignment as in Option 2b.

Proposal #8: We propose to normalize total average channel power to unity in simulations and document the normalization factor C with different channel model and AAV options.

Proposal #9: We support TR 38.827 randomness reduction framework to be included in CDL models of TR 38.753.

Proposal #10: We propose to specify and apply ray-splitting for sub-clusters of CDL models both in this study item and in TR 38.753.

Proposal #11: We propose RAN4 to consider simpler alternatives to ray-splitting to avoid sub-cluster correlation, for example small sub-cluster angular offsets.

Proposal #12: We propose interested companies to contribute to multi-cluster TDL simulation alignment.

# Reference

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- R4-2502378, “Way Forward for [114][320] NR_SCM”, Nokia

- R4-2503268, “Simulation results of spatial channel modelling”, MediaTek Inc

- R4-2503269, “TP to TR38.753: TDL approaches and related Annex”, MediaTek