3GPP TSG-RAN WG4 Meeting #114bis	R4-2503270

Wuhan, China, 7th Apr 2025 – 11th Apr 2025

Agenda item:	7.19.2

Source:	MediaTek Inc.

Title:	Discussion on CSI reporting requirement framework for CSI prediction

Document for:	Discussion

# Introduction

In RAN4#114, RAN4 agreed to continue discussion on CSI reporting requirement framework for CSI prediction issues [1]. Also, Way forward on simulation assumptions for CSI prediction was agreed [2]. In this contribution, we provide our views on CSI reporting requirement framework for CSI prediction issues.

# Discussion

In this Chapter, we provide our views on CSI prediction requirement issues and share simulation results.

Way forward on simulation assumptions for CSI prediction was agreed in the previous meeting [2]. In Table 1 we list used simulation parameters. Compared to assumptions, we have limited our simulation scope to FDD 10MHz bandwidth with 15kHz SCS. We provide simulation results for all agreed Dopplers (20Hz, 50Hz, and 100Hz) and both agreed TX configurations (32 (N1, N2) = (4, 4), and 16 (N1, N2) = (4, 2)). Datasets are generated with link level simulator. Testing dataset consists of channel realizations of all agreed Dopplers (mixed/generalized dataset).

Table 1: Simulation parameters for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Carrier frequency | 2GHz for FDD |
| Bandwidth | 10MHz for FDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD |
| Subcarrier spacing | 15kHz for 2GHz |
| gNB TX antennas | 32 (N1, N2) = (4, 4) as defined in Rel-15 type1 16 (N1, N2) = (4, 2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Doppler spread | 20Hz, 50Hz and 100Hz |
| Delay spread | 30ns |
| Channel estimation | Ideal DL channel estimation |
| Rank per UE | Rank 1 and 2 |
| CSI feedback assumption | Check the CSI prediction (outputs Raw channel) performance |
| KPI | Average of SGCS for intermediate results over all subbands per layer (Comparing the SVD of model output with SVD of the ground truth Raw channel) |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training and validation datasets | Set1: 16TX containing 20Hz, 50Hz, and 100Hz Doppler ideal estimates Set2: 32TX containing 20Hz, 50Hz, and 100Hz Doppler ideal estimates |
| Testing datasets | Set1: 16TX containing only 20Hz Doppler ideal estimates Set2: 16TX containing only 50Hz Doppler ideal estimates Set3: 16TX containing only 100Hz Doppler ideal estimates Set4: 32TX containing only 20Hz Doppler ideal estimates Set5: 32TX containing only 50Hz Doppler ideal estimates Set6: 32TX containing only 100Hz Doppler ideal estimates |

In the following Table 2 we share our average of SGCS for intermediate results over all subbands per layer.

Table 2: Average of SGCS for intermediate results over all subbands per layer

| Test | Doppler [Hz] | Result [SGCS per layer] |
| --- | --- | --- |
| FDD 16TX | 20 | [0.97309, 0.95257] |
|  | 50 | [0.94964, 0.90751] |
|  | 100 | [0.86359, 0.76106] |
| FDD 32TX | 20 | [0.97741, 0.95507] |
|  | 50 | [0.95253, 0.90621] |
|  | 100 | [0.85010, 0.73412] |

From these results we can make the following observations.

Observation #1: FDD 16TX and FDD 32TX results with the same Doppler perform quite similarly.

Observation #2: In lower Doppler accuracy is better than in higher Doppler.

We believe that using only ideal channel estimates for training, validation, and testing is insufficient, as the final test requirements must be conducted with realistic channel estimates. Therefore, we propose incorporating realistic channel estimates in the next phase of evaluation.

Proposal #1: We propose incorporating realistic channel estimates in the next phase of evaluation.

Testing conditions

Agreement:

- Use static conditions as baseline.

- FFS on whether to have non-static condition.

AI/ML Performance

Agreement:

- For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists.

  - FFS which legacy requirement/feature are relevant

  - FFS whether AI/ML based performance should be better than the legacy.

We repeat this discussion from our previous contribution for reference to further test configuration discussions.

In Release 18 we introduced new PMI requirements for eType II doppler codebook in MIMO evolution work item that is CSI prediction without AI/ML. During this work RAN4 observed great difficulties finding suitable test configuration that would enable significantly measurable PMI prediction gains compared to legacy non-predicting eType II reporting. Without reasonable PMI prediction gain we consider such a test infeasible. Therefore, we propose to use legacy non-predicting eType II codebook as reference when designing any new PMI reporting tests to confirm test configuration enables any measurable prediction gains.

Observation #3: When defining Release 18 PMI prediction test requirements, it was challenging to find test configuration that enables measurable PMI prediction gains over legacy baseline.

Proposal #2: We propose to use eType II non-predicting reporting as reference in test case definition to ensure test feasibility.

Proposal #3: We propose to first focus on static condition to guarantee test feasibility.

Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook

Agreement:

  - For FR1 FDD 15kHz SCS case, test setup is as below descriptions:

- Non-CSI slots: scheduled in mod(i, 10) = {1,3,5,7,8,9}, i={0,1,2,…,19}

- CSI-RS is scheduled in mod(i, 10) = {0,2,4,6}, i={0,1,2,…,19}

- SSB slots: i=0

- TRS slots: i=10 and 11.

- CSI report is transmitted in slot#10n+10 to estimate precoder in slot#10n+11 (i.e., δ=1)

- Test equipment apply the estimated precoder in slot#10n+{15, 17, 18, 19, 21, 23}

Test setup for FR1 TDD case of TypeII-Doppler-r18 codebook

Agreement:

  - For FR1 TDD 30kHz SCS case, test setup is as below descriptions for the simulation of following Rel18 Doppler codebook vs random Type I single panel codebook:

- Scheduling pattern for TDD SCS=30kHz with the periodicity of 20ms (40 slots):

- Non-CSI slots: scheduled in mod(i, 10) = {5,6}, i={0,1,2,…,39}

- CSI slots: scheduled in mod(i, 10) = {0,1,2,3}, i={0,1,2,…,39}

- SSB slots: i=0

- TRS slots: i=20 and 21.

- CSI report is transmitted in slot#10n+9 to estimate precoder in slot#10n+10 (i.e., δ=1)

- Test equipment apply the estimated precoder in slot#10n+{15, 16}

For the eType II Doppler codebook in TS38.214 Clause 5.2.2.2.10 (Enhanced Type II codebook for predicted PMI) [3], there are four Rel-18 requirements defined in TS38.101-4 in Clauses 6.3.2.1.8, 6.3.3.1.8, 6.3.2.2.9, and 6.3.3.2.8 [4]. The configurations of these tests can be considered as a starting point.

However, Rel-18 WI ran out of time to fully optimize test configurations, and there could be room to improve the configuration further to enable more PMI prediction gains. We suggest further checking the test configuration in Rel-19 work. The timing-related parameters and diagrams of FDD and TDD tests of the eType II Doppler codebook, shown above, are taken from the Rel-18 WI WF document [5] to help with the Rel-19 requirement discussion.

In our view in Rel-19 requirement work we should focus on prediction timing (“Estimated PMI” in timing figures) and prediction aging (“PDSCH” in timing figures related to prediction timing). Additionally, CSI-RS resource allocation impacts PMI prediction performance and needs to be carefully chosen.

Observation #4: There are four existing PMI requirements for PMI prediction that can be used as starting point for Rel-19 work.

Observation #5: There is still room to improve existing test configuration to enable even better prediction gain.

Proposal #4: We propose to use existing eType II doppler codebook test configurations as a starting point for Rel-19 work.

Proposal #5: We propose to further study if existing eType II doppler codebook test configurations can be enhanced for better prediction opportunities.

Proposal #6: We propose to study prediction timing in test configuration.

Proposal #7: We propose to study prediction aging in test configuration.

Proposal #8: We propose to study CSI-RS resource allocation in test configuration.

# Conclusion

In this contribution we provide the view on the CSI reporting requirement framework for CSI prediction issues. The following observations and proposals are made:

Observation #1: FDD 16TX and FDD 32TX results with the same Doppler perform quite similarly.

Observation #2: In lower Doppler accuracy is better than in higher Doppler.

Proposal #1: We propose incorporating realistic channel estimates in the next phase of evaluation.

Observation #3: When defining Release 18 PMI prediction test requirements, it was challenging to find test configuration that enables measurable PMI prediction gains over legacy baseline.

Proposal #2: We propose to use eType II non-predicting reporting as reference in test case definition to ensure test feasibility.

Proposal #3: We propose to first focus on static condition to guarantee test feasibility.

Observation #4: There are four existing PMI requirements for PMI prediction that can be used as starting point for Rel-19 work.

Observation #5: There is still room to improve existing test configuration to enable even better prediction gain.

Proposal #4: We propose to use existing eType II doppler codebook test configurations as a starting point for Rel-19 work.

Proposal #5: We propose to further study if existing eType II doppler codebook test configurations can be enhanced for better prediction opportunities.

Proposal #6: We propose to study prediction timing in test configuration.

Proposal #7: We propose to study prediction aging in test configuration.

Proposal #8: We propose to study CSI-RS resource allocation in test configuration.

# Reference

- R4-2502856, WF on the requirements for AI/ML air interface, Qualcomm inc.

- R4-2503026, Way forward on simulation assumptions for CSI prediction, OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple

- 3GPP TS 38.214 V18.6.0 (2025-03), Physical layer procedures for data (Release 18)

- 3GPP TS 38.101-4 V18.6.0 (2024-12), User Equipment (UE) radio transmission and reception;
Part 4: Performance requirements (Release 18)

- R4-2406111, Way Forward for [110bis][328] NR_MIMO_evo_DL_UL_demod, Samsung