3GPP TSG-RAN WG4 Meeting #114bis	R4-2503271

Wuhan, China, 7th Apr 2025 – 11th Apr 2025

Agenda item:	7.20.2

Source:	MediaTek Inc.

Title:	Discussion on Testing issues for CSI compression two-sided models

Document for:	Discussion

# Introduction

In RAN4#114, RAN4 concluded that simulation results in RAN4#114 are aligned enough to proceed in two study tracks of Option 3, and Option 4a to further check the feasibility of testing for AI/ML CSI compression [1]. In the first track of Option 3 encoder is trained with frozen pretrained decoders from other companies using companies own dataset. In the second track of Option 3 encoder and decoder are trained with mixed dataset from contributing companies. In Option 4a labeled dataset is generated using frozen pretrained encoder. The labelled dataset is then used to train decoder, and this decoder is furthermore used to train encoder. In this contribution, we provide our simulation results based on the agreed data generation assumptions and AI/ML model.

# Discussion

For training, validation, and testing data, we follow RAN4’s agreement to reuse system level parameters in TR38.843. Here, we consider the baseline case and ideal channel estimation.

Table 1. Parameters for system level simulation

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 |
| CSI feedback |  | Feedback assumption at least for baseline scheme -CSI feedback periodicity (full CSI feedback): 5 ms (baseline) -Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

The detailed structures of ResCNN encoder and decoder for two-sided CSI compression AI/ML model are shown in Figure 1 and 2.

| Figure 1. Detailed model diagrams of encoder | Figure 2. Detailed model diagrams of decoder |
| --- | --- |

We generate training, validation and testing data based on assumptions in Table 1 and use the above encoder and decoder pair for training with the main hyperparameters listed in Table 2. We also use ideal channel estimation.

Table 2. Hyperparameters used for ResCNN model.

| Hyperparameters | MediaTek dataset MTK0R4_113dsei01.zip | Mixed dataset (updated) MIXEDR4_114dsei00.zip |
| --- | --- | --- |
| Number of samples in training dataset (80%) | 480k | 400k |
| Number of samples in validation dataset (10%) | 60k | 50k |
| Number of samples in testing dataset (10%) | 60k | 50k |
| Quantization (for quantized models) | (1/8, 3/8, 5/8, 7/8) | (1/8, 3/8, 5/8, 7/8) |
| Optimizer | Adam | Adam |
| Learning rate | 0.0001 | 0.0001 |
| Batch size | 256 | 256 |
| Max number of epochs | 150 | 150 |
| Stopping criteria | No validation loss improvement over 25 epochs | No validation loss improvement over 25 epochs |
| Loss function | 1 - SGCS | 1 - SGCS |
| Drop rate | No drop out | No drop out |

## 2.1 Option 3 Track 1 (training of encoder using other companies frozen pretrained decoders and own company dataset)

In RAN4#114 companies were encouraged to train encoder, using other companies frozen pretrained decoders, using own dataset, and report the average SGCS. We have collected all updated Track 1 frozen pretrained decoders from companies for evaluation.

We summarize sub-band averaged SGCS for ResCNN encoder trained with frozen pretrained CATT decoder with MediaTek dataset in Table 3.

Table 3. SGCS after training encoder with MediaTek dataset using the frozen pretrained CATT decoder

| Frozen pretrained CATT decoder | Overhead (bits) | MediaTek dataset |
| --- | --- | --- |
|  |  | SGCS |
| ResCNN model 2-bit quantized | 64 bits | 0.6952 |
| ResCNN model Float32 | 32*32 bits | 0.7461 |

We summarize sub-band averaged SGCS for ResCNN encoder trained with frozen pretrained Ericsson decoder with MediaTek dataset in Table 4.

Table 4. SGCS after training encoder with MediaTek dataset using the frozen pretrained Ericsson decoder

| Frozen pretrained Ericsson decoder | Overhead (bits) | MediaTek dataset |
| --- | --- | --- |
|  |  | SGCS |
| ResCNN model 2-bit quantized | 64 bits | 0.7203 |
| ResCNN model Float32 | 32*32 bits | 0.7784 |

We summarize sub-band averaged SGCS for ResCNN encoder trained with frozen pretrained Huawei decoder with MediaTek dataset in Table 5.

Table 5. SGCS after training encoder with MediaTek dataset using the frozen pretrained Huawei decoder

| Frozen pretrained Huawei decoder | Overhead (bits) | MediaTek dataset |
| --- | --- | --- |
|  |  | SGCS |
| ResCNN model 2-bit quantized | 64 bits | 0.6139 |
| ResCNN model Float32 | 32*32 bits | 0.6556 |

We summarize sub-band averaged SGCS for ResCNN encoder trained with frozen pretrained Nokia decoder with MediaTek dataset in Table 6.

Table 6. SGCS after training encoder with MediaTek dataset using the frozen pretrained Nokia decoder

| Frozen pretrained Nokia decoder | Overhead (bits) | MediaTek dataset |
| --- | --- | --- |
|  |  | SGCS |
| ResCNN model 2-bit quantized | 64 bits | 0.7236 |
| ResCNN model Float32 | 32*32 bits | 0.7717 |

We summarize sub-band averaged SGCS for ResCNN encoder trained with frozen pretrained Samsung decoder with MediaTek dataset in Table 7.

Table 7. SGCS after training encoder with MediaTek dataset using the frozen pretrained Samsung decoder

| Frozen pretrained Samsung decoder | Overhead (bits) | MediaTek dataset |
| --- | --- | --- |
|  |  | SGCS |
| ResCNN model 2-bit quantized | 64 bits | 0.7197 |
| ResCNN model Float32 | 32*32 bits | 0.7660 |

We summarize sub-band averaged SGCS for ResCNN encoder trained with frozen pretrained decoders with MediaTek dataset in Table 8, collected from Tables 3-7. In the last row of the table there is Reference corresponding to where MediaTek Encoder and Decoder were trained together with MediaTek dataset.

Table 8. SGCS of ResCNN (Quantized and non-quantized) with MediaTek dataset.
Encoders are trained with frozen pretrained decoders.

|  | MediaTek Encoder with frozen pretrained Decoder ResCNN model 2-bit quantized | MediaTek Encoder with frozen pretrained Decoder ResCNN model Float32 |
| --- | --- | --- |
| Decoder Company | SGCS | SGCS |
| CATT | 0.6952 | 0.7461 |
| Ericsson | 0.7203 | 0.7784 |
| Huawei | 0.6139 | 0.6556 |
| Nokia | 0.7236 | 0.7717 |
| Samsung | 0.7197 | 0.7660 |
| Reference | 0.721 | 0.774 |

From these results we can see that SGCS results after training encoders with own MediaTek dataset using frozen pretrained decoders can in most cases closely match reference SGCS results where Encoder and Decoder are trained together. Training results using frozen pretrained decoder from Huawei are somewhat worse to others and we suspect some statistical difference between MediaTek and Huawei datasets to cause this.

Observation #1: Option 3 Track 1 SGCS results with frozen pretrained decoders using companies own datasets can in most cases closely match reference SGCS results where Encoder and Decoder are trained together (Reference).

Observation #2: Dataset can have an impact on SGCS results for some shared frozen pretrained decoder models.

## 2.2 Option 3 Track 2 (training of encoder using other companies frozen pretrained decoder and mixed dataset)

In RAN4#114 companies were encouraged to train encoder, using other companies frozen pretrained decoders, using updated mixed dataset, and report the average CDF. In Post meeting discussions Samsung decoder was selected as frozen pretrained decoder as the priority. As a second priority, if time allows, companies are encouraged to also try the Nokia decoder as the frozen pretrained decoder. Furthermore, as a third priority, companies are encouraged to also try other companies’ decoders as frozen pretrained decoder. We have selected decoders from Samsung and Nokia for evaluation.

We provide our results for test set performance in Table 9 with MediaTek and updated Mixed datasets where encoder is trained using frozen pretrained decoder from Samsung and Nokia. In the last row of the table there is Reference corresponding to where MediaTek Encoder and Decoder were trained together with MediaTek and updated Mixed dataset.

Table 9. SGCS of ResCNN (Quantized and non-quantized) with MediaTek and updated Mixed datasets.
Encoders are trained with frozen pretrained decoders.

|  | MediaTek Encoder with frozen pretrained Decoder ResCNN model 2-bit quantized |  | MediaTek Encoder with frozen pretrained Decoder ResCNN model Float32 |  |
| --- | --- | --- | --- | --- |
|  | MediaTek dataset | Mixed dataset | MediaTek dataset | Mixed dataset |
| Decoder Company | SGCS | SGCS | SGCS | SGCS |
| Nokia | 0.7191 | 0.6979 | 0.1075 | 0.1005 |
| Samsung | 0.7218 | 0.7008 | 0.7678 | 0.7543 |
| Reference | 0.721 | 0.695 | 0.774 | 0.739 |

Training results using frozen pretrained decoder from Nokia in case of unquantized models are bad and we suspect some training convergence issue with agreed hyperparameters.

Observation #3: Option 3 Track 2 SGCS results with frozen pretrained decoder (trained with mixed dataset) using own or mixed dataset can in most cases closely match reference SGCS results where Encoder and Decoder are trained together (Reference).

Observation #4: Option 3 Track 2 SGCS results with frozen pretrained decoder (trained with mixed dataset) using own or mixed dataset can be poor in unquantized training where we suspect convergence issue with agreed hyperparameters.

## 2.3 Option 4a (training of decoder with labelled dataset)

In RAN4#114 companies were encouraged to train decoder, using labelled dataset generated using selected company’s frozen pretrained encoder as a Step 1. We have selected encoders from Samsung and Nokia for dataset labelling. With trained decoder from previous Step 1 then companies were encouraged to train encoder. The average SGCS of these training step should be reported.

For the Step 1, we provide our results for test set performance in Table 10 where labelled dataset is first generated with MediaTek and updated Mixed datasets using frozen pretrained encoders from Samsung and Nokia. Then decoder is trained using the labelled dataset. In the last row of the table there is Reference corresponding to where MediaTek Encoder and Decoder were trained together with MediaTek and updated Mixed dataset.

Table 10. SGCS of Decoder ResCNN (Quantized and non-quantized) with labelled MediaTek and Mixed datasets.
Decoders are trained with labelled datasets.

| Step 1 | MediaTek Decoder ResCNN model 2-bit quantized |  | MediaTek Decoder ResCNN model Float32 |  |
| --- | --- | --- | --- | --- |
|  | Labelled MediaTek dataset | Labelled Mixed dataset | Labelled MediaTek dataset | Labelled Mixed dataset |
| Encoder Company for dataset labelling | SGCS | SGCS | SGCS | SGCS |
| Nokia | 0.7026 | 0.6833 | 0.7736 | 0.7566 |
| Samsung | 0.7048 | 0.6856 | 0.7668 | 0.7506 |
| Reference | 0.721 | 0.695 | 0.774 | 0.739 |

Observation #5: Option 4a SGCS results with using labelled dataset can closely match reference SGCS results where Encoder and Decoder are trained together (Reference).

For the Step 2, we provide our results for test set performance in Table 11 with MediaTek and updated Mixed datasets where encoder is trained using frozen pretrained decoder from the previous Step 1. In the last row of the table there is Reference corresponding to where MediaTek Encoder and Decoder were trained together with MediaTek and updated Mixed dataset.

Table 11. SGCS of ResCNN (Quantized and non-quantized) with MediaTek and updated Mixed datasets.
Encoders are trained with frozen pretrained decoders from the previous step.

| Step 2 | MediaTek Encoder with Step 1 Decoder ResCNN model 2-bit quantized |  | MediaTek Encoder with Step 1 Decoder ResCNN model Float32 |  |
| --- | --- | --- | --- | --- |
|  | MediaTek dataset | Mixed dataset | MediaTek dataset | Mixed dataset |
| Encoder Company for dataset labelling in Step 1 | SGCS | SGCS | SGCS | SGCS |
| Nokia | 0.7040 | 0.6836 | 0.7753 | 0.7572 |
| Samsung | 0.7060 | 0.6859 | 0.2727 | 0.7498 |
| Reference | 0.721 | 0.695 | 0.774 | 0.739 |

Observation #6: Option 4a SGCS results with frozen pretrained decoder from previous step can in most cases closely match reference SGCS results where Encoder and Decoder are trained together (Reference).

Observation #7: Option 4a SGCS results with frozen pretrained decoder from previous step can be poor in unquantized training where we suspect convergence issue with agreed hyperparameters.

# Conclusion

In this contribution, simulation results for alignment are provided. Considering ResCNN AI/ML model, we have the following observations:

Observation #1: Option 3 Track 1 SGCS results with frozen pretrained decoders using companies own datasets can in most cases closely match reference SGCS results where Encoder and Decoder are trained together (Reference).

Observation #2: Dataset can have an impact on SGCS results for some shared frozen pretrained decoder models.

Observation #3: Option 3 Track 2 SGCS results with frozen pretrained decoder (trained with mixed dataset) using own or mixed dataset can in most cases closely match reference SGCS results where Encoder and Decoder are trained together (Reference).

Observation #4: Option 3 Track 2 SGCS results with frozen pretrained decoder (trained with mixed dataset) using own or mixed dataset can be poor in unquantized training where we suspect convergence issue with agreed hyperparameters.

Observation #5: Option 4a SGCS results with using labelled dataset can closely match reference SGCS results where Encoder and Decoder are trained together (Reference).

Observation #6: Option 4a SGCS results with frozen pretrained decoder from previous step can in most cases closely match reference SGCS results where Encoder and Decoder are trained together (Reference).

Observation #7: Option 4a SGCS results with frozen pretrained decoder from previous step can be poor in unquantized training where we suspect convergence issue with agreed hyperparameters.

# Reference

- R4-2502953, WF on AI/ML air interface study, Ericsson