# 3GPP TSG-RAN WG4 Meeting #114bis 	                                R4-2503286

Wuhan, China, 07th – 11th April, 2025

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on Potentially new requirements for SBFD operation

Agenda Item:       7.23.2.1

Document for:     Discussion

# Introduction

This contribution presents our views on the potential new requirements for SBFD operation. In the RAN4 #114 meeting, RAN4 agreed to further investigate the duration and characteristics of the transient period between SBFD and non-SBFD operation, as outlined in the following agreement [1].

Agreement: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols - FFS guard symbol(s) in this case is SBFD symbol or not. - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission: - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design. - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall - either send LS to RAN1 to ask for clarification, - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact. - Case-D: in TA_offset in non-SBFD UL symbol(s). - Note: The illustrative figure for Case A, B, C and D is provided below:

# Discussion

## Location of transient period between SBFD and non-SBFD

In SBFD systems, the transient time must align with existing TDD switching mechanisms to maintain a consistent total TX switching time budget during transitions between fully allocated UL/DL and SBFD RBs. Here, we present our perspectives on all four cases.

Case A: TA_offset is applied in SBFD symbols.

Case B: If SBFD symbols are used as guard symbols, they preserve SBFD-specific waveform properties, potentially enabling a more stable transition. If non-SBFD symbols are used instead, this approach offers greater flexibility for different transmission schemes but may introduce additional synchronization overhead. A viable solution could be to allow gNB implementation flexibility based on system requirements and network conditions.

Case C: If the guard period is placed in non-SBFD DL symbols, its impact on DL-to-UL switching efficiency—particularly in TDD scenarios—should be analyzed. Using blanked SBFD symbols can help mitigate interference; however, it may lead to resource wastage depending on the blanking duration. The design of the guard period influences UE synchronization, channel estimation accuracy, and overall system overhead. It may be beneficial to send an LS to RAN1 to request further clarification on guard period design.

Case D: TA_offset is applied in non-SBFD UL symbols.

Observation 1: The placement of the transient period should be treated as a gNB implementation aspect rather than a strict RAN4 requirement. This decision should consider key factors such as network synchronization, interference management, and scheduling flexibility. Input from RAN1 may be useful to ensure alignment with overall system design.

Proposal 1: Recognize the placement of the transmitter transient period as an implementation-specific decision, with no direct impact on RAN4 standards. However, given its influence on network synchronization and guard period design, it may be beneficial to seek input from RAN1 to ensure alignment with overall system considerations.

# Conclusion

In this contribution, we summarised the potentially new requirements for SBFD operation with the following proposals and observations.

Observation 1: The placement of the transient period should be treated as a gNB implementation aspect rather than a strict RAN4 requirement. This decision should consider key factors such as network synchronization, interference management, and scheduling flexibility. Input from RAN1 may be useful to ensure alignment with overall system design.

Proposal 1: Recognize the placement of the transmitter transient period as an implementation-specific decision, with no direct impact on RAN4 standards. However, given its influence on network synchronization and guard period design, it may be beneficial to seek input from RAN1 to ensure alignment with overall system considerations.

# Reference

- R4-2502310: Way Forward for [114][306] NR_duplex_evo_General.