# 3GPP TSG-RAN WG4 Meeting #114bis 	                                R4-2503287

Wuhan, China, 07th – 11th April, 2025

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on existing Tx requirements for SBFD operation

Agenda Item:       7.23.2.2

Document for:     Discussion

# Introduction

In this contribution, we present overview of impact on existing BS RF TX requirements for unwanted emissions.

# Discussion

## Unwanted emissions

In the RAN4 #113 meeting, RAN4 agreed to further study on ACLR and OBUE with following agreement and WF [1].

Agreement: - FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

In the RAN4 #114 meeting, discussions on unwanted emissions are still ongoing [2]. Here, we present our views on ACLR/OBUE.

### ACLR/OBUE

We present our views on FR1 macro deployments coexisting with a new SBFD system in an adjacent channel. To ensure reliable coexistence, we propose tightening ACLR and OBUE requirements for FR1 WA-BS, as existing standards do not sufficiently address gNB-to-gNB CLI challenges. Stricter isolation standards and enhanced RF specifications are necessary. A feasibility study [3] outlines key requirements, including a 49 dBm base station output power, 100 MHz channel bandwidth, and a maximum 1 dB degradation in victim base station sensitivity. Achieving the required 144 dB isolation depends on multiple factors, including ACLR, coupling loss, and RF BPF rejection. While PA linearization techniques such as Digital Predistortion (DPD) have improved ACLR performance to ≥55 dBc, achieving 144 dB isolation in real deployments remains challenging. Additional filtering solutions or design modifications may be required to mitigate gNB-to-gNB CLI interference and ensure robust, interference-free SBFD system deployment.

Observation 1: Current ACLR and OBUE standards for FR1 macro-BSs fall short in ensuring coexistence with SBFD systems, particularly in dense macro-cell environments where interference is more severe.

Observation 2: Strengthening ACLR1 and ACLR2 requirements enhances BPF rejection, leading to more effective interference mitigation in co-located deployments.

Proposal 1: Define scenario-specific ACLR and OBUE requirements. For SBFD co-located deployments, strengthen ACLR1 and ACLR2 to meet the minimum necessary co-location ACLR values.

Proposal 2: Implement a CLI mitigation framework to manage gNB-to-gNB interference in adjacent channels. This framework should provide clear guidelines for dynamic interference management and establish co-location requirements specifically designed for SBFD-capable gNBs.

# Conclusion

In this contribution, we would like to present our perspectives on the RF TX requirements for SBFD BS, outlined as follows.

Observation 1: Current ACLR and OBUE standards for FR1 macro-BSs fall short in ensuring coexistence with SBFD systems, particularly in dense macro-cell environments where interference is more severe.

Observation 2: Strengthening ACLR1 and ACLR2 requirements enhances BPF rejection, leading to more effective interference mitigation in co-located deployments.

Proposal 1: Define scenario-specific ACLR and OBUE requirements. For SBFD co-located deployments, strengthen ACLR1 and ACLR2 to meet the minimum necessary co-location ACLR values.

Proposal 2: Implement a CLI mitigation framework to manage gNB-to-gNB interference in adjacent channels. This framework should provide clear guidelines for dynamic interference management and establish co-location requirements specifically designed for SBFD-capable gNBs.

# Reference

- R4-2419908, Way Forward for [113][307] NR_duplex_evo_BSRF

- R4-2502380, Way Forward for [114][307] NR_duplex_evo_BSRF

- R4-2417788, “Discussion on existing Tx requirements for SBFD operation”, Tejas Networks