3GPP TSG-RAN WG4 Meeting #114bis	R4-2503290

# WuHan, China, 07 April – 11 April, 2025

Source: 	vivo

Title: 	On RRM requirements for UE-to-UE CLI handling

Agenda Item:			7.23.3.1

Document for:	   Discussion

- Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

The updated objective in [1] which are related to RRM are copied below for reference:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

    - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - PUSCH resource mapping, i.e., rate-matching around the muted REs

    - UCI resource determination in symbols with muted REs

  - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] (Reuse major principles)

    - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

    - At least aperiodic reporting

    - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - CLI measurement accuracy requirement

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, we provide our considerations on RRM requirements for SBFD based on [2] and [3].

- Discussion

At previous meeting, based on the WF [2], the following agreements are available for discussion:

- Sub-topic 1-1: General issues

Issue 1-1-2: Rx beam

- Proposals

  - Proposal 1a (LGE):

    - RAN4 can capture the original statements of RAN1 agreements. If necessary minor change based on the statement of RAN1 agreements can be considered.

  - Proposal 1b (ZTE, CATT):

    - directly refer to RAN1 spec instead of repeating all details in RAN spec.

  - Proposal 2 (Nokia):

    - The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period.

- Recommended WF

  - Agree on P2: The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period.

  - Further discuss P1a and P1b in next meetings.

For the RX beam, no matter the Rel-16 method is followed or following Rel-19 design where a UE is given a TCI state for the CLI measurement, either by configuring a list of TCI states or configuring a unifiedTCI-StateType, UE will not perform beam sweeping hence there is no impact on measurement period. Hence we agree with the recommended WF using P2.

Proposal 1: For the Rx beam issue, agree with the WF to agree P2.

Issue 1-1-6: Measurement restriction

- Proposals for collision between L1 CLI measurements and L1/L3 RRM measurements

  - Proposal 1 (Nokia, E///, MTK):

    - Legacy measurement restriction applies, i.e. do not consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel

  - Proposal 1a (HW):

    - For L1-SRS-RSRP and L1-CLI-RSSI Method #3, R16 requirements are re-used as baseline.

  - Proposal 2a (CATT):

    - Consider the exception cases where UE can receive CLI resources and other resources at the same time

  - Proposal 2b (HW):

    - For L1-CLI-RSSI Method #1, measurement restriction applies only when UE cannot perform CLI measurement and other measurement at same time.

- Proposals for collision between different CLI measurements

  - Proposal 1a (Apple):

    - FFS if it should be clarified in the RAN4 specification that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

  - Proposal 1b (Nokia):

    - RAN4 shall specify that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

  - Proposal 2a (Nokia):

    - L1-CLI-RSSI measurement requirements do not apply when the configured L1-CLI-RSSI measurement resources are partially or fully overlapping with configured SRS-RSRP for method #2

  - Proposal 2b (QC):

    - RAN4 to discuss how to handle the collisions of L1-CLI-RSSI resources with L1-SRS-RSRP resources configured for L1 CLI measurements, i.e., which one of the two should the UE prioritize?

- Recommended WF for collision between L1 CLI measurements and L1/L3 RRM measurements

  - For L1-SRS-RSRP, measurement requirements for L1-SRS-RSRP do not apply if L1-SRS-RSRP measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

  - For L1-CLI-RSSI Method #1, measurement requirements for L1-CLI-RSSI do not apply if L1-CLI-RSSI measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, FFS whether to consider exception case

    - Option 1: do not consider exception case, i.e. measurement requirements for L1-CLI-RSSI do not apply

    - Option 2: consider exception case, i.e. measurement requirements for L1-CLI-RSSI apply when UE can perform L1-CLI-RSSI measurement and other measurement at same time.

  - For L1-CLI-RSSI Method #3,

    - If the measurement timing is same as L1-SRS-RSRP, as baseline, measurement restriction requirements are same as L1-SRS-RSRP.

    - If the measurement timing is same as L1-CLI-RSSI Method #1, as baseline, measurement restriction requirements are same as L1-CLI-RSSI Method #1.

    - FFS whether any adaptation is needed on top of baseline.

- Recommended WF for collision between different CLI measurements

  - FFS whether to capture in RAN4 spec that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

  - FFS how to handle collision between L1-SRS-RSRP and L1-CLI-RSSI measurement resources

    - Option 1: prioritize L1-SRS-RSRP measurement

    - Option 2: prioritize L1-CLI-RSSI measurement

    - Option 3: other

On how to handle collision between different CLI measurements, we prefer capture in RAN4 specs that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. On how to handle collision between L1-SRS-RSRP and L1-CLI-RSSI measurement resources，option 1: prioritize L1-SRS-RSRP measurement is preferred.

Proposal 2: Prefer to capture in RAN4 specs that UE is not expected to perform CLI measurement using Method #1 and Methods#3 in the same OFDM symbol. On how to handle collision between L1-SRS-RSRP and L1-CLI-RSSI measurement resources，option 1: prioritize L1-SRS-RSRP measurement is preferred.

Issue 1-1-9: Requirement applicability

- Proposals

  - Proposal 1 (LGE):

    - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

      - SRS-RSRP measurement resource is fully confined within BW of UL sub-band

      - CLI-RSSI measurement resource is configured within DL sub-band

  - Proposal 2 (Nokia, CATT, HW, MTK, QC):

    - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

      - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP

      - when CLI-RSSI measurement resource is configured within active BWP

  - Proposal 3 (ZTE):

    - The L1 CLI measurement requirements are applicable for RRC_CONNECTED:

      - For SRS-RSRP resource, it should be within the UL active BWP, UE could measure the SRS-RSRP resource within the UL active BWP.

      - For CLI-RSSI resource

        - For CLI-RSSI measurement within UL SB, no restriction on resource configuration, but UE only measure the resource within UL active BWP.

        - For CLI-RSSI measurement within DL SB, no matter the CLI-RSSI resource cross two DL SBs or not, UE measures the CLI-RSSI resource within one or two DL SBs, no restriction on resource configuration.

      - No gap is needed for UE-to-UE CLI measurement

- Recommended WF

  - The proposals are close to each other, further discuss the following issues and come up with combined the wording

    - Whether we define L1 CLI measurement resources to be confined within UL or DL subband or just DL active BWP

    - Whether we define L1 CLI measurement as intra-frequency

For requirement applicability, P2 is ok.

Proposal 3: For requirement applicability, support P2.

- Sub-topic 1-3: L1-CLI-RSSI

Issue 1-3-3: Measurement period

- Proposals

  - Proposal 1a (ZTE):

    - For Method#1, for the case of two DL subbands configured, no matter whether puncturing happens, no impact on UE processing since all non punctured CSI-RS resource are within active DL BWP and half-duplex operation at UE side.

  - Proposal 1b (HW):

    - For Method#1, measurement delay requirements for L1-CLI-RSSI are agnostic to measurement resource types (type#1 or type#2)

  - Proposal 2 (CATT, E///, HW):

    - Measurement period for Method #3 is same as Method #1, i.e. based on single sample.

Proposal 4: For measurement period for L1-CLI-RSSI, Ok with P1b.

- Conclusions

In this contribution we provided our initial considerations on RRM requirements for SBFD and have the following proposals:

Proposal 1: For the Rx beam issue, agree with the WF to agree P2.

Proposal 2: Prefer to capture in RAN4 specs that UE is not expected to perform CLI measurement using Method #1 and Methods#3 in the same OFDM symbol. On how to handle collision between L1-SRS-RSRP and L1-CLI-RSSI measurement resources，option 1: prioritize L1-SRS-RSRP measurement is preferred.

Proposal 3: For requirement applicability, support P2.

Proposal 4: For measurement period for L1-CLI-RSSI, Ok with P1b.

- Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2502699, WF on RRM requirements for NR_duplex_evo, Huawei, RAN4 114

[3] R4-2500536,	Topic summary for [114][222] NR_duplex_evo, Huawei, RAN4 114