3GPP TSG-RAN WG4 Meeting #114bis	R4-2503291

# WuHan, China, 07 April – 11 April, 2025

Source: 	vivo

Title: 	Further consideration on RRM impacts for SBFD operation

Agenda Item:			7.23.3.2

Document for:	   Discussion

- Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

The updated objective in [1] which are related to RRM are copied below for reference:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

    - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - PUSCH resource mapping, i.e., rate-matching around the muted REs

    - UCI resource determination in symbols with muted REs

  - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] (Reuse major principles)

    - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

    - At least aperiodic reporting

    - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - CLI measurement accuracy requirement

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, we provide our considerations on RRM requirements for SBFD.

- Discussion

This WI has been discussed at RAN4 112 and RAN4 112bis meeting. At previous meeting, based on the WF [2] and topic summary [3], the following agreements are available:

Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

- Agreements

  - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case

    - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

    - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

    - Option 3: Consider the UE measurement capability

  - FFS whether and how to account for the UE capability on CSI processing time.

  - Further discuss the accuracy requirements for the following cases

    - Case 1: X1 >= X3 and X2 >= X3

    - Case 2: X1 >= X3 and X2 < X3

    - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

Issue 2-1-2: Requirements for CSI-RS based L3 measurement

- Agreements (from Wed AH session)

  - For DU case:

    - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols.

  - FFS on DUD case

In this contribution we discuss the following open issues.

Issue 2-1-1a: CSI-RS L1 measurement – collision with UL

- Proposals

  - Proposal 1 (ZTE):

    - For the collision handling between L1 periodic/semi-persistent CSI-RS resource and semi-statically configured/dynamic UL transmission, wait for RAN1 reply. This is applicable to CSI-RS based L1-RSRP, L1-SINR, RLM, BFD and CBD measurements

  - Proposal 2 (Samsung):

    - Dynamic UL transmission shall be prioritized over than L1 CSI-RS based on measurement.

In the RAN1 reply LS, the following statements are available:

Issue#1: Collision Case 2 that involves CSI-RS resources used for L1-RSRP/SINR and RLM/BFD/CBD measurements

For collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) that involves CSI-RS resources used for L1-RSRP/SINR and RLM/BFD/CBD measurements, RAN1 would like to provide the following response:

The intention of the mentioned RAN1 agreement is to follow the existing prioritization rules between semi-static DL channels/signals and dynamically scheduled UL transmission including semi-statically configured CSI-RS resources for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement. The existing prioritization rules for serving-cell for collision Case 2 are as follows: - TS 38.213, Clause 11.1 “Slot configuration” “For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot.” - TS 38.213, Clause 11.1.1 “UE procedure for determining slot format” “If a UE is configured by higher layers to receive a CSI-RS or a PDSCH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink or flexible, or the UE detects a DCI format indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the CSI-RS reception in the set of symbols of the slot or cancels the PDSCH reception in the slot.”

To our understanding on RAN1 LS, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) that involves CSI-RS resources used for L1-RSRP/SINR and RLM/BFD/CBD measurements, UL transmission has the higher priority. The corresponding measurement delay of CSI-RS could be extended when it collides with dynamically scheduled UL transmission.

Proposal 1: The corresponding measurement delay of CSI-RS used for L1-RSRP/SINR and RLM/BFD/CBD could be extended when it collides with dynamically scheduled UL transmission.

- Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

For the issue 2-1-1b, although some agreements have been achieved, the agreements need further clarification:

  - Further discuss measurement period requirements fc

    - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

    - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

    - Option 3: Consider the UE measurement capability

For the measurement period for CSI-RS based L1 measurement in SBFD with DUD case, we think the measurement delay requirements should be decoupled with whether the UE measures at both DL subbands or not. The reason is assuming one big DL subband in DUD configuration, the legacy accuracy and measurement delay requirements can still be met. It is also better to determine the delay requirement after resolving the accuracy issue below.

Proposal 2: The measurement delay requirements for CSI-RS based L1 measurement for SBFD under DUD mode should be decoupled with whether the UE measures at both DL subbands or not.

  - Further discuss the accuracy requirements for the following cases

    - Case 1: X1 >= X3 and X2 >= X3

    - Case 2: X1 >= X3 and X2 < X3

    - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

For the accuracy requirements, it is still possible that only one of the DL subband of DUD configuration can satisfy the side conditions (24 or 48 RBs) requirements, then under this scenario whether the configuration on the other DL subband satisfy the side condition maybe not important. Hence case 1 and case 2 can be combined. Regarding whether case 3 should be considered, the consideration is the DUD configuration of SBFD should have large spectrum hence both DL subbands is less than the side conditions (24 or 48 RB) maybe not a typical deployment case, at least the scenario when one of DL band is quite small and the other DL band is quite large should be avoided. Hence we have the following proposal:

Proposal 3: For the accuracy requirements for CSI-RS based L1 measurement for SBFD under DUD mode, consider the following case:

    - For case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3, only consider case 3 when both X1 and X2 > y, y is a lower bound up to discussion.

Issue 2-1-1c: CSI-RS L1 measurement – different symbol types

- Proposals

  - Proposal 1 (Nokia):

    - The same principle, that only CSI-RS transmission occasions within one valid symbol type are used for CSI derivation, shall apply to at least CSI-RS based L1-RSRP and L1-SINR as these are one type of CSI report.

    - RAN4 to discuss the CSI-RS based L1-RSRP and L1-SINR measurement period depending on the valid symbol type (SBFD or non-SBFD symbols).

  - Proposal 2 (ZTE):

    - For one CSI-RS occasion, as long as the CSI-RS bandwidth is not below 48 PRBs, occasion on SBFD and non-SBFD symbols could be combined. This is applicable to CSI-RS based L1-RSRP, L1-SINR, RLM, BFD and CBD measurements

  - Proposal 3 (E///, Samsung):

    - Wait for RAN1 reply to conclude the impact on CSI-RS measurement on SBFD and non-SBFD symbols.

IN RAN1 reply LS [R1-2501560], the following are available:

Issue#2: Behaviour for CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD measurements

For resource configuration and/or reporting configuration for CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement, RAN1 would like to provide the following response:

- According to the following RAN1 agreements, Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement.

Agreement (RAN1#120) The configuration of Configuration 1 or 2 for DL BWP only applies to PDSCH receptions within the DL BWP. Agreement (RAN1#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only. - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols. - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability. Agreement (RAN1#118bis) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.

From RAN1’s reply LS, Configuration 1 or 2 for DL BWP are not applicable for CSI-RS resources hence RAN4 needs figure out how to handle SBFD and non-SBFD symbols when defining corresponding CSI-RS measurement requirements. To our understanding it is possible to combine the CSI-RS resources used for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement, however when combining these resources, CSI-RS resource over SBFD symbols could have different resource number compared with that on non-SBFD symbols due to the difference on the spectrum range seen by a SBFD-aware UE. This may impact accuracy assuming the CSI-RS resources on a SBFD symbol is less than a particular RBs. For simplicity two options can be considered, option 1 is the valid symbol are only either the SBFD symbols or non-SBFD symbols. Option 2 is the valid symbol are both the SBFD symbols or non-SBFD symbols providing CSI-RS on SBFD symbols is larger than X on DU configuration or Y on DUD configuration.

Proposal 4: For valid symbol for CSI-RS L1 measurement, consider the following proposals:

P1: The valid symbols are only either the SBFD symbols or non-SBFD symbols for CSI-RS based L1-RSRP/SINR measurement/reporting.

P2: The valid symbol are both the SBFD symbols or non-SBFD symbols providing CSI-RS on SBFD symbols is larger than X RBs for DU configuration or Y RBs for DUD configuration for CSI-RS based RLM/BFD/CBD measurement.

Issue 2-1-3: Requirements for scheduling restriction

- Proposals related to UL/DL transition

  - Proposal 1 (Nokia):

    - The existing RAN4 scheduling restriction requirements are sufficient to accommodate the Tx-Rx transition time in FR1 and FR2-1. There is no need to modify the existing scheduling restrictions.

  - Proposal 2 (CMCC, E///, HW, vivo):

    - RAN4 scheduling restriction requirement is not needed for DL/UL transition.

- Proposals related to inter-frequency SSB measurement without gap

  - Proposal 1 (Nokia, MTK):

    - There is no need to define new DL scheduling restrictions for inter-frequency measurements without measurement gaps.

    - The existing scheduling restriction of TDD bands in RRM requirements should be applicable to the SBFD operation.

  - Proposal 2 (HW):

    - RAN4 to discuss whether scheduling restriction applies when SSB for L3 measurement is within DL active BWP but not completely contained in DL subbands on SBFD symbols/slots.

  - Proposal 3 (vivo):

    - For the UE support Rel-16 inter-frequency measurement without gap, when the neighbor cell SSB is outside DL subband however still within the active BWP, update on existing scheduling restriction is needed and the scheduling restriction in section 9.3.9.4 can be used as a reference.

For the scheduling restrictions for the inter-frequency SSB measurement without gap, the issue is for the UE support Rel-16 inter-frequency measurement without gap, the neighbour cell SSB can be measured within the DL subband and within the active BWP or outside DL subband however still within the active BWP. For the first scenario where actually the neighbour cell SSB is completely contained within the DL subband, existing scheduling restriction is sufficient and no extra update is needed. However when the neighbor cell SSB is outside DL subband however still within the active BWP, update on existing scheduling restriction is needed and the scheduling restriction in section 9.3.9.4 can be used as a reference. In addition, UE is also not expected to have reception on data outside DL subband on SBFD symbol.

Proposal 5: For the UE support Rel-16 inter-frequency measurement without gap, when the neighbor cell SSB is outside DL subband however still within the active BWP, update on existing scheduling restriction is needed and the scheduling restriction in section 9.3.9.4 can be used as a reference.

- Conclusions

In this contribution we provided our initial considerations on RRM requirements for SBFD and have the following proposals:

Proposal 1: The corresponding measurement delay of CSI-RS used for L1-RSRP/SINR and RLM/BFD/CBD could be extended when it collides with dynamically scheduled UL transmission.

Proposal 2: The measurement delay requirements for CSI-RS based L1 measurement for SBFD under DUD mode should be decoupled with whether the UE measures at both DL subbands or not.

Proposal 3: For the accuracy requirements for CSI-RS based L1 measurement for SBFD under DUD mode, consider the following case:

    - For case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3, only consider case 3 when both X1 and X2 > y, y is a lower bound up to discussion.

Proposal 4: For valid symbol for CSI-RS L1 measurement, consider the following proposals:

- P1: The valid symbols are only either the SBFD symbols or non-SBFD symbols for CSI-RS based L1-RSRP/SINR measurement/reporting.

- P2: The valid symbol are both the SBFD symbols or non-SBFD symbols providing CSI-RS on SBFD symbols is larger than X RBs for DU configuration or Y RBs for DUD configuration for CSI-RS based RLM/BFD/CBD measurement.

Proposal 5: For the UE support Rel-16 inter-frequency measurement without gap, when the neighbor cell SSB is outside DL subband however still within the active BWP, update on existing scheduling restriction is needed and the scheduling restriction in section 9.3.9.4 can be used as a reference.

- Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2502699, WF on RRM requirements for NR_duplex_evo, Huawei, RAN4 114

[3] R4-2500536,	Topic summary for [114][222] NR_duplex_evo, Huawei, RAN4 114