# 3GPP TSG-RAN WG4 Meeting #114bis  		R4-2503295

WuHan, China, 07 April – 11 April, 2025

Source: 	vivo

Title: 	Discussion on RRM requirement impacts from on-demand SIB1 in R19 NES

Agenda Item:	7.25.2.3

Document for:	Discussion

Introduction

In [1], the following objective is listed.

Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized Specify the corresponding core requirements, for the above features [RAN4].

Based on this, and latest RAN1/2 progress on this, we provide our views on the RRM requirement impacts from on-demand SIB1 in R19 NES.

Discussion

In the agreed WF in RAN4 #114 [R4-2502689], for OD-SIB1, the following is listed.

Issue 2-1-1: RRM requirements impacts for OD-SIB1 Agreement: RAN4 to discuss the potential RRM impact for OD-SIB1 from the next meeting.

Our understanding to this agreement is that, RAN4 may further discuss the RRM requirement impact from OD-SIB1 when the related RAN1/2 works are stable enough for RAN4 discussion.

Figure 1  Flow chart of OD-SIB1 procedure

Till RAN1 #119 and RAN2 #128, the related RAN1/2 agreements for OD-SIB1 is listed in the annex as reference. Based on these agreements and the WID, the basic UE procedure of OD-SIB1 is as follows, which is shown in Figure 1.

    - UE receives UL WUS configuration from cell A, via SIB. The UL WUS is for wake-up of NES cell. Triggering condition of the UL WUS can be configured to the UE in the UL WUS configuration.

    - After UE meets the WUS triggering conditions, UE shall send UL WUS to the NES cell. The UL WUS uses PRACH formats, but sending UL WUS does not trigger an initial access to any cell.

    - After UE transmits the PRACH, UE waits for msg2. Potential retransmission and power ramping are possible and the details are still being discussed in RAN1/2.

    - A successful decoding of msg2 means UE is going to try to obtain the SIB1 from NES cell within a time window. SIB1 repetition is performed within this window.

    - UE obtains the SIB1 from NES cell and evaluates whether UE shall camp on the NES Cell.

Note that in the above procedure, UE does not enter RRC connected mode. Only idle/inactive state needs to be considered. Moreover, according to RAN2 agreements in the annex, NES-capable UE actually performs cell re-selection to NES cell while non-NES-capable UE are prevented form reselecting to NES cell via Re-selection priority rules or ExcludedCellList.

Observation 1  OD-SIB1 related procedure only impacts UE behaviour in RRC idle/inactive states.

Observation 2  UE performs cell re-selection to NES cell with OD-SIB1 enabled, when UL WUS triggering condition is met, i.e. UL WUS triggering condition is the same as cell re-selection condition and OD-SIB1 acquisition shall be regarded as a special case of cell re-selection.

In legacy cell re-selection requirements, the related UE behaviour is classified into two parts:

1)  Performing re-selection evaluation based on RRM measurements, while paging reception is not interrupted.

2)  Performing target cell relevant SIBs reception, while paging is interrupted.

Figure 2 is an illustration of UE behaviour on above two parts. The overall NR cell re-selection delay requirements, which are tested since R15, is the summation of these two parts. In legacy cell re-selection, SIB1 reception and other SIBs reception are considered without UL WUS requests. SIB reception with UL WUS configured are not considered in legacy requirements. Therefore, it is RAN4’s responsibility to check the impacts on cell re-selection delay from R19 UL WUS procedure, especially for the 2nd part above, i.e. the impacts on paging interruption.

Figure 2  UE behaviour in idle/in-active state, which comprise 1) neighbour cell measurements when camping on a cell; 2) SIBs reception during cell re-selection procedure.

Note that in existing TS 38.133, as stated in Clause 4.2.2.6, the paging interruption requirements are specified without providing a specific value on TSI_NR, as cited below.

TS 38.133 v18.8.0 Clause 4.2.2.6 - 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms. TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell. These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

In TS 38.133 Clause A.6.1.1 test cases, TSI_NR is assumed as 1280ms for 1.28s DRX cycle, 0.64s DRX cycle and 0.32s DRX cycle. Checking Table 4.4.3.1.3-1, Table 4.4.3.1.3-2, Table 4.4.3.1.3-3 in TS 38.508-1, the max SIB periodicity assumed for cell re-selection test cases is 640ms. Therefore, our understanding is that, considering 0.64s DRX cycle length, 2 DRX cycles would be needed for all relevant SIBs reception, as shown in Figure 2.

TS 38.508-1 v18.4.0 - 4.4.3.1.2 Combinations of system information blocks for NR standalone and NE-DC … Combination NR-2 applies to the following test case scenarios: - NR FDD intra-frequency multi cell scenario - NR TDD intra-frequency multi cell scenario - NR FDD and NR TDD dual mode multi cell roaming scenario - NR FDD single cell RRM test case scenario - NR TDD single cell RRM test case scenario Combination NR-3 applies to the following test case scenarios: - NR FDD intra-frequency multi cell scenario with neighbouring cell related information - NR TDD intra-frequency multi cell scenario with neighbouring cell related information Combination NR-4 applies to the following test case scenarios: - NR FDD inter-frequency multi cell scenario - NR TDD inter-frequency multi cell scenario - NR FDD inter-band multi cell scenario - NR TDD inter-band multi cell scenario - NR FDD and NR TDD dual mode multi cell non-roaming scenario - NR FDD intra-band carrier aggregation component carriers cell scenario - NR FDD inter-band carrier aggregation component carriers cell scenario - NR TDD intra-band carrier aggregation component carriers cell scenario - NR FDD and NR TDD inter-band carrier aggregation component carriers cell scenario … ##### 4.4.3.1.3 Scheduling of system information blocks The scheduling configurations for combinations of system information blocks are defined in the following tables. There is no scheduling information for combination NR-1. Table 4.4.3.1.3-1: Scheduling for combination NR-2 Scheduling Information No.; Periodicity [radio frames]; Mapping of system information blocks. 1; 32; SIB2. Table 4.4.3.1.3-2: Scheduling for combination NR-3 Scheduling Information No.; Periodicity [radio frames]; Mapping of system information blocks. 1; 32; SIB2. 2; 64; SIB3. Table 4.4.3.1.3-3: Scheduling for combination NR-4 Scheduling Information No.; Periodicity [radio frames]; Mapping of system information blocks. 1; 32; SIB2. 2; 64; SIB4.

We are not sure whether 1280ms is still an adequate estimation for OD-SIB1 case. We would like to encourage RAN4 for further discussion.

Note that compared to Other SI, SIB1 is essential for cell re-selection. Therefore, although no RRM requirements were specified for OD-OSI, which is an R15 feature, in our view it would be important to specify RRM requirements for OD-SIB1.

Proposal 1  RAN4 further discuss the potential RRM requirement impacts on paging interruption for R19 OD-SIB1 related procedure.

As stated in the WID, UL WUS is sent to NES cell, but not cell A. For OD-SIB1 related procedure, the SSB from NES cell is always available, at least in this release. Therefore, the reference timing for UL WUS transmission needs to be clarified in TS 38.133. Note that this is not the first time to clarify UL timing reference for an UL signal in idle/inactive state. For R18 positioning, the UL timing for SRS during idle mode is already captured.

Proposal 2  For R19 OD-SIB1 related procedure, clarify DL timing reference of UL-WUS in TS 38.133. RAN4 may further discuss whether additional relaxation or additional applicability restrictions (e.g. SNR lower bound) on Te requirements is needed.

Another issue is that, in legacy, when UE performs RACH, UE will either setup or resume RRC connection. In other word, RACH procedure happens at the start or during RRC connected mode, where the UE power consumption for RACH power ramping is not a significant issue compared to traffic data latency. However, UL WUS for OD-SIB1 request is RACH procedure in idle mode, where UE has no traffic data but only paging or SIB reception to handle. According to TS 38.321, the RACH triggered for SI request shall be performed as follows.

TS 38.321 v18.4.0 Clause 5.1.2 1> if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and 1> if ra-AssociationPeriodIndex and si-RequestPeriod are configured: 2> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by ra-AssociationPeriodIndex in the si-RequestPeriod permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] corresponding to the selected SSB). 1> else if an SSB is selected above: 2> if the set of Random Access resources associated with Msg1 repetition is selected for this Random Access procedure: 3> determine the next available set of PRACH occasions (as specified in TS 38.213 [6]) for the Msg1 repetition number applicable for this Random Access procedure corresponding to the selected SSB (the MAC entity shall select a set of PRACH occasions randomly with equal probability amongst sets of PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB and selected Msg1 repetition number for this Random Access procedure; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available set of PRACH occasions corresponding to the selected SSB). 2> else: 3> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured, or indicated by PDCCH, or indicated by the LTM Cell Switch Command MAC CE (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected SSB).

Since UE only performs SSB measurement during its wake-up time in the idle/inactive mode, the selected RO shall also be during UE wake-up time in the idle/inactive mode. Hence, in our understanding, WUS transmission shall be only done when UE wakes up. Therefore, there are two potential possibilities for UE performing OD-SIB1 request, as shown in Figure 3, considering potential msg1 retransmission as RAN1/2 agreed.

Figure 3  Two options for OD-SIB1 request procedure. Option 1 assumes UE performs all WUS msg1 retransmission in one paging cycle. Option 2 assumes UE performs WUS msg1 retransmission in multiple paging cycles.

In Figure 3, Option 1 assumes UE shall perform all WUS msg1 retransmission in one paging cycle, if possible. Option 2 assumes UE may perform WUS msg1 retransmission in multiple paging cycles. Two different options may have different impacts on paging interruption requirements, including test cases. We are not sure following legacy principles, existing 1280ms in the test cases is still enough of OD-SIB1 related cell re-selection. Therefore, we would prefer to clarify in clause 4.2.2.6 of TS 38.133 that for OD-SIB1 relate cell re-selection, UL WUS transmission and msg2 reception shall be counted as necessary terms of paging interruptions.

Proposal 3 Clarify in TS 38.133 4.2.2.6 that, time for UL WUS msg1 transmission and time for msg2 reception are components of paging interruption for OD-SIB1 cell re-selection requirements.

In legacy paging interruption requirements, RAN4 assumes sufficient radio conditions, so that decoding of system information can be made without errors. This is because legacy cell re-selection is only related to downlink radio link quality judgements. In other word, UE may judge target cell downlink radio link quality before determining to perform cell re-selection to the target cell, and prevent such error case so as to ensure performance itself.

However, if UL WUS request for OD-SIB1 is considered, uplink radio link quality information, which is not available at UE side before any UL WUS transmission, may also impact the overall latency of cell re-selection. From UE power saving perspective, option 1 above shall be prevented if there is msg1 retransmission, especially if cell-reselection happens in consecutive paging cycles, e.g. when there are multiple NES cells, and UE bars NES cells one after another due to RACH failure. Therefore, it is important to allow UE to have more flexibility in determine how many RACH attempts would be required during each paging cycle. As shown in option 2 above, this would need relaxation on paging interruption requirements. It would be necessary to specify RRM requirements for the minimum msg1 attempts during each paging cycle, so as to balance between UE power saving and paging performance.

Proposal 4 RAN4 further discuss the RRM requirements on minimum WUS msg1 attempts during each paging cycle, as part of paging interruption requirements.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  OD-SIB1 related procedure only impacts UE behaviour in RRC idle/inactive states.

Observation 2  UE performs cell re-selection to NES cell with OD-SIB1 enabled, when UL WUS triggering condition is met, i.e. UL WUS triggering condition is the same as cell re-selection condition and OD-SIB1 acquisition shall be regarded as a special case of cell re-selection.

Proposal 1  RAN4 to further discuss the potential RRM requirement impacts on paging interruption for R19 OD-SIB1 related procedure.

Proposal 2  For R19 OD-SIB1 related procedure, clarify DL timing reference of UL-WUS in TS 38.133. RAN4 may further discuss whether additional relaxation or additional applicability restrictions (e.g. SNR lower bound) on Te requirements is needed.

Proposal 3  Clarify in TS 38.133 4.2.2.6 that, time for UL WUS msg1 transmission and time for msg2 reception are components of paging interruption for OD-SIB1 cell re-selection requirements.

Proposal 4  RAN4 to further discuss the RRM requirements on minimum WUS msg1 attempts during each paging cycle, as part of paging interruption requirements.

- References

[1]  RP-242354   Revised WID: Enhancements of network energy savings for NR, Ericsson.  RAN #105

- Annex 1 RAN1 agreements on R19 OD-SIB1

### RAN1#116(202402)

Agreement

For discussion purpose, the following assumption will be used in RAN1

- Cell A: A cell that is periodically transmitting at least its own SIB1

- NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 studies the following options.

On target cell of UL WUS transmission:

- Option 1: UE transmits UL WUS to NES Cell

- Option 2: UE transmits UL WUS to Cell A

On configuration provision for UL WUS transmission

- Option A: UE obtains the UL WUS configuration from NES Cell

- Option B: UE obtains the UL WUS configuration from Cell A

Other options are not precluded

Agreement

For further study of achievable NES gain with on-demand SIB1 for idle/inactive mode UE,

- Assume the following for network energy evaluation of non-NES cell in FR1:

  - Empty/low/medium cell load as defined in 38.864

  - Cat 1/Cat 2 BS as defined in 38.864

  - 30kHz SCS, DDDSU TDD pattern

    - Case A: 20ms SSB period with 20ms SIB1 period;

    - Case C: 20ms SSB period with 160ms SIB1 period;

    - Case D: 20ms SSB period with 40ms SIB1 period;

Note: Other SSB/SIB1 periodicity assumptions are not precluded (up to companies to report)

  - 4 or 8 SSBs in a SSB burst with SSB pattern case C

  - 20ms or 160ms PRACH monitoring period

- Assume the following for network energy evaluation of NES cell in FR1:

  - Empty/low/medium cell load as defined in 38.864

  - Cat 1/Cat 2 BS as defined in 38.864

  - 30kHz SCS, DDDSU TDD pattern

    - Case 1: 20ms SSB period with no SIB1 transmitted;

Note: Other SSB/SIB1 assumptions are not precluded (up to companies to report)

  - 4 or 8 SSBs in a SSB burst with SSB pattern case C

  - 20ms/160ms UL WUS monitoring period

- Note: SSB/CORESET0 multiplexing pattern 1 is used

Agreement

- For study of UL WUS design, consider at least PRACH as a starting point

  - FFS: Whether there is dedicated PRACH resource for SIB1 request

- Other option(s) not precluded

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 to discuss triggering conditions for sending UL-WUS.

Agreement

For the study of on-demand SIB1 for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported including associated details.

Agreement

For the further study on UL WUS configuration among the following options:

- Option 1: Pre-defined UL WUS configuration

- Option 2: UL WUS configuration that applies to multiple NES cell

- Option 3: UL WUS configuration that applies to a single NES cell

### RAN1#116bis(202404)

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases:

- Case 1: Option 1+A+X

- Case 2: Option 1+B+X

- Case 3: Option 2+B+Y

Where the options 1/2/A/B/X/Y are defined below:

- On target cell of UL WUS transmission:

  - Option 1: UE transmits UL WUS to NES Cell

  - Option 2: UE transmits UL WUS to Cell A

- On configuration provision for UL WUS transmission

  - Option A: UE obtains the UL WUS configuration from NES Cell

  - Option B: UE obtains the UL WUS configuration from Cell A

- On receiving of SIB1

  - Option X: UE receives on-demand SIB1 from NES Cell

  - Option Y: UE receives on-demand SIB1 from Cell A

Agreement

RAN1 to further study the following UE operation scenarios in the UL WUS design:

- Scenario 1: UE requests SIB1 to camp on NES cell

- Scenario 2: UE request SIB1 to perform random access procedure to make RRC connection to NES cell

Agreement

RAN1 to further study UE identification of NES cell with on-demand SIB1 based on one, both, or combination of the following options:

- Option 1: By WUS configuration

- Option 2: By PBCH payload of NES cell

Agreement

Companies to report at least the following key settings used in the evaluation/simulation of achievable NES gain with on-demand SIB1 in idle/inactive mode

- Setting A: SIB1 period (20ms/40ms/160ms)

- Setting B1: Cell load (Empty/low/medium)

- Setting B2: Traffic model

- Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

- Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

- Setting E: PRACH configurations (including PRACH configuration index in 38.211 Table 6.3.3.2-3) for WUS and initial/random access

- Setting F: Cat1/Cat2 BS

- Setting G: Number of SSB beams

- Setting H: NES gain/loss on Cell A

- Setting I: On-demand SIB1 transmission rate (how often UE requests on-demand SIB1)

Agreement

For further study of the NES gain/loss evaluation assumption on Cell A with on-demand SIB1 on NES cell for idle/inactive mode UE,

- Assume the following for network energy evaluation of Cell A in FR1:

  - Company to report among empty/low/medium cell load as defined in 38.864

  - Same Cat BS as the Non-NES cell

  - 30kHz SCS, DDDSU TDD pattern

    - Same SSB period as the Non-NES cell and company to report SIB1 period

  - Same number of SSBs in a SSB burst as the Non-NES cell with SSB pattern case C

  - 20ms PRACH configuration periodicity for WUS and/or initial access RACH and company to report RACH configuration index in 38.211 Table 6.3.3.2-3

  - Same SSB/CORESET0 multiplexing pattern and same SIB1 PDSCH time domain resource allocation as the Non-NES cell

  - Same traffic model as the Non-NES cell

  - Companies to report the assumption of WUS configuration provision or UL WUS monitoring or on-demand SIB1 transmission on Cell A if Case 2 (Option 1+B+X) or Case 3 (Option 2+B+Y) is considered

Agreement

For UL WUS design for SIB1 request, at least dedicated PRACH resource is the assumption for further study in RAN1

- FFS: Details on time, frequency, and/or PRACH preamble resources for UL WUS

- FFS: whether RACH resource for SIB1 request could be used for an initial access procedure and/or an on-demand SI procedure

Agreement

Conditions for triggering UL WUS transmission is up to RAN2. Any related work in RAN1 to be triggered by RAN2 LS. Send an LS to RAN2. Final LS in R1-2403779.

### RAN1#117(202405)

Agreement

For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3

- Case 2 (Option 1+B+X) is feasible from RAN1 perspective.

- Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

Agreement

For further study of on-demand SIB1 in idle/inactive mode, it is assumed that always-on SSB is transmitted on the NES cell with on-demand SIB1.

Agreement

At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point:

- Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window

  - FFS: How the search space zero configuration is provided (e.g. from  searchSpaceZero  in  MIB  or  from a new search space that is indicated by UL-WUS configuration)

  - FFS: Details of the time window, including at least the starting time and duration

  - FFS: Whether/how to support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS

Agreement

From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it.

At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

Agreement

For further study of on-demand SIB1 in idle/inactive mode, on the spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB, and UL WUS, as UL WUS is using dedicated PRACH resource, it is assumed that spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB and UL WUS can follow legacy mechanism.

Agreement

For further study of on-demand SIB1 in idle/inactive mode, use the following Table I (from R1-2405106, Ericsson) as a starting point to discuss the required parameters/contents inside the UL WUS configuration.

Table I.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| To which cell does the config applies | NES-CellId | PhysCellId |  |
|  |  | ARFCN-ValueNR |  |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResource | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |
|  |  | rsrp-ThresholdSSB |  |
|  |  | prach-RootSequenceIndex |  |
|  |  | msg1-SubcarrierSpacing |  |
|  |  | restrictedSetConfig |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  | frequencyInfoUL | frequencyBandList |  |
|  |  | absoluteFrequencyPointA |  |
|  |  | offsetToCarrier |  |
|  |  | p-Max |  |
|  |  | frequencyShift7p5khz |  |
| SIB1 reception | pdcch-ConfigSIB1 | ssb-SubcarrierOffset |  |
|  |  | controlResourceSetZero |  |
|  |  | searchSpaceZero |  |
| RAR Reception | pdcch-ConfigOD-SIB1-RAR | controlResourceSet |  |
|  |  | monitoringSlotPeriodicityAndOffset |  |
|  |  | Duration |  |
|  |  | monitoringSymbolsWithinSlot |  |
|  |  | aggregationLevels |  |

Conclusion

For further study of on-demand SIB1 in idle/inactive mode, enabling or disabling specific operations (e.g. paging, RACH receiving, OSI request …) of the NES cell with on-demand SIB1 is up to RAN2.

### RAN1#118(202408)

Agreement

For further work on on-demand SIB1 in idle/inactive mode, as a baseline, it is assumed that the transmit power control of UL WUS transmission based on PRACH is applied in the same manner as the legacy PRACH transmission.

- FFS: Potential optimization of the power ramp-up procedure

Agreement

For further work on on-demand SIB1 in idle/inactive mode related to dedicated PRACH resource usage, RAN1 to study the following options:

- Option 1 (shared RO): The dedicated WUS resource shares the same PRACH resource pool with PRACH resource for other usages.

- Option 2 (separated RO): The dedicated WUS resource uses an independent RACH resource pool with PRACH resource for other usages.

Agreement

RAN1 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Agreement

RAN1 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Agreement

At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options:

- Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission

- Option 2: starting time and duration are indicated in the UL WUS configuration

Agreement

At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options:

- Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission

  - FFS: Definition of RAR reception time

- Option 2: The reference time point is defined based on the UL-WUS transmission time

- Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission

Agreement

For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective.

- RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for Case 3

Conclusion

- For on-demand SIB1 in idle/inactive mode, RAN1 was not able to achieve consensus to support Case 1 (Option 1+A+X) in Rel-19, while Case 1 is technically possible from RAN1’s perspective.

Agreement

RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19.

- Note: RAN1 strive to minimize impact to legacy UE.

- Note: RAN1 specification impact to support this feature should be minimized.

Agreement

For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options:

- Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell

- Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration

- Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS.

Combination of multiple options is not precluded.

### RAN1#118bis(202410)

Conclusion

No further optimization in RAN1 on power control and power ramp-up procedure for UL WUS in R19.

Agreement

For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |
| --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |
|  |  | ARFCN-ValueNR |

Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell.

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration:

- rsrp-ThresholdSSB

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon

Agreement

For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration.

| Purpose | Parameters |
| --- | --- |
| WUS transmission | frequencyBandList |
|  | absoluteFrequencyPointA |
|  | offsetToCarrier |
|  | p-Max |
|  | frequencyShift7p5khz (working assumption)ULSubCarrierSpacing |

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | SSB-positionInBurst |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResources | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |

Note:

- In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB

- Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA

Agreement

For further work of on-demand SIB1 in idle/inactive mode, it is assumed that:

- SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster

- UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

Agreement

For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2.

Agreement

For The repetition periodicity of SIB1 within the time window of on-demand SIB1,:

- Up to NW implementation (no change to existing specification)

Agreement

RAN1 to discuss the contents of RAR in response to UL WUS using the legacy RAR for OSI as a starting point

Agreement

Search space for RAR in response to UL WUS on a NES Cell can be provided by the UL WUS configuration. If not, follow the search space zero for the NES Cell.

Agreement

It is up to gNB to configure whether RACH occasions for UL WUS are shared or separated from the RACH occasions for other usages.

### RAN1#119(20241118)

Agreement

The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration.

Agreement

For parameters agreed for UL WUS, whether it is mandatory or optional is for further discussion.

Agreement

At least for RO validation determination for WUS transmission, the parameter “ssb-PeriodicityServingCell” is included in the UL-WUS configuration at least for TDD system.

- FFS: FDD system

Agreement

Confirm the working assumption below from RAN1 #118-bis to include it in the UL-WUS configuration.

- (working assumption) ULSubCarrierSpacing

Agreement

CORESET0 of NES cell is used for RAR CORESET of that NES cell.

Agreement

At least for the case where SSB is transmitted on sync-raster, the indication of the quantity K_SSB (defined in TS 38.211 as subcarrier offset from subcarrier 0 in common resource block NCRBSSB to the lowest-numbered subcarrier of the SS/PBCH block) is included in the UL-WUS configuration for FDD and TDD NES cell.

Agreement

The reference time point to determine the window starting time for on-demand SIB1 is based on the RAR window for UL WUS wherein UE successfully received a RAR.

- FFS: Use starting or ending slot of the RAR window as reference time

Agreement

The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration.

- Unit of the duration is in slot

- FFS: Starting time

Agreement

On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, down-select from the following options:

- Option 1: Use it to indicate frequency assistance information to search SSB for Cell A

  - FFS: Details on the range/granularity of the frequency assistance information

  - FFS: Potential impact to RAN3

- Option 2: Use it to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1

- Option 3: Above feature is not supported.

Agreement

At least the contents of RAR in response to SI request are included in RAR in response to UL WUS

Conclusion

There is no RAN1 consensus to support UL WUS repetition in R19.

Agreement

The mapping rule between WUS resource ra-PreambleStartIndex for OD-SIB1 and SSB follows the mapping rule between ra-PreambleStartIndex for OSI request resource and SSB as in legacy specification.

- Annex 2 RAN2 agreements on R19 OD-SIB1

### RAN2#125bis(202404)

- At least RAN2 starts scenario 1a (Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell). Other scenarios are not excluded.

- RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1.

- UL WUS configuration includes at least RACH configuration.

- A UE needs to know a UL WUS configuration to request SIB1 of which cell.

- Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

- The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

### RAN2#126(202405)

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

### RAN2#127(202408)

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

### RAN2#127bis(202410)

Agreements on OD-SIB1

1.	We will inherit all agreements made during SI phase to WI phase.

2.	A NES cell can include neighbouring NES cell’s WUS configuration.

3.	NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

4.	In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

5.	The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

6.	The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

7.	A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

8.	If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

9.	Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

10.	NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

11.	RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

### RAN2#128(202411)

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.