3GPP TSG-RAN WG4 Meeting #114bis	R4-2503298

WuHan, China, 07 April – 11 April, 2025

Source: 	vivo

Title: 	Consideration on remaining RRM issues for LP-WUR

Agenda Item:	7.26.4.2

Document for:	Discussion

Introduction

In RAN Plenary #102 meeting, WI for “Low-power wake-up signal and receiver for NR” was agreed [1]. The WI was further updated at RAN Plenary #103 meeting as [2] and RAN #105 [3].

The objectives of WI are duplicated as below:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

  - Specify necessary RRM requirements

The initial discussion for this topic was initially discussed at RAN4 110bis meeting. The WF on this WI of RAN4 114 meeting and corresponding agreements can be found at [4]. The topic summary of RAN4 114 can be found at [5].

Discussion

General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

- Proposals

Regarding case 2:

P1: Support (LG CMCC vivo);

P2: De-prioritize (xiaomi oppo);

P3: Not consider (ZTE Apple Ericsson Huawei MTK);

The case to be considered for LP-WUR has been discussed, case 1 and case 3 have already been agreed.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

The remaining listed cases at RAN4 111 meeting are case 2 and case 4 where case 4 is covered by higher priority frequency search hence only case 2 is considered here.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |

As we indicated a few times based on figure 1, case 2 is a natural solution if the threshold between case 3 and case 1 is not the legacy for triggering neighbour cell measurement, i.e., whether a new threshold will be defined for entry/exit for case 1 or the legacy threshold to trigger neighbour cell measurement (intra or inter) is reused as the threshold for entry/exit of case 1.

Having more thresholds will provide extra NW implementation flexibility, of course, with the cost of increasing implementation complexity. In practice the thresholds used for triggering neighbour cell measurements could be configured close the edge of the cell coverage. Under this scenario, if case 1 is triggered by using the legacy threshold for neighbour cell measurement, the performance of LP-WUR maybe not at ideal since it is close to LP-SS coverage.

Figure 1 Illustration on fully offloading and relaxed cases of LP-WUR

If a new threshold for case 1 is introduced, as illustrated in figure 1, case 2 is a natural extension of case 3 when neighbour cell measurement is not needed, as mentioned before. Compared with case 1, case 2 create a different trade-off point between power consumption and mobility performance, since the measurement of MR on the serving cell may also help mobility performance. Hence, for the RRM relaxation for MR and serving cell RRM measurement offloaded to LP-WUR, besides case 1 and case 3, it is preferred to consider case 2.

Alternatively whether have case 2 or not can be up to RAN2’s decision.

- For the “Specify further RRM relaxation of UE MR for both serving and neighbour cell measurements” in the WID, besides agreed case 1 and case 3, case 2 is preferred to be considered. Alternatively whether have case 2 or not can be up to RAN2’s decision.

Issue 1-1-2: Threshold for switch between different cases

- Proposals

  - P1: Up to RAN2 decision (CMCC)

  - P2: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1). LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin). (Apple)

  - P3: (Huawei)

  - For switch from Case #1 to Case #3 based on LR measurement.

    - Option 1: Use existing thresholds for SS-RSRP and SS-RSRQ, and define new thresholds for LP-RSRP and LP-RSRQ

    - Option 2: Use existing thresholds for SS-RSRP and SS-RSRQ, and define offsets for LP-RSRP and LP-RSRQ

  - For switch from Case #3 to Case #1 based on LR or both LR and MR measurements.

    - Option 1: LR measurement only

    - Option 2: both LR and MR measurements, and UE stops neighbor cell measurement when both measurements are below their corresponding thresholds

  - P4: Need to discuss whether case 2 will be considered or not first (LG)

  - P5: New LP-based threshold(s) based on LP-RSRP or LP-RSRQ used for switch into case 1 from other cases or exit from case 1 to other cases are needed (ZTE vivo)

For issue 1-1-2, to transfer from case 1 to case 3 or case 2 (to be determined), a LR-based threshold should to be defined since MR measurement is not available anyway. The alternative method proposed for switching from case 1 to other cases by using existing thresholds for SS-RSRP and SS-RSRQ plus offset(s) is not preferred since the offset should also be configured by NW and the total efforts is not reduced compared with introducing a new LP-based threshold. Introducing new thresholds based on LP-RSRP and LP-RSRQ is also consistent with the following RAN2 #127 meeting agreement:

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

- Introducing new LP-based threshold(s) based on LP-RSRP and LP-RSRQ used for switch/exit from case 1 to other cases is necessary and details should be up to RAN2.

Issue 1-1-8: LP-WUR operating carrier frequency

For this issue, the latest reply LS from RAN2 [R2-2501394] is copied below:

RAN2 would like to thank RAN1 for the LS on LP-WUS operation in IDLE/INACTIVE mode in R2-2407921/R1-2407559. In addition to RAN2 reply LS in R2-2410957, RAN2 further discussed the details/pros/cons of solutions based on the way forward from RAN plenary in RP-242656. The corresponding discussion report is in R2-2501443. And RAN2 achieved the following conclusions:

- RAN2 understands that UE can report which band(s) is supported by LR to NW.

- RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

Based on RAN2’s reply LS, there is no RAN4 impact from this issue.

- No impact on RAN4 RRM requirements based on RAN2’s solution for issue 1-1-8.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

- Proposals

  - P1: LP-WUR is ON for serving cell measurement, but when and how to turn on LR for serving cell measurement is up to UE implementation. (LG Apple xiaomi CMCC oppo Huawei vivo, ZTE)

    - P1-1: No LP-WUR measurement and evaluation requirement is applicable when LR is “on” at the legacy state; (xiaomi CMCC oppo Huawei Ericsson vivo)

      - Note: in detail, no requirement is applicable on LR for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3).

  - P2: After MR measurement result is higher than MR threshold, UE turn on its LR. The requirement for LR measurement at legacy case should also be defined. (CMCC)

  - P3: LP-WUR status outside of the LP-WUS monitoring does not have impact to legacy procedures (Nokia)

Agreement:

- At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

- The entry/exit conditions between different cases are summarized in the following table 1:

Table 1 Summary on entry/exit conditions for LP-WUS monitoring, case 1 and case 3

| Case | Entry conditions | Exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | MR threshold and optional LR threshold | LR threshold |
| RRM measurement fully offloading (Case 1) | MR threshold and optional LR threshold | LR threshold |
| RRM measurement relaxation (Case 3) | For serving cell RRM relaxation, the entry condition can include both MR and LR measurements FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation | FFS |

- Since the entry conditions may include LP-based threshold, the LP-WUR should be ON to evaluate the LP-based threshold. In practice if the UE enters MR RRM relaxation or fully offloading cases early than enter into LP-WUS monitoring, there is no issue since the LP-WUR is already ON.

- However, one issue is when before starting LP-WUS monitoring, the UE is in the legacy scenario performing legacy serving cell or serving and neighbour cell measurement (either legacy Rel-15 scenario or legacy Rel-16 UE power saving scenario), then the LP-WUR still need “ON”, or at least to be “ON” some time before the entry conditions, following RAN2’s agreement.

- At previous meeting we have the agreement that “ when and how to turn on LR for serving cell measurement is up to UE implementation” and when LR is “on” at legacy state, whether the LR evaluation requirement apply or not still need discussion.

- Our view is at legacy the LR evaluation requirement does not apply even when LR is “ON” and evaluating LR related threshold. The reason is the configuration of the LR threshold at legacy state seems strange since the LR need be “ON” to determine whether the LR can be “ON” or not. The logic is not clear hence it is preferred that the LR evaluation requirement does not apply when LR is “ON’ at legacy state.

- At legacy state, when LP-WUR measurement is “ON” for evaluating LR threshold, LR evaluation requirement will not apply under this scenario, i.e., no LR entry threshold evaluation requirements for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3).

Issue 1-1-10: Considerations on higher priority frequency layer

- Proposals

  - P1: Support relaxed higher priority frequency layer search at case 1 and case 3 (xiaomi CT ZTE Huawei vivo Ericsson CT)

  - P1-1: For case 1 and case 3, when the serving cell is above threshold for inter-frequency neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search. (Ericsson vivo)

  - P1-2: If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #4), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed. If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements. (Huawei)

  - P2: Higher priority layer neighbou cell measurement shall also be OFF for case #1. (Apple vivo)

  - P3: When LR and MR operates at different band, whether to turn on higher priority measurement depends on the solution for issue 1-1-8. If Solution 1 for issue 1-1-8 is adopted, then the higher priority measurement can be OFF. If Solution 2 or 3 for issue 1-1-8 is adopted, the higher priority measurement should be ON (CMCC)

For higher priority frequency layer search, it could be stopped in case 1 since the MR is off in case 1. However since the frequency of higher priority frequency layer search is not high, especially considering the further relaxation defined in Rel-16, it is fine that the higher priority frequency layer search is performed in relaxed mode similar to what has been defined in Rel-16 UE power saving WI. For relaxed cases 3, higher priority frequency layer search could be performed in relaxed mode when relaxed conditions are satisfied.

In Rel-16, when the serving cell quality is higher than the threshold for inter-frequency neighbour cell measurement, relaxation on higher priority frequency layer search is performed as below and similar approach can be reused.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60.

- For case 1 and case 3, when the serving cell is above threshold for inter-frequency neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search.

Issue 1-1-12: On LR measurement metrics LP-RSRP and LP-RSRQ

- Proposals

  - P1: For frequency range 1, the reference point for the LP-RSRP and LP-RSRQ shall be the antenna connector of the UE (vivo)

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ agreed at RAN1 118 meeting are copied below:

Agreement

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows:

- LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS.

- LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS.

However, the reference point of the above measurement metrics has not yet been discussed and usually the reference point of measurement metrics should be discussed within RAN4.

The reference point of SS-RSRP and SS-RSRQ from TS38.215 are copied below:

5.1.1	SS reference signal received power (SS-RSRP)

| Definition | … For frequency range 1, the reference point for the SS-RSRP shall be the antenna connector of the UE. For frequency range 2, SS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRP value shall not be lower than the corresponding SS-RSRP of any of the individual receiver branches. |
| --- | --- |

5.1.3	SS reference signal received quality (SS-RSRQ)

| Definition | … For frequency range 1, the reference point for the SS-RSRQ shall be the antenna connector of the UE. For frequency range 2, NR Carrier RSSI shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch, where the combining for NR Carrier RSSI shall be the same as the one used for SS-RSRP measurements. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRQ value shall not be lower than the corresponding SS-RSRQ of any of the individual receiver branches. |
| --- | --- |

During the NR reference point discussion, the reference point for FR1 is relative straightforward and it took few meetings to achieve consensus for FR2 reference point. For the LP-WUR, for FR1 the reference point for SS-RSRP can be reused. For FR2, the reference point definition is up to further discussion.

- For frequency range 1, the reference point for the LP-RSRP and LP-RSRQ shall be the antenna connector of the UE.

Issue 1-1-13: Impact on specs

Based on the WI and current progress on this WI, the following impacts on core specs are provided. The impacts are mainly on idle/inactive state requirements and based on the WI, there will be no RAN4 scope on connected state requirements.

|  | Clause | Impact Comment |
| --- | --- | --- |
| 3 | Definitions, symbols and abbreviations | Yes |
| 4 | SA: RRC_IDLE state mobility | Yes |
| 5 | SA: RRC_INACTIVE state mobility | Yes. |
| 6 | RRC_CONNECTED state mobility | No |
| 7 | Timing | No |
| 8 | Signalling characteristics | [No] |
| 9 | Measurement Procedure | [No] |
| 10 | Measurement Performance requirements | [No] |
| Annex B | Conditions for RRM requirements applicability for operating bands | Yes |

In detail, the following observation is available for new sections, updates and clarifications:

3	Definitions, symbols and abbreviations

- 3.1 Definitions

- 3.2 Symbols

- 3.3 Abbreviations

- 3.5 Frequency bands grouping (clarification on current frequency band grouping is applicable for LP-WUR may needed)

- 3.6 Applicability of requirements in this specification version

4  SA: RRC_IDLE state mobility

- 4.X LP-WUR measurements

  - 4.X.1	Introduction

  - 4.X.2	Requirements

    - 4.X.2.1	[LP-WUR and MR measurement capability]

    - 4.X.2.2	[LP-WUR serving cell measurement and evaluation requirements]

    - 4.X.2.3	[MR serving cell measurement and evaluation requirements]

    - 4.X.2.4	[Measurements of intra-frequency NR cells for UE with LP-WUR]

    - 4.X.2.5	[Measurements of inter-frequency NR cells for UE with LP-WUR]

    - 4.X.2.6	[Measurements of inter-RAT E-UTRAN cells for UE with LP-WUR]

    - [4.X.2.7	Maximum MR activation delay with LP-WUS operation]

5  SA: RRC_INACTIVE state mobility

- 5.X LP-WUR measurements

  - 5.X.1	Introduction

  - 5.X.2	Requirements

    - 5.X.2.1	[LP-WUR and MR measurement capability]

    - 5.X.2.2	[LP-WUR serving cell measurement and evaluation requirements]

    - 5.X.2.3	[MR serving cell measurement and evaluation requirements]

    - 5.X.2.4	[Measurements of intra-frequency NR cells for UE with LP-WUR]

    - 5.X.2.5	[Measurements of inter-frequency NR cells for UE with LP-WUR]

    - 5.X.2.6	[Measurements of inter-RAT E-UTRAN cells for UE with LP-WUR]

    - [5.X.2.7	Maximum MR activation delay with LP-WUS operation]

Annex B (normative): Conditions for RRM requirements applicability for operating bands

- [B.1.x	Conditions for measurements on NR serving cells with LP-WUS operation]

New sections, updates and clarifications for LP-WUR WI

In detail, the following content will be added in each new sections.

| Sections | New Content to be added |
| --- | --- |
| - 3. Definitions, symbols and abbreviations | - All related terms - 3.5 Frequency bands grouping (clarification on current frequency band grouping is applicable for LP-WUR may needed) |
| - 4.X.1 Introduction | Introduction of the feature |
| - 4.X.2.1 [LP-WUR and MR measurement capability] | - LP-WUR capability on monitoring serving cell; - Duplicate MR capability |
| - 4.X.2.2 [LP-WUR serving cell measurement and evaluation requirements] | - General description - LP-WUR evaluation requirement (OOK and OFDM based) - LP-WUR measurement periodicity, filtering and [measurement requirements] (OOK and OFDM based) - LR-WUR measurement accuracy，represented by margins (OOK and OFDM based) |
| - 4.X.2.3 [MR serving cell measurement and evaluation requirements] | - General description - MR measurement and evaluation requirements for cell selection - With MR measurement relaxation - Without MR measurement relaxation (same as legacy) - MR measurement and evaluation requirements for [LP-WUS and MR offloading] - With MR measurement relaxation - Without MR measurement relaxation - MR measurement and evaluation requirements for [MR RRM relaxation] - MR Serving cell offloading - MR Serving cell RRM measurement relaxation requirements |
| - 4.X.2.4 [Measurements of intra-frequency NR cells for UE with LP-WUR] | - MR Intra-frequency neighbour cell RRM measurement relaxation requirements |
| - 4.X.2.5 [Measurements of inter-frequency NR cells for UE with LP-WUR] | - MR Inter-frequency neighbour cell RRM measurement relaxation requirements - Higher priority frequency layers related requirements |
| - 4.X.2.6 [Measurements of inter-RAT E-UTRAN cells for UE with LP-WUR] | - MR Inter-RAT E-UTRAN cells RRM measurement relaxation requirements - Higher priority frequency layers related requirements |
| [4.X.2.7 Maximum MR activation delay with LP-WUR operation] | - TBD |
| [B.1.x Conditions for measurements on NR serving cells with LP-WUS operation] | - OFDM based LP-WUR SSB Ês/Iot and LP-SS Ês/Iot - Minimum SSB_RP or LP-SS_RP for each NR operating band groups |

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1: LP-WUR accuracy and side conditions requirements

- Proposals

  - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (xiaomi LG Apple CT Huawei Ericsson MTK)

Regarding the accuracy requirements, no dedicated accuracy requirement could be fine since it follows traditional RAN4’s methodology. However currently in RAN4 specs these margins are only defined in the sections for intra/inter/inter-RAT measurement requirements and there is no margin defined for the serving cell measurement. For LP-WUR, it is possible to introduce a margin to reflect accuracy performance when LP-WUR serving cell measurement results is used to compare with a threshold since it is a new introduced receiver.

- It is fine to use P1, “introduce a margin to reflect accuracy performance of LP-WUR in core specs”, when LP-WUR serving cell measurement results is used to compare with a threshold.

Regarding the concrete number of accuracy, at RAN4 112bis meeting the following assumption for accuracy for LP-RSRP and LP-RSRQ are agreed for simulation purpose:

Agreement:

Candidate absolute baseband accuracy to derive the number of samples for simulation alignment purpose only:

- For FR1:

  - For OOK based LP-WUR, {3.5, 2.5} dB for LP-RSRP and {3.5, 2.5} dB for LP-RSRQ

    - 1st priority: 3.5dB, 2nd priority: 2.5dB

  - For OFDM based LP-WUR, {3.5, 2.5} dB for SS-RSRP and {3.5, 2.5} dB for SS-RSRQ

    - 1st priority: 3.5dB, 2nd priority: 2.5dB

Although the simulation results need more alignments, based on simulation results so far, it seems no serious issue is identified by using 3.5 dB accuracy. In addition, considering the SS-RSRP and SS-RSRQ accuracy, at connected state the accuracy range of SS-RSRP is larger compared with the range of SS-RSRQ since RSRP accuracy requirement includes the impact of the RF impairment and a margin is added whose typical value is from 2-3 dB. At idle/inactive state accuracy is not directly defined however it is reflected as a margin and the margin of SS-RSRP is also larger than that for SS-RSRQ due to the above reason. Hence, 3.5 dB can be used as the base for core requirements of LP-RSRQ and a relaxed value 5.5 dB can be considered for core requirements of LP-RSRP with [2] dB margin accounting for RF impairment, under the side conditions LP-SS Ês/Iot [x], with OOK based LR or type A NF. Same values of margins are suggested to used for SSB based LR, under the side conditions SSB Ês/Iot [y] or type B NF, with OFDM based LR NF. x and y could be -3dB.

Regarding side conditions, in legacy requirements, the conditions for NR RRC_IDLE state mobility is specified in section Annex B. For example, conditions for measurements on NR intra-frequency cells for cell re-selection is specified in the following table:

Table B.1.2-1: Conditions for intra-frequency cell re-selection in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
|  | NR_FDD_FR1_N | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

To our understanding the Minimum SSB_RP is derived from the reference sensitivity defined in the RF session. For LP-WUR, current in the RF session two categories of LR reference sensitivity were discussed. Hence we suggest the Ês/Iot could be [-3 dB] and two tables of side conditions are specified and each table has the minimum [LR_RP] corresponding to one particular category of LR reference sensitivity defined by RF session.

- [3.5] dB can be used as the base for core requirements of LP-RSRQ and [5.5] dB value can be considered for core requirements of LP-RSRP, under the side conditions Ês/Iot [x, x = -3] dB, with type A [or OOK LR] reference sensitivity. Same values of margins are suggested to be used for type B [or OFDM based] LR, under the side conditions Ês/Iot [y, y = -3], with type B reference sensitivity.

- Two tables of side conditions are specified for LR in related section of Annex B. Ês/Iot in each table is -3 dB and the minimum [LR_RP] in each table is corresponding to one particular category of LR reference sensitivity defined by RF session.

Issue 1-2-2: Periodicity for LP-WUR RRM requirements

- Proposals

  - P1: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. (LG Apple CMCC oppo)

  - P2: For periodicity for SSB based LP-WUR measurement delay requirements, when LP-SS is configured, the periodicity for SSB based LP-WUR measurement delay requirements can be max { LO periodicity, LP-SS periodicity}. (CATT)

  - P3: For periodicity for SSB based LP-WUR measurement delay requirements, postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity. (Huawei)

  - P4: For periodicity for SSB based LP-WUR measurement delay requirements, no requirement is defined if LO periodicity is smaller than LP-SS periodicity.(Ericsson)

The agreements of RAN4 113 meeting are copied below:

Agreement:

If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements

- based on LO periodicity

  - FFS whether to define an upper bound

  - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement.

Agreement:

When LP-SS is configured, apply the same agreement as for when LP-SS is not configured

- The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

To our understanding, LO periodicity is always >= LP-SS periodicity, hence P1 is ok.

- Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

- Proposals

  - P1: In case the periodicity of LP-SS can be configured to small value like 80ms or 160ms (xiaomi)

    - Option 1: define the lower limit for measurement delay requirements;

    - Option 2: define the measurement delay requirements based on max(LP-SS periodicity, LO periodicity).

  - P2: Periodicity for measurement and evaluation delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode is 320ms if LP-SS periodicity 80ms and 160ms are determined to be included. (Huawei vivo)

  - P3: No need for lower bound, before RAN1 agrees other periodicities. (Nokia)

It is ok to wait for more RAN1 agreement before determine whether a lower bound is needed or not. If in the end only 320ms periodicity is available for LP-SS, lower bound is not necessary.

- Wait for more RAN1 progress before determining whether to have lower bound.

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity

- Proposals

  - P1: Do not define upper bound for SSB-based LP-WUR measurement periodicity. (xiaomi CMCC oppo Huawei)

  - P2: Prefer the periodicity for measurement and evaluation delay requirements for LP-WUR based on SSB in IDLE/Inactive mode is 1.28s if the LO periodicity is 2.56s. (vivo)

- For the upper bound on SSB based LR measurement periodicity, 2.56s may too longer and an upper bound is preferred.

Issue 1-2-4: On LR measurement/evaluation requirements

Issue 1-2-4-0: On LR measurement requirement

- Proposals

  - P1: For LR measurement requirements, the following options are available (vivo)

    - Option 1: Define LR measurement requirement as the minimum duration where one LR measurement shall be executed, for example  like “ The UE shall measure the LP -RSRP and LP -RSRQ level of the serving cell at least once every LP-SS periodicity for LP-SS based LR or [LO periodicity] for SSB based LR”

    - Option 2: Define LR measurement requirement as the minimum duration to meet accuracy requirement as [x] * LP-SS periodicity for LP-SS based LR or [y] * LO periodicity for SSB based LR, assuming [x] samples and [y] samples are number of samples to meet LP-SS based LR or SSB based LR accuracy requirement.

At RAN4 114 meeting we have the following agreement:

Issue 1-2-4-2-3: On how to define LR evaluation requirements

Agreement:

LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y.

Number x and y are defined as measurement requirement as  [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

- Based on RAN4 114 agreement, if necessary, LR measurement requirement can be defined as:

- [x samples]*LP-SS periodicity (for OOK based LR) or [y samples] *LO periodicity (for SSB based LR), assuming [x] samples and [y] samples are number of samples to meet LP-SS based LR or SSB based LR accuracy requirement.

Issue 1-2-4-1: On LR measurement filtering requirement

- Proposals

  - P1: The UE shall filter the LP-RSRP and LP-RSRQ and [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by either LP-SS periodicity/2 for LP-SS based LR or [LO periodicity /2] for SSB based LR. (Huawei Ericsson vivo)

For the filtering related requirements, the following could be used:

- The UE shall filter the LP-RSRP and LP-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by LP-SS periodicity/2 for LP-SS based LR.

- The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR.

Issue 1-2-4-2: On LR requirements for entry/exit threshold evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3)

Issue 1-2-4-2-1: On applicability LR evaluation requirements

- Proposals

  - P1: The LR evaluation requirements will apply for the following scenario (vivo):

    - entry to legacy case from case 1  (Xiaomi)

    - entry to case 1(RRM measurement fully offloading) from case 3 (Xiaomi)

    - [entry to legacy case from LP-WUR monitoring case]

    - [entry to legacy case from case 3]  (Xiaomi)

    - [entry to case 3(RRM measurement relaxation) from case 1] (Xiaomi)

  - P2: The LR evaluation requirements will apply for the exit condition from case 1(LG)

  - P3: RAN4 shall wait for RAN2 to define the relaxed measurement condition for serving cell measurement if it has; and wait for RAN2 to define the final relaxed measurement conditions for neighbour cell measurement. (ZTE)

  - P4: which requirements to be used for each case depends on the threshold configured and can be left to RAN2 to decide. (OPPO)

  - P5 (Apple):

    - to check the criteria for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation(case 3), use the MR measurement period and/or LR measurement period, i.e., MR measurement delay or LR measurement delay are sufficient for criteria checking; and do not need a dedicated evaluation delay requirement (e.g., in how long time the MR/LR measurement results can meet the condition).

    - to check the criteria for MR fully offloading (case 1), use the MR measurement period and/or LR measurement period for measurement filtering; and need a dedicated evaluation delay requirement (i.e., in how long time the MR/LR measurement results can meet the criteria).

  - P6: RAN4 to focus on discussing requirements for LP-WUR for entry/exit criteria(threshold) evaluation for MR RRM relaxation, whereas the LR-WUS monitoring requirement discussion pending on RAN2’s progress. (Xiaomi)

Background:

Recommendations:

Suggest to confirm at least the following two applicability scenarios for LR evaluation requirements.

- entry to legacy case from case 1

- entry to case 1(RRM measurement fully offloading) from case 3

For this issue we agree with P1 and have the following proposal:

- The LR evaluation requirements will apply for the following scenario:

- exit to legacy case from case 1

- entry to case 1(RRM measurement fully offloading) from case 3

- entry to case 3(RRM measurement relaxation) from case 1

- [exit from LP-WUR monitoring case to legacy case]

- [exit to legacy case from case 3]

- [Entry to LP-WUR monitoring, to RRM measurement fully offloading (case 1), to RRM measurement relaxation (case 3) from legacy case]

Issue 1-2-5: On requirements for MR for entry/exit threshold evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3)

Issue 1-2-5-1: On applicability MR evaluation requirements

- Proposals

  - P1: The MR evaluation requirements will apply for the following scenario (vivo):

    - Entry to LP-WUR monitoring, to RRM measurement fully offloading (case 1), to RRM measurement relaxation (case 3) from legacy case (xiaomi)

    - [entry to legacy case from case 3]

    - [entry to case 1 (RRM measurement fully offloading) from case 3]

  - P2: The MR evaluation requirements will apply for the entry condition to case 3 (LG)

  - P3: which requirements to be used for each case depends on the threshold configured and can be left to RAN2 to decide. (OPPO)

  - P4 (Apple):

    - to check the criteria for MR fully offloading (case 1), use the MR measurement period and/or LR measurement period for measurement filtering; and need a dedicated evaluation delay requirement (i.e., in how long time the MR/LR measurement results can meet the criteria).

The MR measurement and evaluation should also be defined for LP-WUR monitoring, case 1 and case 3. Based on table 1, the application of the MR evaluation requirements is described in the following propose.

- The MR evaluation requirements will apply for the following scenario:

- Entry to LP-WUR monitoring, to RRM measurement fully offloading (case 1), to RRM measurement relaxation (case 3) from legacy case

- [exit to legacy case from case 3]

- entry to case 1(RRM measurement fully offloading) from case 3

Issue 1-2-5-2: On MR evaluation requirements

- Proposals

  - P1: For MR evaluation requirement for LP-WUR monitoring, case 1 and case 3, consider the following options (vivo):

    - Option 1: reuse Nserv ;

    - Option 2: reduce Nserv value by half;

    - Option 3: Nserv  always = 1; (LG xiaomi)

In the legacy specification, the MR measurement and evaluation for the serving cell is quite mature and the detail is as below:

“…

the UE has evaluated according to Table 4.2.2.2-1 or Table 4.2.2.2-4 in Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.”

To our understanding, the frame of the above specs should be reused for MR evaluation requirement for LP-WUR monitoring, case 1 or case 3 and only the value of Nserv needs further investigation. Regarding detail requirements, firstly whether M1 needs kept or not needs be discussed and current M1 is kept. Regarding Nserv_WUR, 3 options are provided. Option 1 simply reuses the current Nserv value. Option 2 reduces current Nserv value by half considering the threshold for LP-WUR monitoring, case 1 and case 3 should be much higher than the cell selection threshold hence less samples could be used for evaluating purpose.

At Rel-16 UE power saving WI, the fulfilment of the not at cell edge criteria is defined as below,

The evaluation duration is not explicitly specified however it was agreed in [6] that:

- The evaluation rate for measurement relaxation mode triggering shall be the same as current serving cell evaluation rate defined in 38.133.

And the serving cell evaluation rate defined in 38.133 is copied below:

4.2.2.2	Measurement and evaluation of serving cell

The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle;

Hence one evaluation can be performed in M1*N1 DRX according to Rel-16 UE power saving WI, which is the option 3 in the following table.

Table 2 Nserv_WUR for MR evaluation requirement for LP-WUR monitoring, case 1 and case 3

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Option 1: Nserv_WUR [number of DRX cycles] | Option 2: Nserv_WUR [number of DRX cycles] | Option 3: Nserv_WUR [number of DRX cycles] |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1 | FR2-2 |  |  |  |
| 0.32 | 1 |  |  | M1*N1*4 | M1*N1*2 | M1*N1 |
| 0.64 | 1 |  |  | M1*N1*4 | M1*N1*2 | M1*N1 |
| 1.28 | 1 |  |  | N1*2 | N1*1 | N1 |
| 2.56 | 1 |  |  | N1*2 | N1*1 | N1 |

As mentioned above, the threshold for LP-WUR monitoring, case 1 and case 3 should be much higher than the cell selection threshold hence less samples could be used for MR evaluating purpose. In addition, the evaluation process of MR is be slower compared with that of LR, for example when the MR DRX is 2.56 and the LR’s LP-SS periodicity is still 320 ms, especially when the UE is in case 3 where the relax factor is 16. To reduce the gap between the availability of the MR evaluation outcome and LR evaluation outcome on the time domain, option 3 is preferred to be used as the MR evaluation requirement. Anyway, option 3 has been used for Rel-16 UE power saving WI.

- For MR evaluation requirement for LP-WUR monitoring, case 1 and case 3, consider Option 3: Nserv =1.

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR

- Proposals

  - P1: MR wake-up time is pending on UE capability which can be 400ms and 800ms based on RAN1 agreement; MR sync time, need to be further discuss with different side conditions (LG)

    - Condition 1: MR and LR well sync, and LR sync info can be fully reused e.g.  1~2 SSB sufficient

    - Condition 2: MR and LR not well synced and MR need to apply sync procedure e.g. at least 10 SSBs required

  - P2: Wait for RAN1 further progress. (CMCC CATT oppo Huawei Ericsson)

  - P3: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. (Apple)

  - P4: After MR is triggered to be waken up and during the waking up duration, MR should follow SSB periodicity instead of DRX periodicity to perform measurement (vivo)

  - P5: Defining MR wake up requirement for the normal case and the “wake up delay” duration for the normal is: “wake up delay” duration = Ramping duration + [5] samples * SSB periodicity; Whether to further consider a worst case where a cell reselection is needed after ME is wake up needs FFS. (vivo)

  - P6: RAN4 shall define the minimum required time for MR synchronisation and related side conditions (Nokia)

For issue related to MR wake up delay, the following alternatives will be considered by RAN1 #120 meeting:

Agreement

For UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

  - FFS: different sets of 3 candidate value for different SSB periodicities

- Send a LS to RAN4 after the down-selection to confirm the number of SSBs. Final LS in R1-2501624.

The content of R1-2501624 is copied below:

In the LS the number of SSBs assumed by RAN1 should be checked by RAN4. In our previous contribution we already provided analysis on this part [7], [8]. For a MR to get synchronous after triggered from ultra-deep sleep mode, this component will consist of at least the time for AGC, raw timing sync and other related procedure if necessary. Typically 1-2 SSBs (1-2 samples) are needed for AGC and 1-2 SSBs are needed for raw timing sync respectively. When the MR is wake up from the offloading status (ultra-deep sleep) by LP-WUS, normally its serving cell measurement should be in good status and a few samples (1-2 SSBs) is sufficient for MR measurement purpose otherwise it will exit these modes. As the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring is considered for definition in RAN1, the RAN1’s assumption of 5 SSBs from ultra-deep sleep mode is reasonable. In addition, 3 SSBs is considered when UE is wake up from deep-sleep mode, the value is reasonable since the number of samples used for AGC, raw synchronization and measurement can be reduced, if not fully removed, compared with that of the waking up from ultradeep-sleep mode scenario.

In addition, when the SSB periodicity is 5ms or 10ms, a lower bound may necessary to avoid too demanding requirement on the UE processing capability. Hence when the SSB periodicity is 5ms or 10ms, a virtual SSB periodicity 20ms should be used, for example for the 70ms case, when the SSB periodicity is 5mr or 10ms, the MR wake up delay should be 10ms + 3*20ms instead of 10ms + 3*5ms or 10ms + 3*10ms.

- RAN4 confirm the number of SSBs used for MR wake up delay definition assumed by RAN1, in particular, 5 SSBs used for the wake up delay from ultra-deep sleep mode and 3 SSBs used for the wake up delay from deep-sleep mode, is right.

- RAN4 further confirms that it is reasonable to assume that the same number of SSBs for MR synchronization for different SSB periodicities.

- When the SSB periodicity is 5ms or 10ms, 20ms should be used as a virtual SSB periodicity.

Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1

- Proposals

  - P1: No need to define the LR-WUR based cell selection criterion evaluation in case 1. (Apple CMCC vivo Huawei MTK)

  - P2: Define LP-WUR based serving cell selection criterion S evaluation requirement in case 1. (CATT Ericsson)

  - P2-1：The existing MR based evaluation requirements can be the baseline (CATT)

For the cell selection evaluation requirement for case 1, the cell selection for LP-WUR is used to trigger MR to perform neighbor cell measurement, as indicated in P3-1, however the exit conditions of case 1 serve the same purpose and may even with a higher exit threshold hence the necessity to have a cell selection evaluation requirements for LP-WUR is low.

- For issue on LP-WUR cell selection evaluation requirements for case 1, support P1, i.e., no need to define the LR based cell selection criterion evaluation in case#1.

Issue 1-2-8-1: Accuracy for normal or relaxed MR serving cell measurement

- Proposals

  - P1: No new accuracy requirements will be introduced for MR either normal or relaxed serving cell measurement,

    - no margin will be introduced for MR serving cell measurement in specs. (vivo)

    - same margins as existing MR neighbour cell measurement apply (Huawei)

  - P2: To enable reliable offloading from MR it is necessary to have absolute measurement accuracy requirements defined for MR. The absolute measurement accuracy already defined for idle mode can be re-used when defining idle mode measurements for LR offloading purposes (Nokia)

Recommendations:

Check P1 is agreeable

For accuracy for MR, margins, which reflect accuracy requirements, are directly defined for intra-frequency/inter-frequency and inter-RAT measurements. However margins are not explicitly defined for serving cell measurement when serving cell measurement is used to compared with a threshold, either for cell selection or for triggering intra/inter frequency measurement or for triggering Rel-16 RRM relaxation.

Since the margin has never been explicitly defined for the MR serving cell measurement before, following the same logic, it is not necessary to have any explicitly margin defined when the MR serving cell measurement is used to compare with thresholds for LP-WUR operation.

Issue 1-2-8-2: Accuracy for relaxed MR neighbour cell measurement

- Proposals

  - P1: The legacy accuracy for relaxed MR measurement should be reused if Case#2 and/or Case#3 introduced. (CMCC vivo Huawei)

For the accuracy requirements for relaxed MR measurement, the legacy requirements are used during Rel-16 UE power saving WI. It is straightforward to the use same principle, i.e., use the legacy accuracy requirements for relaxed MR measurement.

- For MR, margin is not explicitly specified in specs when MR serving cell measurement result comparing with a threshold as that of the legacy specs. For relaxed MR measurement, the legacy accuracy requirements for MR are re-used.

MR RRM relaxation

LP-WUR connected mode

Issue 1-4-1: LP-WUR at CONNECTED mode

- Proposals

  - P1: No RRM objectives is expected for connected mode in this WI. (CATT xiaomi oppo ZTE Apple CT MTK)

  - P2: Postponed until more progress in other groups (Ericsson Huawei vivo)

  - P3: RAN4 to introduce interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. The interruption on WUS monitoring follows the current interruption requirements on DL data during MGs. (Qualcomm)

Regarding P4, the assumption is that MR and LR works at different time, when the WUS monitoring occasion collides with a MG, one possible solution is the MR uses the MG to perform measurement and LR monitors LP-WUR after MG. However the detailed design could be up to RAN1 design and since there is no RAN4 scope on CONNECTED mode, it is suggested to postpone the discussion on this issue.

- For issue 1-4-1, postpone the discussion.

Other issues

Issue 1-5-1: LR based RRM with EMR in IDLE/inactive mode

- Proposals

  - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple Nokia)

  - P2: Postpone the discussion on the EMR issue (CMCC vivo)

  - P3: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact (Ericsson)

Regarding EMR related issue, to our understanding the principle P1 is reasonable. The issue itself is not crucial for Rel-19 requirements point of view and can be discussed later.

- For issue 1-5-1, the principle of P1 is reasonable. The issue can be discussed later.

Conclusion

In this contribution, we provide our considerations on the LP-WUS/WUR WI and have the following observations and proposals.

- For the “Specify further RRM relaxation of UE MR for both serving and neighbour cell measurements” in the WID, besides agreed case 1 and case 3, case 2 is preferred to be considered. Alternatively whether have case 2 or not can be up to RAN2’s decision.

- Introducing new LP-based threshold(s) based on LP-RSRP and LP-RSRQ used for switch/exit from case 1 to other cases is necessary and details should be up to RAN2.

- No impact on RAN4 RRM requirements based on RAN2’s solution for this issue 1-1-8.

- At legacy state, when LP-WUR measurement is “ON” for evaluating LR threshold, LR evaluation requirement will not apply under this scenario, i.e., no LR entry threshold evaluation requirements for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3).

- For case 1 and case 3, when the serving cell is above threshold for inter-frequency neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search.

- For frequency range 1, the reference point for the LP-RSRP and LP-RSRQ shall be the antenna connector of the UE.

- It is fine to use P1, “introduce a margin to reflect accuracy performance of LP-WUR in core specs”, when LP-WUR serving cell measurement results is used to compare with a threshold.

- [3.5] dB can be used as the base for core requirements of LP-RSRQ and [5.5] dB value can be considered for core requirements of LP-RSRP, under the side conditions Ês/Iot [x, x = -3] dB, with type A [or OOK LR] reference sensitivity. Same values of margins are suggested to be used for type B [or OFDM based] LR, under the side conditions Ês/Iot [y, y = -3], with type B reference sensitivity.

- Two tables of side conditions are specified for LR in related section of Annex B. Ês/Iot in each table is -3 dB and the minimum [LR_RP] in each table is corresponding to one particular category of LR reference sensitivity defined by RF session.

- Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

- Wait for more RAN1 progress before determining whether to have lower bound.

- For the upper bound on SSB based LR measurement periodicity, 2.56s may too longer and an upper bound is preferred.

- Based on RAN4 114 agreement, if necessary, LR measurement requirement can be defined as: [x samples]*LP-SS periodicity (for OOK based LR) or [y samples] *LO periodicity (for SSB based LR), assuming [x] samples and [y] samples are number of samples to meet LP-SS based LR or SSB based LR accuracy requirement.

- The UE shall filter the LP-RSRP and LP-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by LP-SS periodicity/2 for LP-SS based LR.

- The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR.

- The LR evaluation requirements will apply for the following scenario:

- exit to legacy case from case 1

- entry to case 1(RRM measurement fully offloading) from case 3

- entry to case 3(RRM measurement relaxation) from case 1

- [exit from LP-WUR monitoring case to legacy case]

- [exit to legacy case from case 3]

- [Entry to LP-WUR monitoring, to RRM measurement fully offloading (case 1), to RRM measurement relaxation (case 3) from legacy case]

- The MR evaluation requirements will apply for the following scenario:

- Entry to LP-WUR monitoring, to RRM measurement fully offloading (case 1), to RRM measurement relaxation (case 3) from legacy case

- [exit to legacy case from case 3]

- entry to case 1(RRM measurement fully offloading) from case 3

- For MR evaluation requirement for LP-WUR monitoring, case 1 and case 3, consider Option 3: Nserv =1.

- RAN4 confirm the number of SSBs used for MR wake up delay definition assumed by RAN1, in particular, 5 SSBs used for the wake up delay from ultra-deep sleep mode and 3 SSBs used for the wake up delay from deep-sleep mode, is right.

- RAN4 further confirms that it is reasonable to assume that the same number of SSBs for MR synchronization for different SSB periodicities.

- When the SSB periodicity is 5ms or 10ms, 20ms should be used as a virtual SSB periodicity.

- For issue on LP-WUR cell selection evaluation requirements for case 1, support P1, i.e., no need to define the LR based cell selection criterion evaluation in case#1.

- For MR, margin is not explicitly specified in specs when MR serving cell measurement result comparing with a threshold as that of the legacy specs. For relaxed MR measurement, the legacy accuracy requirements for MR are re-used.

- For issue 1-4-1, postpone the discussion.

- For issue 1-5-1, the principle of P1 is reasonable. The issue can be discussed later.

3	Definitions, symbols and abbreviations

- 3.1 Definitions

- 3.2 Symbols

- 3.3 Abbreviations

- 3.5 Frequency bands grouping (clarification on current frequency band grouping is applicable for LP-WUR may needed)

- 3.6 Applicability of requirements in this specification version

4  SA: RRC_IDLE state mobility

- 4.X LP-WUR measurements

  - 4.X.1	Introduction

  - 4.X.2	Requirements

    - 4.X.2.1	[LP-WUR and MR measurement capability]

    - 4.X.2.2	[LP-WUR serving cell measurement and evaluation requirements]

    - 4.X.2.3	[MR serving cell measurement and evaluation requirements]

    - 4.X.2.4	[Measurements of intra-frequency NR cells for UE with LP-WUR]

    - 4.X.2.5	[Measurements of inter-frequency NR cells for UE with LP-WUR]

    - 4.X.2.6	[Measurements of inter-RAT E-UTRAN cells for UE with LP-WUR]

    - [4.X.2.7	Maximum MR activation delay with LP-WUS operation]

5  SA: RRC_INACTIVE state mobility

- 5.X LP-WUR measurements

  - 5.X.1	Introduction

  - 5.X.2	Requirements

    - 5.X.2.1	[LP-WUR and MR measurement capability]

    - 5.X.2.2	[LP-WUR serving cell measurement and evaluation requirements]

    - 5.X.2.3	[MR serving cell measurement and evaluation requirements]

    - 5.X.2.4	[Measurements of intra-frequency NR cells for UE with LP-WUR]

    - 5.X.2.5	[Measurements of inter-frequency NR cells for UE with LP-WUR]

    - 5.X.2.6	[Measurements of inter-RAT E-UTRAN cells for UE with LP-WUR]

    - [5.X.2.7	Maximum MR activation delay with LP-WUS operation]

Annex B (normative): Conditions for RRM requirements applicability for operating bands

- [B.1.x	Conditions for measurements on NR serving cells with LP-WUS operation]

New sections, updates and clarifications for LP-WUR WI

In detail, the following content will be added in each new sections.

| Sections | New Content to be added |
| --- | --- |
| - 3. Definitions, symbols and abbreviations | - All related terms - 3.5 Frequency bands grouping (clarification on current frequency band grouping is applicable for LP-WUR may needed) |
| - 4.X.1 Introduction | Introduction of the feature |
| - 4.X.2.1 [LP-WUR and MR measurement capability] | - LP-WUR capability on monitoring serving cell; - Duplicate MR capability |
| - 4.X.2.2 [LP-WUR serving cell measurement and evaluation requirements] | - General description - LP-WUR evaluation requirement (OOK and OFDM based) - LP-WUR measurement periodicity, filtering and [measurement requirements] (OOK and OFDM based) - LR-WUR measurement accuracy，represented by margins (OOK and OFDM based) |
| - 4.X.2.3 [MR serving cell measurement and evaluation requirements] | - General description - MR measurement and evaluation requirements for cell selection - With MR measurement relaxation - Without MR measurement relaxation (same as legacy) - MR measurement and evaluation requirements for [LP-WUS and MR offloading] - With MR measurement relaxation - Without MR measurement relaxation - MR measurement and evaluation requirements for [MR RRM relaxation] - MR Serving cell offloading - MR Serving cell RRM measurement relaxation requirements |
| - 4.X.2.4 [Measurements of intra-frequency NR cells for UE with LP-WUR] | - MR Intra-frequency neighbour cell RRM measurement relaxation requirements |
| - 4.X.2.5 [Measurements of inter-frequency NR cells for UE with LP-WUR] | - MR Inter-frequency neighbour cell RRM measurement relaxation requirements - Higher priority frequency layers related requirements |
| - 4.X.2.6 [Measurements of inter-RAT E-UTRAN cells for UE with LP-WUR] | - MR Inter-RAT E-UTRAN cells RRM measurement relaxation requirements - Higher priority frequency layers related requirements |
| [4.X.2.7 Maximum MR activation delay with LP-WUR operation] | - TBD |
| [B.1.x Conditions for measurements on NR serving cells with LP-WUS operation] | - OFDM based LP-WUR SSB Ês/Iot and LP-SS Ês/Iot - Minimum SSB_RP or LP-SS_RP for each NR operating band groups |

- References

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 102

[2] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR ((LP-WUS/WUR), RAN 103

[3] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 105, vivo, NTT DOCOMO

[4] R4-2502600, WF on RRM requirements for NR_LPWUS, RAN4 114, vivo

[5] R4-2500539,	Topic summary for [114][225] NR_LPWUS, RAN4 114, vivo

[6] R4-2009265, WF on RRM measurement relaxation for Power Saving, RAN4 96e, CATT

[7] R4-2418892, On RRM for LP-WUR, RAN4 113, vivo

[8] R4-2501418, Further considerations on RRM for LP-WUR, RAN4 114, vivo

3GPP TSG-RAN WG4 Meeting #114bis	R4-24xxxxx

WuHan, China, 07 April – 11 April, 2025

Title:		Reply LS on LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode

Response to:		RAN1

Release:		Release 19

Work Item:       NR_LPWUS

Source:          RAN4

To:	RAN1

Cc:

Contact Person:   Xusheng Wei, xusheng.wei@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	No

- 1	Overall description

RAN4 would like to thank RAN1 for the LS on LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode.

In the LS RAN1 asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities

Agreement

For UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

  - FFS: different sets of 3 candidate value for different SSB periodicities

From RAN4 perspective, RAN4 confirms that the number of SSBs assumed by RAN1, i.e., 3 SSBs for 70ms case and 5 SSBs for 500ms and 900ms cases, are right.

RAN4 further confirms that it is reasonable to assume that the same number of SSBs for MR synchronization for different SSB periodicities.

RAN4 also agrees that from RRM requirements point of view, for the scenario when the SSB periodicity is 5ms or 10ms, 20ms should be used as a virtual SSB periodicity.

- 2	Actions

To RAN1 group.

ACTION:	RAN4 kindly asks RAN1 to take the above information into account in their further work.

3. Date of Next RAN4 Meeting:

TSG-RAN WG4#115	                May. 19-23, 2025			Malta

TSG-RAN WG4#116	                Aug. 25-29, 2025			Bangalore, India