3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503306

Wuhan, China, April 7th – 11th, 2025

Source: 	Xiaomi

Title: 	Discussion on general aspects of AI/ML testability and interoperability

Agenda Item:	7.19.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss about general aspects of AI testability and interoperability.

- Discussion

Issue 1-4-4: Relation to legacy RAN4#112bis WF: Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management - Proposals - Option 1: Keep same side conditions for legacy and AI/ML (Apple proposal 18, CATT proposal 1 option 1, CMCC proposal 1) - Option 2: Measurement error may have an impact on whether AI can exceed “legacy” requirements (Xiaomi proposal 1) - Option 3: Allow larger margin (presumably more relaxed requirement) for higher overhead reduction (CATT proposal 1 option 2) - For CSI compression, better than legacy with the same overhead, and no worse than legacy with reduced overhead (Huawei proposal 1) - For positioning, re-use legacy requirement for case 2a/3a if AI and non-AI cannot be distinguished (Huawei proposal 3) - for CSI prediction, it is proposed that AI/ML based performance requirement should not be worse than the legacy PMI requirements for 'typeII-doppler-r18 (CMCC proposal 2) - Option 4: Legacy performance is the baseline; allow margin for scenarios in which legacy is not defined compared to the baseline (ZTE proposals 3, 4). - Option 5: When a test is defined for AI for which there is no legacy test, need to decide whether to (i) Investigate non-AI performance in that scenario and set AI requirement greater or (ii) set an absolute requirement on AI (without considering non-AI) (Huawei proposal 5) - Recommended WF Discuss the following: - Same side conditions as legacy - What does this mean in practice ? When no existing requirement exists, or some other types of condition should be tested - Does measurement error impact comparison to non-AI - If no non-AI requirement exists for some functionality/condition, is it necessary to consider non-AI performance for comparison to AI, or sufficient just to set an absolute requirement for AI ? - When there is a non-AI requirement, if the AI functionality operates with lower overhead, should the same requirement be met or a more relaxed requirement ?

For AI based beam management, currently, there are several candidate test metrics:

Issue 2-1: Metrics/KPIs for beam prediction If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x

From our understanding, whether the AI requirements exceed “legacy” requirements depends on whether the same metric and side condition is defined for AI and non-AI legacy requirement.

For example, if test metric is beam ID prediction accuracy, in legacy requirement, there is no such metric. We prefer just to set a specific requirement for AI.

Proposal 1: For beam ID prediction accuracy, there is no performance requirement in legacy. Set a specific requirement for AI.

If test metric is absolute RSRP prediction accuracy for the same beam. In legacy non-AI, there is also similar metric. For this metric, predicted RSRP or legacy measured RSRP will be compared with ground truth. RSRP delta is the accuracy requirement.

However, currently, the impact of measurement error is studied. The AI input is measured L1-RSRP with error, it will degrade the RSRP prediction performance. It depends on how the AI model is trained. there are different implementations:

- AI input is measured RSRP without error, label is ground RSRP without error

- AI input is measured RSRP with error, label is measured RSRP with error

- AI input is measured RSRP with error, label is measured RSRP without error

Whether the accuracy of prediction RSRP can still outperform the accuracy of legacy measured RSRP is an issue which needs to be studied.

Proposal 2: For RSRP prediction accuracy, measurement error may have impact on whether AI requirements can exceed “legacy” requirements.

Issue 1-2: Post deployment testing Way forward: The following table presents the description of options 1 and 2 and sub-options as already agreed. A new name for “option 1” and “option 2” is proposed to reduce ambiguity when discussing in the future. The descriptions of the options are the same as the already agreed description. The “further potential clarifications” box captures issues that have been raised that could further clarify the options. It does not represent any agreement on the issues, or that all are relevant, but is intended to stimulate input and discussion to future meetings. The “significant issues” line is intended to capture issues that have been raised that should be answered in order to make a decision. Option 1; Option 2. New name; Pre-activation functionality/model update testing; Post deployment management based on LCM. Description; Conduct the validation of a change in AI functionality before its deployment/activation in already deployed UEs • Validation takes into account the UE hardware in which the model is to be deployed/activated.; Using performance monitoring and LCM procedures • Performance monitoring will be designed in other groups • RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.. Sub-options; Possible collection of input data to a model during conformance testing for use later on.. Further potential clarifications (not resolved); The following proposals have been made for option 1 but not discussed. o Whether the testing of a new model is on a device or in a lab. o Whether models that have been updated post conformancetesting should be kept in a device. o Whether parallel operating of one model and (in-device) testing of another can be assumed. o Whether signaling from a UE of changes/updates are needed. - Whether there is any relation to GCF procedures. - Whether it is possible to differentiate “major” and “minor” model changes and only apply option 1 to major changes. - Definition of Fine-tuning and Model update - Model update can have a performance impact. - Fine-tuning might have little performance impact. - Hybrid Validation approach - A hybrid approach integrating both pre-activation functionality/model update testing (Option 1) and post-deployment management based on LCM (Option 2) aims to provide a balanced validation strategy.; The following proposals have been made for option 1 but not discussed. - Whether the metric used for performance testing and the metric used for monitoring can be aligned. - Whether monitoring used for post-deployment testing should be NW sided only.. Significant issues; Whether the option is reasonable or would cause too much complexity and test time burden when introducing updates.; Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.) Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs..

Option 1 and option 2 are designed for different purpose.

Option 1 is related to model performance validation under dedicated condition. It guarantees that model can work well under specific scenarios.

Option 2 is related to model monitoring between different conditions. It guarantees that model can be switched correctly when conditions or scenarios changes.

Generally, Model performance validation is the pre-requisite for model monitoring. If there are several models applied for different scenarios respectively, model performance under each scenarios needs to be guaranteed firstly. Then during model monitoring, when the performance degrades, we can know the degradation is caused by scenario mismatch.

From our understanding, option 2 is anyway needed since model management can’t be skipped. The question is that whether option 1 can be skipped.

Take beam management as example:

The possible metric for beam index prediction accuracy which will be discussed in high priority in RAN1 is as below:

- High priority discussion points for next meeting ### # Issue #1: performance monitoring for Option 2 ##### (report quantity and metric) - Introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring, where BAI is Np (0 ≤ Np≤ N), and the corresponding beam prediction accuracy is NpN, (0≤ NpN≤ 1). - the size of CSI field associated with the BAI is log2(N+1) - Where N is implicitly or explicitly configured by NW, as the number of the reported inference result(s) linked with performance monitoring instance(s) - Where Np as the number of the reported inference result(s) linked with performance monitoring instance(s), for which, - at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) - Where K is the number of predicted beam(s) in the corresponding inference report - Where Top M beam(s) is the beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, and M is up to 4

RAN1 would like to introduce a new quantity for performance monitoring for beam ID prediction accuracy. For each instance, the prediction will be correct if at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s), which is Top K/M.

It is quite similar as conformance test metric discussed in RAN4 for beam ID prediction accuracy. Currently, Top K/N is also a candidate option.

It’s possible that similar KPI, i.e. Top K/M, is defined for conformance test and performance monitoring for beam ID prediction accuracy.

Observation 1: It’s possible that similar KPI, i.e. Top K/M, is defined for conformance test and performance monitoring for beam ID prediction accuracy.

If the metric for performance monitoring is similar as conformance test metric, it’s possible that option 1 may be skipped. However, there are still some differences which needs to be considered when considering whether option 1 can be skipped.

For example, for conformance test, the ground truth best beam index will be measured under high SNR. While for performance monitoring after deployment, the SNR will dynamically change and in some cases it will be low. It will have impact on derive the ground truth best beam index. In other words, the ground truth derived in real field may not be accurately enough. Then the value of K and M in Top K/M may be different for conformance test in test lab and in real field after deployment. Therefore, the test condition in conformance test may also needs to be extended to consider the impact of dynamic scenario in real field.

Therefore, we suggest to design the similar metric for performance monitoring and conformance test and further discuss the conditions to skip option 1, e.g. how to expand lab tests to to emulate real-world dynamics. This ensures field reliability while eliminating redundant testing for non-critical updates.

Proposal 3: Suggest to design the similar metric for performance monitoring and conformance test. Further discuss the condition to skip option 1.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: For beam ID prediction accuracy, there is no performance requirement in legacy. Set a specific requirement for AI.

Proposal 2: For RSRP prediction accuracy, measurement error may have impact on whether AI requirements can exceed “legacy” requirements.

Observation 1: It’s possible that similar KPI, i.e. Top K/M, is defined for conformance test and performance monitoring for beam ID prediction accuracy.

Proposal 3: Suggest to design the similar metric for performance monitoring and conformance test. Further discuss the condition to skip option 1.

- Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interfa