3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503307

Wuhan, China, April 7th – 11th, 2025

Source: 	Xiaomi

Title: 	Discussion on core requirement and testing framework for AI beam management

Agenda Item:	7.19.3

Document for:	Discussion

- 1 Introduction

In this contribution, we will discuss core part requirement and testing framework for AI based beam management.

- Discussion

      - 2.1 Performance metric for BM-case 1

According to agreement in last meeting, definition of RSRP prediction accuracy is agreed. Currently, there are 3 options for AI based beam management accuracy KPI:

Issue 2-1: Metrics/KPIs for beam prediction If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x

2.1.1 Beam ID prediction accuracy

For Top-K/1 metric, it requires that UE can successfully found the ground truth of Top-1 beam index by measurement and sorting. However, we found that it’s difficult for UE to identify the ground truth of Top-1 beam by measurement even at high SNR conditions. That’s because in some cases, the L1-RSRP of Top-1 beam and Top-2 beam are quite close. It requires very tight measurement accuracy.

Suppose that there is total 32 TX beams in set A. Next, we will first provide the CDF curve of ideal L1-RSRP difference between best 1st and nth beams for 32 TX beams in Fig.1. L1-RSRP delta between 1th and 2th best beams is L1-RSRP difference between the largest and the second largest beams. RSRP delta between 1th and 3th best beams is RSRP difference between the largest and the third largest beams, and so on. It is noted that the CDF in all figures is calculated based on ideal L1-RSRP without any error. The ideal L1-RSRPs are derived from SLS channel model defined in 38.901, which is used to train AI model. The TX beam range is equally divided from -60 degree to 60 degree in azimuth. For each TX beam, there is 8 RX beams. The largest L1-RSRP by RX beam sweeping for each TX beam is selected.

Fig.1 Ideal L1-RSRP difference for 32 TX beams

In Fig.1, we take the CDF curve “RSRP delta between 1st and 2nd best beam” as an example. 10 percentile of CDF curve shows the L1-RSRP delta between 1st and 2nd best beam is smaller than 0.16dB for 10% cases. If UE can successfully distinguish between the 1st and 2nd beams in 90% case, the measurement accuracy has to be within 0.16dB. 20 percentile of CDF is 0.3dB, it means that UE can successfully distinguish between the 1st and 2nd beams in 80% case if measurement accuracy can be within 0.3dB.

As another example as shown in the CDF curve “RSRP delta between 1st and 4th best beam”, 10 percentile results in 1dB L1-RSRP difference. That means the measurement accuracy has to be within 1dB in order to successfully distinguish between the 1st and 4th beams in 90% case.

Besides, the beam resolution is dependent on total Tx beam numbers. The more Tx beams, the more difficult to identify the best beam.

When UE performs measurement to derive ground truth of beams, UE will have baseband(BB) measurement error and RF error. With high SNR, UE can reduce measurement error of BB, however, there will still be some measurement error due to RF. Since UE can’t distinguish the 1st best beam due to limited measurement accuracy, the Top-1 best needs to be extended to Top-N best beams.

Observation 1: For derive ground truth for best Top-1 beam, UE needs to measure for all beams in set A under high SNR. However, there is still some measurement uncertainty due to RF and BB. It’s challenging for UE to detect the best 1st beam even at high SNR due to the small gap between best 1st and 2nd beam.

Observation 2: For beam prediction accuracy, option 2 is not correct since UE can’t distinguish which is the best beam index due to measurement error.

Besides, RAN1 is also discussing similar metric of relax top K-1 to top K-M in performance monitoring, which will be in high priority discussion in next meeting [1].

- High priority discussion points for next meeting ### # Issue #1: performance monitoring for Option 2 ##### (report quantity and metric) - Introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring, where BAI is Np (0 ≤ Np≤ N), and the corresponding beam prediction accuracy is NpN, (0≤ NpN≤ 1). - the size of CSI field associated with the BAI is log2(N+1) - Where N is implicitly or explicitly configured by NW, as the number of the reported inference result(s) linked with performance monitoring instance(s) - Where Np as the number of the reported inference result(s) linked with performance monitoring instance(s), for which, - at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) - Where K is the number of predicted beam(s) in the corresponding inference report - Where Top M beam(s) is the beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, and M is up to 4

Observation 3: From RAN1 performance monitoring discussion, the similar metric Top-K/M is under discussing, where at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s).

We prefer to align the metric with performance metric in performance monitoring.

Therefore, for beam ID prediction accuracy, we suggest to update option 2 to:

- Top-K/N (%): the percentage of "any one of the Top-N genie-aided beam is one of the Top-K predicted beams"

Here, Top-1 genie-aided beam is extended to Top-N genie-aided beams. UE will report predicted best K beams and ideal best N beams to TE.

Another option is to use L1-RSPR difference, similar as option 3. Here, we refine option 3 as: the maximum measured L1-RSRP of top-K predicted beams is larger than the maximum measured L1-RSRP – X dB.

Proposal 1: For beam prediction accuracy performance metric,

- If beam index is used as reference, the test metric is:

  - Top-K/N (%): the percentage of "at least one of the Top-N genie-aided beam is one of the Top-K predicted beams".

2.1.2 RSRP absolute accuracy

If UE also reports predicated RSRP, similar as legacy, predicated RSRP accuracy based on the same beam can be defined.

Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

In previous agreement, it’s agreed that the same beam index will be used to evaluate RSRP delta. The index i may be any beam index [in top-K beams based on ground truth L1-RSRP]. It’s still FFS how to decide the index i.

Besides, how to apply absolute RSRP accuracy is still FFS.

Issue 2-1: Metrics/KPIs for beam prediction If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS

From RAN1 agreements in RAN1#120:

Agreement For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size. Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value.

From RAN1 agreement, reporting Mechanism:

For K > 1 beams:

- 1st-ranked beam: Reported as absolute quantized RSRP

- 2nd-Kth beams: Reported as differential RSRP (4-bit resolution, 2dB step size) relative to the 1st-ranked beam

Prediction Phase Reporting (Step 1)

UE reports predicted RSRP for Top-K beams with format: {A₁, ΔB₁, ΔC₁}

- Beam A (predicted 1st): Absolute RSRP

- Beams B/C: Differential RSRP (ΔB₁/ΔC₁)

The reconstructed absolute RSRP for 2nd-Kth beams contains quantization uncertainty:

Rkpred=RApred-ΔRkpred±Δquantdiff

where Δquantdiff=12×differential step size (1dB for 2dB step), Considering the quantization method with rounding off.

Ground Truth Reporting (Step 2)

UE reports measured ground truth RSRP for Top-K beams, with two possible scenarios:

Case 1: Unchanged beam ranking {A₂, ΔB₂, ΔC₂}

Case 2: Changed ranking {B₂, ΔA₂, ΔC₂'}

The reconstructed ground truth RSRP for 2nd-Kth beam follows:

Rktrue=R1sttrue-ΔRktrue±Δquantdiff

Here, we omit the absolute quantization step error. Since in legacy, we didn’t consider this. Here, we focus about the additional uncertainty when re-constructing absolute value.

The differential quantization introduces cumulative uncertainty in absolute RSRP reconstruction:

| Beam Type | Uncertainty Source | Total Uncertainty |
| --- | --- | --- |
| Twice-differential (e.g., beam C) | 2 × differential quantization errors | 2Δquantdiff |
| Single-differential (e.g., beam A in Case 2) | 1 × differential quantization error | Δquantdiff |
| No differential (e.g., beam A in Case 1) | 0 | 0 |

Observations 4: Differential reporting induces error propagation in absolute RSRP reconstruction:

- 1dB uncertainty per differential report (for 2dB step size)

- Cumulative effect when beam ranking changes between reports

Observation 5: Direct comparison of predicted vs. measured RSRP for 2nd-Kth beams becomes unreliable due to:

- Potential beam ranking changes

- Compounded quantization errors

Proposal 2: For BM-case 1, only define absolute RSRP accuracy for 1st-ranked predicted beam in report:

predicted L1-RSRP of beam index i – ground truth of L1-RSRP of beam index i.

where the index i is the 1st-ranked beam index in report.

The network shall configure K=1 for ground truth reporting specifically targeting the 1st-ranked predicted beam. This configuration ensures:

1) Maintain beam index consistency by retaining the 1st-ranked predicted beam in the measurement phase,

2) Eliminate quantization error propagation through avoidance of differential RSRP reporting, and

3) Enable direct absolute RSRP accuracy verification without requiring reconstructed values.

This configuration ensures deterministic error analysis by enforcing identical reporting mechanisms (absolute quantization only) between prediction and measurement phases.

Proposal 3: The network shall configure K=1 for the RSRP measurement report configuration, applied exclusively to the 1st-ranked predicted beam.

      - 2.2 RX beam sweeping

In legacy for FR2, RX beam sweeping is considered for SSB based measurement and CSI-RS with repletion ON.

Measurement period will be scaled by RX beam sweeping factor, e.g. N=8 for SSB.

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

For AI beam management, two kinds of RX beam are assumed in TR 38.843:

Beam prediction accuracy related KPIs, including: - Top-1 genie-aided Tx beam considers the following definitions: - Option A (baseline), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B (optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - Specific Rx beam(s) are to be reported. Note: specific Rx beams are a subset of all Rx beams.

For option B, the best beam can be predicted by specific RX beam(s). It means that UE can use subset RX beam to predict best TX beam and UE don’t need to sweep full RX beams. Therefore, it may have impact on RX beam sweeping factor in measurement period. Since RX beam sweeping will apply for spatial domain and time domain beam prediction, the impact to BM case-1 and BM case-2 is common.

Proposal 4: For both BM case-1 and BM case-2, RAN4 to discuss whether there is impact on measurement period due to specific RX beam assumption.

      - 2.3 Measurement time && prediction time

Issue 2-3: Measurement period for inference - Proposals - Option 1: Use “legacy” L1-RSRP measurement as baseline - FFS whether same values for M,N,P should be reused - Option 2: Differentiate case 1 and case 2 - Reuse L1 measurement period for Case 1 - Introduce configurable number of measurement occasions for case 2 - Option 3: Study the measurement period based on simulations - Option 4: Others

BM-case 1 and BM-case 2 are different.

For BM-case 1, legacy measurement period can be used as baseline. However, as outlined in section 2.2, measurement period will be scaled by RX beam sweeping. If UE apply specific RX beams, N may be modified.

Proposal 5: for BM-case 1, legacy measurement period can be used as baseline. FFS for the impact of specific RX beam(s).

For BM-case 2, RAN1 agrees that NW can configure UE to predict future N L1-RSRP instance.

RAN1 #118 Agreement For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - FFS: how to determinate reference time for the time instance(s) - FFS: duration values of the N time instance(s) that can be predicted. Agreement(RAN 1#120) For inference, for BM-Case 2 of UE-side model, - The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC. - time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - N = [1, 2, 4, 8] - Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report. Agreement (RAN 1 #120) For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, - the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. The time instance information of the beam with the largest RSRP are additionally indicated in the report.

Besides, TR defined several types for time-domain prediction:

Case A: based on number of measurements/RSs and prediction time. An example is shown in Figure 6.3.1-2. - where T2 is the time duration for beam prediction - where Mt is the number of time instances for measurement as AI/ML inputs with a periodicity of Tper - where Pt is the number of time instance(s) for prediction with a periodicity of Tper in T2 Case B: based on a periodicity T of the required reference signals for measurements to achieve a certain beam prediction accuracy. An example is shown in Figure 6.3.1-3. - For AI, every T=Y ms, reference signals for measurements are needed - In this case, - For Set B = Set A, the RS overhead reduction is 1-X/Y. - For Set B (N beams) is a subset of Set A (M beams), the RS overhead reduction is [1-XN/(YM)].

The figures for case A and case B from TR are shown below:

Figure 6.3.1-2: Example for Case A

Figure 6.3.1-3: Example for Case B

The main difference of the two cases is the length of measurement periodicity and prediction periodicity.

  - Case A: the periodicity for history measurement instance is same as that of predicted future time instance.

  - Case B: the periodicity for history measurement instance is larger than that of predicted future time instance.

From our observation, at least case A will have impact on UE measurement requirement. In legacy, measurements on all instance are needed. Measurement period is defined for each instance periodically. While when AI is applied, for case A, some measurement instance can be skipped and UE can acquire predicted results for these time slots. The legacy measurement period requirement can’t reflect such benefits introduced by AI. Suppose that there are totally 10 instances. In legacy, the total measurement time is 10*TRS. While when AI is applied, suppose that AI can measure for 5 instance and predict for another 5 instances, the total measurement time will be reduced.

Since RAN1 introduce the concept of measurement time and prediction time, RAN4 needs to discuss how to define new requirement for measurement time and prediction time for BM-case 2 either. If measurement time and prediction time is defined, RAN4 can further discuss whether measurement restriction and scheduling restriction can be relaxed in prediction time.

Proposal 6: For BM-case 2, RAN4 to discuss impact on measurement requirement, e.g. measurement time, prediction time, measurement restriction and scheduling restriction, etc.

      - 2.4 TCI state activation

UE will measure for set B first and then predict and report best TX beam index in set A, then NW can configure TCI state activation. There are two possibilities for the reported best predicted TX beam index:

- Case 1: best predicted TX beam is in set B.

- Case 2: best predicted TX beam is in set A but not in set B.

For case 1, UE knows the corresponding best RX beam since UE performs RX beam sweeping for all TX beams in set B. For case 2, UE may know or not know the best RX beam. It will depend on UE implementation. If the input of AI model and output of AI model are based on specific RX beam, UE can know the corresponding RX beam. If the input of AI model and output of AI model are based on RX beam sweeping, UE can first predict the L1-RSRP based on different TX-RX pairs, then UE sort L1-RSRP and report the best beam index. Then it’s possible for UE to know the corresponding best RX beam. If UE directly predict best TX beam index, it’s possible that UE didn’t know the RX beam index.

Observation 6: When best predicted TX beam is in set B, UE knows the best RX beam. When best predicted TX beam is in set A but not in set B, it’s still possible for UE to know the RX beam corresponding to the best TX beam.

Whether UE knows the RX beam or not will have impact on TCI state activation delay. Current known condition only considers whether UE has sent the report within a period and it didn’t consider whether UE has known the RX beam or not. We prefer to define UE capability to indicate whether UE can know the RX beam or not. Then the L1 reporting part in known condition for TCI state will be updated to:

The UE has sent at least 1 L1-RSRP report for the target downlink TCI state before the downlink TCI state switch command. Where, - the target TCI state in L1-RSRP report has been measured, or - the target TCI state in L1-RSRP report is predicted and UE support [RX beam known] capability

Proposal 7: RAN4 may know RX beam based on UE capability.

      - 2.5 Measurement error

Fit RSRP delta CDF of LLS by Gaussian distribution

Step 1: calculate RSRP delta between ideal RSRP and measured RSRP by LLS and plot CDF curve.

Step 2: determine RSRP delta corresponding to 5% and 95%, which corresponding to the lower and upper limits of the 90% confidence interval. Suppose that the lower and upper limits for 5% and 95% points of CDF curve from LLS are a and b.

Step 3: Calculate the standard deviation, σ, for Gaussian distribution. Where Z0.95 is the 95th percentile of the standard normal distribution, which is approximately 1.645.

σ=b-a2∙Z0.95

First, we derive the measured RSRP accuracy CDF for AWGN channel under SNR = -3dB obtained from LLS, where a = 1.4 and b = -1.4. then we also derive the measured RSRP accuracy CDF for TDL-C channel under SNR = -3dB obtained from LLS where a = -2.2 and b = 1.6. By step 3, σ=0.6245 for AWGN channel and σ= 1.3341 for TDL-C.

Here, we plot the RSRP measurement delta CDF curve from LLS and the fitted curve by Gaussian distribution for AWGN and TDL-C in the following figure. It shows that Gaussian distribution can fit the measurement error models from LLS well.

- AWGN                                (b) TDL-C

Fig.2 RSRP measurement delta CDF curve from LLS and fitted by Gaussian

Prediction accuracy degradation due to measurement error

- Case 1: Same measurement error is applied for all TX beams

Here, we apply the same measurement error to all TX beams. Where measurement error is modeled as Gaussian distribution.

| Measurement error (dB) | Prediction accuracy % |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| 0 | 76.35 | 93.85 | 98.17 |
| 2 | 53.89 | 74.92 | 92.46 |
| 4 | 48.17 | 66.39 | 86.75 |
| 6 | 42.94 | 63.02 | 82.34 |

- Case 2: Different measurement error is applied for different TX beams

Here, we also apply the different measurement error to different TX beams with different SNR conditions. Where measurement error is modeled as Gaussian distribution.

We generate L1-RSRP for multiple UEs and corresponding SINR for all TX beams from SLS. For AI model, the input for each UE will be 8 TX beams from set B, i.e. 8 SINRs. According to maximum SINR of each UE, we split UE into 5 groups.

- UE group 1: 10dB≤maximum SINR<20dB

- UE group 2: 5dB≤maximum SINR<10dB

- UE group 3: 0dB≤maximum SINR<5dB

- UE group 4: -5dB≤maximum SINR<0dB

- UE group 5: -10dB≤maximum SINR<-5dB

SINR distribution for 5 UE groups are plotted in Fig.3:

Fig.3 SINR distribution of 8 TX beams in set B for different UE groups

It means that even if the best SINR for UE is high, SINRs of other TX beams for the same UE can still be low. SINR will span a large range from high SINR to low SINR. Relatively large measurement error will be added for certain worse TX beams.

Observation 7: SINR distribution for all UE group will span a large SINR range for all UE groups.

The predication accuracy for different group of UE is as below:

| Maximum SINR of UE |  | Prediction accuracy % |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | K=1 | K=2/1 | K=4/1 | K=4/2 |
| UE group 1 | 10dB≤maximum SINR<20dB | 44.3 | 77.8 | 92.9 | 98.2 |
| UE group 2 | 5dB≤maximum SINR<10dB | 41.5 | 72.3 | 90.7 | 97.6 |
| UE group 3 | 0dB≤maximum SINR<5dB | 36.3 | 65.1 | 89.8 | 95.3 |
| UE group 4 | -5dB≤maximum SINR<0dB | 30.2 | 54.5 | 72.4 | 87.5 |
| UE group 5 | -10dB≤maximum SINR<-5dB | 6.8 | 14.95 | 30.9 | 42.1 |

If different TX beam will add different measurement error, it will indeed change the distribution of L1-RSRP power between different beams, which will degrade the prediction performance.

Due to the fact that large measurement error will be added to TX beams of low SINR, For K=1, the prediction accuracy is low. The prediction accuracy will improve when K increases. With K=4/1, when maximum SINR is larger than 0dB(UE group 1,2,3), prediction accuracy can be nearly around or higher than 90%. When -5dB≤maximum SINR<0dB(UE group 4), prediction accuracy for K/1 will degrade to 72%. If relax to K=4/2, the performance will increase to 87%. When -10dB≤maximum SINR<-5dB, prediction accuracy for all KPI is bad.

Observation 8: For K=1, measurement accuracy error will degrade performance a lot for all UE groups.

Observation 9: With K=4/1, when maximum SINR is larger than 0dB(UE group 1,2,3), prediction accuracy can be nearly around or higher than 90%.

Observation 10: When -5dB≤maximum SINR<0dB(UE group 4), prediction accuracy for K/1 will degrade to 72%. If relax to K=4/2, the performance will increase to 87%.

Observation 11: When -10dB≤maximum SINR<-5dB, prediction accuracy for all KPI is bad.

Therefore, we suggest to define SNR side condition for the best TX beam in set B. When the SNR for the best TX beam in Set B is low, AI prediction accuracy is bad and there is no requirement for beam prediction accuracy. Besides, we also suggest to relax beam index prediction accuracy KPI to Top K/N when taking measurement error impact into consideration.

Proposal 8: Impact of measurement error has impact on prediction accuracy requirement, RAN4 to define SNR side condition for the best TX beam in Set B.

Proposal 9: Considering measurement error impact, RAN4 to relax beam index prediction KPI to Top K/N.

      - 2.6 Test Motivation

Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

There are lots of discussions about channel model, e.g. single AOA or multiple AoAs.

Some company mentioned that without multiple AoA, UE RX beam sweeping functionality can’t be verified. On the other hand, some company mentioned that the main test motivation is to verify the AI algorithm and single AOA channel is enough to generate the emulated received signal.

If AI model can achieve final prediction accuracy, it involves two steps:

- Input data of AI model achieve accuracy requirement

- AI model algorithm achieve prediction requirement

From our understanding, it’s better to split the two parts in testing, otherwise, it’s hard to distinguish UE fails in which step. The total test includes two parts:

- Part 1: Verify input data accuracy of model AI based on RX beam sweeping

- Part 2: Verify AI model algorithm

For Part 1, it’s related to measured RSRP accuracy based on RX beam sweeping. This is not a new issue for AI. While for legacy beam management, it also has the similar issue. However, in legacy, the RX and TX beamforming gain has been transformed to equivalent SNR. RSRP accuracy is tested under certain SNR level. No RSRP accuracy requirement for RX beam sweeping is defined. If company really thinks it’s important, the RX beam sweeping performance can be tested separately. For this part, multi-AOA model can be used.

For Part 2, the emulated data can be used as input for AI model, then AI model prediction accuracy can be tested by single AOA model.

Proposal 10: For OTA test, prefer to split the test into two parts.

- Part 1: Verify AI model prediction algorithm, using single AOA model.

- Part 2: Verify input data accuracy of model AI based on RX beam sweeping, using multi-AOA model.

      - 2.7 Channel model

### 2.7.1 Single AOA

For test part 1, single AOA can be used.

There are several possible options to generate single AOA:

  - Option 1: Emulated signal considers both TX and RX beamforming gain.

Since the emulated signal is after UE RX beamforming, single AOA is assumed, then the signal shall consider both TX and RX beamforming gain:

i=1NPT,iCiPR,i

Where N is number of multi-path signals. PT,i is Tx beamforming gain for ith-path signal. Ci is power of ith-path signal. PR,i is Rx beamforming gain for ith-path signal. It means that TX side needs to know the UE RX beam forming codebook in advance.

For this method, no simplified channel is needed as TX side can generate all kinds of channels. This method requires that RX beam pattern is known by TE in advance. However, since there are many UE vendors, then the test will be UE specific. we prefer to design a common pre-defined RX beam pattern for both TE and UE.

Observation 12: If Emulated signal considers both TX and RX beamforming gain, RX beam pattern is UE specific. In order to reduce test complex, prefer to design a common pre-defined RX beam pattern for both TE and UE.

  - Option 2: The emulated signal is before UE RX beamforming.

    - Option 2-1: Not reduce path number, but combine all TX signal by a fixed RX beamforming.

For option 2-1, the emulated signal is before UE RX beamforming, then the signal only considers TX beamforming gain, it will combine all TX path together by multiply TX beamforming gain. Before RX beamforming, multiple TX signal has already be combined into one path. UE needs to receive the combined TX signal by RX beam. the final input data for AI model will be:

PRi=1NPT,iCi

It means that the same RX beamforming gain, PR, is applied for different TX path signals. however, in reality, there will be different RX beamforming gain for different TX path signals. There will be some mismatch between emulated signal for test and original signals for training. The mismatch impact needs to be evaluated.

Observation 13: if a fixed RX beamforming gain is used for multi-path TX signals, there will be some mismatch between emulated signal for test and original signals for training. The mismatch impact needs to be evaluated.

    - Option 2-2: Simplify multi-path into single-path.

The following figure compare the L1-RSRP difference between different TX beam index with single path, the detail configuration and analysis can be found in[1]. By our simulation, the less the path number, the larger gap between different beams, which means that AI prediction performance will be relaxed. It’s more easiler for AI to distingwish the best Top-K TX beam. Therefore, we prefer not to reduce probe number to verify AI prediction performance.

Simplified CDL-C with 3 cluster, 1 AOA

Fig.4 CDF curve L1-RSRP difference between CDL-C and simplified CDL-C

Observation 14: For simplified channel model, L1-RSRP delta between different TX beams will increase if cluser number and AOA number is further reduced. It’s more easiler for AI to distingwish the best Top-K TX beam. The AI prediction requirement will be relaxed.

In summary, we prefer option 1 or option 2-1 for single AOA solutions to verify AI prediction accuracy.

For option 1, further discuss whether it’s possible to design common pre-defined RX beam pattern to generate emulated single AOA signals.

For option 2, further discuss the impact of mismatch between emulated signal for test and original signals for training.

Proposal 11: For single AOA, RAN4 to discuss whether it’s possible to design common pre-defined RX beam pattern to generate emulated single AOA signals.

### 2.7.2 Multi-AOA

For test part 2, multi-AOA can be used. For multi-AOA, due to probe limitation in chamber, simplified channel model needs to be studied.

For CDL model, there are several clusters and for each cluster, there are 20 rays. In order to reduce the total number of AOA, for each cluster, only one ray is considered.

Additionally, the number of clusters can be further reduced using the following options:

- Angle Offset Threshold:

  - If the AOA offset between two clusters is smaller than a threshold, these clusters can be merged into one. The AOA offset threshold may depend on the angle resolution of the RX antenna and test setup limitations, which can be further discussed in RAN4.

- Power Threshold:

  - Remove clusters with low power. The power threshold can be further discussed.

- Combination of Angle and Power Thresholds:

  - Apply both angle and power thresholds to reduce the number of clusters.

How to choose cluster for simplified CDL channel needs to be discussed. Here, we select 8 clusters with 3 probes for simplified CDL-C

- 1st probe: cluster {#2,#3,#4}with the same angle

- 2nd probe: cluster{#6,7,8}with the same angle

- 3rd probe: cluster{#1,#5} with small angle offset

| Cluster # | Normalized delay | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4 | -46.6 | -101 | 97.2 | 87.6 |
| 2 | 0.2099 | -1.2 | -22.8 | 120 | 98.6 | 72.1 |
| 3 | 0.2219 | -3.5 | -22.8 | 120 | 98.6 | 72.1 |
| 4 | 0.2329 | -5.2 | -22.8 | 120 | 98.6 | 72.1 |
| 5 | 0.2176 | -2.5 | -40.7 | -127.5 | 100.6 | 70.1 |
| 6 | 0.6366 | 0 | 0.3 | 170.4 | 99.2 | 75.3 |
| 7 | 0.6448 | -2.2 | 0.3 | 170.4 | 99.2 | 75.3 |
| 8 | 0.6560 | -3.9 | 0.3 | 170.4 | 99.2 | 75.3 |

Proposal 12: For simplified multi-AOA CDL model, cluster number can be reduced from spatial angle offset and power aspect:

- Combine clusters if AOA offset between these clusters is smaller than a threshold

- Remove cluster with low power.

      - 2.8 Test procedure

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

it’s agreed that the ground truth of the predicted RSRP is the ideal measurement of RSRP on the predicted TX beam.

Test procedure

Fig.5 test procedure

As shown in Fig.5, the main steps for test beam or L1-RSRP accuracy are as below:

- Step 1: TE configures resource sets for measurement of set B and prediction report for set A

- Step 2: UE measures for beams in set B in T1

- Step 3: UE performs beam prediction for set A

- Step 4: UE report predicted best beam(s) and/or L1-RSRP(s)

- Step 5: TE configures resource set for measurement of set A

- Step 6: UE measures for beams in set A in T2 under high SNR

- Step 7: UE report ground truth of L1-RSRP of set A and/or best beam index

- Step 8: TE compares predicted beam with ground truth based on different metric

We identified that there are several issues that will have impact on the test metric accuracy.

### 2.8.1 Ground truth for best beam index

If the test metric is beam index prediction accuracy, UE will report the best beam index in set A after measurement. However, as we analysed in performance metric part, UE may not distinguish for the best 1st beam index and 2nd best beam index due to small L1-RSRP delta between the best 1st beam index and 2nd best beam index.

Therefore, UE may need to report for N best beam index. RAN4 can further discuss how to decide N.

Proposal 13: If the test metric is beam index prediction accuracy, UE may need to report ground truth of N best beam index for set A.

### 2.8.2 SNR impact

Same as SNR difference analysed for set B, there will be great SNR difference for 32 TX beams in set A either. If SNR for the worst TX beam is set as 10dB, then SNR for best TX beam will be too high. There may be some test issue. Suppose that if SNR of best several TX beams is set as 10dB, SNR for the worst 10 TX beam can be as low as -15dB. There will be great measurement error for the worst beam.

For set A, UE only needs to identify several best beam indexes and/or corresponding L1-RSRPs, UE doesn’t need to measure for all beams accurately. The reason is that since the L1-RSRP delta between best beam and worst beam is relatively large, the measured L1-RSRP for worst beam is still quite small, which will not exceed the value of L1-RSRP of best TX beams.

Therefore, for deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Observation 15: For deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Therefore, RAN4 needs to discuss how to set SNR condition for measurement of set A to derive ground truth.

Proposal 14: For deriving ground truth of set A, SNR level should satisfy that for Top-M beams, the measurement error is small. RAN4 to discuss how to decide M and how to set SNR.

### 2.8.3 Channel time variation impact

Since the beam prediction will be based on the measurement of set B at T1. Ground truth L1-RSRP will be measured at T2. There will be a time gap between T1 and T2. Due to doppler, L1-RSRP will also vary in time domain. L1-RSRP time domain variation will also have impact on ground truth.

For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation. While for BM case-2, UE will be assumed to move. Therefore, channel doppler can’t be set for 0. Doppler will depend on UE speed and UE trajectory model. The RSRP variation can be evaluated by different doppler and time gap. RSRP variation impact to BM case-2 can be considered in RSRP accuracy requirement.

Proposal 15: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 16: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

### 2.8.4 How to solve UE cheating issue

In legacy L1-RSRP test, ground truth is set by TE. Therefore, UE can honestly report the measured L1-RSRP results.

In current AI based BM test, UE will report both best predicted beam and ground truth value to TE. TE can only rely on the two reports from UE to decide whether UE can pass the test. It’s easy for UE to cheat if the configuration for set A at T1 and T2 are the same. For example, suppose that for both T1 and T2, the same resource is configured and beam is mapping to these resources with the same order. Then in T1, UE report the best predicted RS index is 7. And in T2, after measurement for all RSs, UE find out the best beam is in RS 6. However, UE can just report the best measured RS index with 7 to TE. since TE didn’t know which beam is the best, it will only compare two indexes from UE reported at T1 and T2. If the two index is the same, TE will assume that predicted best beam is correct. Therefore, RAN4 needs to discuss how to avoid the issue.

Observation 16: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating.

The issue can be solved by several possible options.

- Option 1: Configure different resource for set A in T1 and T2.

- Option 2: Configure same resource for set A in T1 and T2 while the mapping relation of RS index and beam index is different. For example, suppose the beam index and RS index mapping relation for T1 is shown in Fig.6(a). For T2, TE will re-transmit all beams in set A on all RSs. TE will change the mapping relationship between RS index and beam index as shown in Fig.6(b). In this way, UE can only honestly report the best measured beam in T2.

Fig.6(a) beam index and RS index mapping relation for set A in T1

Fig.6 (b) beam index and RS index mapping relation for set A for T2

Proposal 17: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: For derive ground truth for best Top-1 beam, UE needs to measure for all beams in set A under high SNR. However, there is still some measurement uncertainty due to RF and BB. It’s challenging for UE to detect the best 1st beam even at high SNR due to the small gap between best 1st and 2nd beam.

Observation 2: For beam prediction accuracy, option 2 is not correct since UE can’t distinguish which is the best beam index due to measurement error.

Observation 3: From RAN1 performance monitoring discussion, the similar metric Top-K/M is under discussing, where at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s).

Proposal 1: For beam ID prediction accuracy performance metric, the KPI is

  - Top-K/N (%): the percentage of "at least one of the Top-N genie-aided beam is one of the Top-K predicted beams".

Observations 4: Differential reporting induces error propagation in absolute RSRP reconstruction:

- 1dB uncertainty per differential report (for 2dB step size)

- Cumulative effect when beam ranking changes between reports

Observation 5: Direct comparison of predicted vs. measured RSRP for 2nd-Kth beams becomes unreliable due to:

- Potential beam ranking changes

- Compounded quantization errors

Proposal 2: For BM-case 1, only define absolute RSRP accuracy for 1st-ranked predicted beam in report:

predicted L1-RSRP of beam index i – ground truth of L1-RSRP of beam index i.

where the index i is the 1st-ranked beam index in report.

Proposal 3: The network shall configure K=1 for the RSRP measurement report configuration, applied exclusively to the 1st-ranked predicted beam.

Proposal 4: For both BM case-1 and BM case-2, RAN4 to discuss whether there is impact on measurement period due to specific RX beam assumption.

Proposal 5: for BM-case 1, legacy measurement period can be used as baseline. FFS for the impact of specific RX beam(s).

Proposal 6: For BM-case 2, RAN4 to discuss impact on measurement requirement, e.g. measurement time, prediction time, measurement restriction and scheduling restriction, etc.

Observation 6: When best predicted TX beam is in set B, UE knows the best RX beam. When best predicted TX beam is in set A but not in set B, it’s still possible for UE to know the RX beam corresponding to the best TX beam.

Proposal 7: RAN4 may know RX beam based on UE capability.

Observation 7: SINR distribution for all UE group will span a large SINR range for all UE groups.

Observation 8: For K=1, measurement accuracy error will degrade performance a lot for all UE groups.

Observation 9: With K=4/1, when maximum SINR is larger than 0dB(UE group 1,2,3), prediction accuracy can be nearly around or higher than 90%.

Observation 10: When -5dB≤maximum SINR<0dB(UE group 4), prediction accuracy for K/1 will degrade to 72%. If relax to K=4/2, the performance will increase to 87%.

Observation 11: When -10dB≤maximum SINR<-5dB, prediction accuracy for all KPI is bad.

Proposal 8: Impact of measurement error has impact on prediction accuracy requirement, RAN4 to define SNR side condition for the best TX beam in Set B.

Proposal 9: Considering measurement error impact, RAN4 to relax beam index prediction KPI to Top K/N.

Proposal 10: For OTA test, prefer to split the test into two parts.

- Part 1: Verify AI model prediction algorithm, using single AOA model.

- Part 2: Verify input data accuracy of model AI based on RX beam sweeping, using multi-AOA model.

Observation 12: If Emulated signal considers both TX and RX beamforming gain, RX beam pattern is UE specific. In order to reduce test complex, prefer to design a common pre-defined RX beam pattern for both TE and UE.

Observation 13: if a fixed RX beamforming gain is used for multi-path TX signals, there will be some mismatch between emulated signal for test and original signals for training. The mismatch impact needs to be evaluated.

Observation 14: For simplified channel model, L1-RSRP delta between different TX beams will increase if cluser number and AOA number is further reduced. It’s more easiler for AI to distingwish the best Top-K TX beam. The AI prediction requirement will be relaxed.

Proposal 11: For single AOA, RAN4 to discuss whether it’s possible to design common pre-defined RX beam pattern to generate emulated single AOA signals.

Proposal 12: For simplified multi-AOA CDL model, cluster number can be reduced from spatial angle offset and power aspect:

- Combine clusters if AOA offset between these clusters is smaller than a threshold

- Remove cluster with low power.

Proposal 13: If the test metric is beam index prediction accuracy, UE may need to report ground truth of N best beam index for set A.

Observation 15: For deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Proposal 14: For deriving ground truth of set A, SNR level should satisfy that for Top-M beams, the measurement error is small. RAN4 to discuss how to decide M and how to set SNR.

Proposal 15: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 16: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

Observation 16: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating.

Proposal 17: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Reference

[1] R1-2501595, FL summary #6 for AI/ML in beam management