3GPP TSG-RAN WG4 Meeting #114bis																		R4-2503315

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	Xiaomi

Title: 	Initial D2R SNR simulation results

Agenda item:	7.24.2.1

Document for:	Discussion

- Introduction

It is decided that D2R SNR simulation is needed. Preliminary simulation assumptions was agreed in last RAN4 meeting [1]. This contribution provides initial D2R SNR simulation results based on certain assumptions. We also discussed some missing simulation assumptions.

- Discussion

In our simulation, the following assumptions are used. The highlighted parameters are the options we chose from the WF [1].

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 900 MHz |
| SCS | 15 kHz as baseline |
| Block structure | Depend on RAN1 |
| Channel model | AWGN |
| Number of Tx/Rx chains for Ambient IoT device | 1 |
| Number of BS antenna elements | 1 |
| Reference data rate | 1 kbps (M) |
| Message size | Option 1： {20 bits, 96 bits, 400 bits} are considered for message size. |
| BLER target | 10% |
| Sampling frequency（SFO） | Randomly select a value from the range of [104 ppm~105 ppm] for device 1 |
| Transmission bandwidth（DSB） | Option 2: 15kHz |
| [OOK/BPSK chip rate] | 7.5kHz based on the TBW |
| Receiver bandwidth | 5 order Butterworth LPF with 8 kHz cut off |
| Waveform (CW) | unmodulated single tone |
| Modulation | BPSK |
| Line code | Manchester encoding |
| FEC | Option 2：1/3 Convolutional Code |
| ADC bit width | Ideal |
| D2R receiver | coherent receiver |
| Repetition | None |

We have some initial simulation results,

| amble(s) | Message size | BLER target | SNR (dB) |
| --- | --- | --- | --- |
| preamble | 20bits | 10% | -2.9 |
| preamble | 96bits | 10% | 8.9 |
| preamble | 400bits | 10% | Can’t be decoded |
| preamble+midamble | 20bits | 10% | -8 |
| preamble+midamble | 96bits | 10% | -6.3 |
| preamble+midamble | 400bits | 10% | -4.7 |

It can be seen that preamble/midamble configuration have impact on the simulation results, we propose add a parameter to the simulation assumptions. The details can be decided after RAN1 draw conclusions [2]. The possible way in RAN4 is to choose one configuration from RAN1 decided configurations.

Proposal: Add preamble/midamble configuration parameter to simulation assumptions. Detail can be decided after RAN1 draw conclusions.

- Conclusion

This contribution provides initial D2R SNR simulation results. For the simulation assumptions, we have the following proposal,

Proposal: Add preamble/midamble configuration parameter to simulation assumptions. Detail can be decided after RAN1 draw conclusions.

- References

[1] R4-2502859, “WF on requirements for A-IoT BS and CW”, Huawei, RAN4#114

[2] R1-2502443, “Discussion on timing acquisition and synchronization for Ambient IoT”, Xiaomi, RAN1#120bis