3GPP TSG-RAN WG4 Meeting #114bis																		R4-2503316

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	Xiaomi

Title: 	Discussion on AIoT device 1 RF requirements

Agenda item:	7.24.2.2

Document for:	Discussion

- Introduction

Device 1 RF requirements were discussed in last RAN4 meeting, the agreements are open issues are captured in the WF [1]. This contribution analyzes the remaining issues to provide our views.

- Discussion

  - Tx requirements

2.1.1 Transmit output power

For device 1 backscattering power requirement, the details should be decided. First is if it’s defined as EIRP or TRP.  It’s not difficult to conclude EIRP considering the test cost and the communication with reader.

Proposal 1: Device 1 output power is defined as EIPR not TRP.

However, if EIRP is used as the metric, an issue how to define the requirement for different directions comes up. For AIoT device, we don’t think FR2 spherical coverage approach is a good reference for device considering the test cost. Peak EIRP can be considered. There may be one issue needs some discussion if one peak EIRP direction is sufficient for AIoT deployment. If more than one EIPR directions are considered, NR FR2 BS approach can be a reference. But considering the very low-cost device, this discussion should be careful to avoid bringing too much burden to the commercial production.

Proposal 2: Peak EIRP can be considered as the requirement for device 1. If more than one peak EIRP directions are defined can be discussed further.

For the exact EIRP requirement, the maximum CW input power to device should be decided. It can be calculated according to the assumptions in the co-existence simulation. Reader output power is assumed as 33 dBm and the minimum distance between reader and device is 1m. FSPL for 1m distance of 900MHz is ~31 dB. To have some margin, the maximum input power to device can be assumed approximately as 3 dBm.

Proposal 3: The maximum input CW power to calculate device 1 radiated power can be assumed as 3 dBm.

It seems there’s no strong justification to choose OOK or BPSK modulation for the radiated power. If the same CW power is received by device, the backscattered OOK power is 3 dB less than BPSK. BPSK SNR threshold is less than OOK, so we think BPSK modulation is more proper to be used as the output power modulation.

Proposal 4: Radiated output power uses BPSK modulation.

For the output power level, it needs more discussion considering the device performance and implementation margin.

2.1.2 Occupied bandwidth

Occupied bandwidth is usually a regulation requirement. If it needs to be defined according to regulation, the requirement can be defined following NR approach as following.

Proposal 5: If OBW needs to be defined according to regulation, it can be defined as following,

The occupied bandwidth for all transmission bandwidth configurations shall be less than the channel bandwidth.

2.1.3 Modulation quality

Modulation quality is one of the valid RF requirements to guarantee the link performance. We still think modulation quality is defined.

Proposal 6: EVM requirement is defined for Device 1.

The EVM requirement should be different for BPSK and QPSK. EVM can be derived from SNR using the following equation,

EVM = 1/sqrt(SNR)

Tx signal should transmit a good SNR signal to guarantee the SNR at receiver side. After the signal is traveled in the channel and combined at receiver side, Tx noise contribution should be at a relatively low level compared with receiver NF. The situation may be different for large signal and small signal. If it’s large signal, Tx SNR is large enough that there’s no DEMOD problem. If it’s small signal, receiver NF should be the dominate noise. If the EVM requirement is the same for maximum and minimum output signal, minimum output signal SNR should be taken as reference. If Tx signal’s SNR is 10 dB higher than the receiver demod threshold, Tx noise contribution to the final SNR is negligible. So we have the following proposal

Proposal 7: Device 1 Tx signal SNR is defined [10] dB higher than receiver DEMOD SNR threshold.

For the small frequency shift part, we’re not sure if it belongs to modulation quality requirement. But of course EVM can be measured for small frequency shift mode if it needs to be tested.

Observation: EVM can be tested for small frequency shift mode if it needs to be tested.

2.1.4 Emission requirement

The emission requirement discussion may be a little difficult that there’re many aspects to be considered. First is if frequency shift mode emission needs to be tested. Considering the test is OTA test, and the emission requirement may be defined to span a large frequency range, only testing no small frequency shift mode brings benefit to the device manufacturer. For the emission performance, our understanding is that the two modes performance can be considered very similar because no Tx filter is assumed.

Proposal 8: Emission requirement is only tested for no small frequency shift mode.

For the detail requirement, both device performance and the regulation should be considered.

  - Rx requirements

2.2.1 Reference sensitivity

For reference sensitivity, NR REFSENS approach can’t be used because no throughput methodology in AIoT system. BER or BLER may also not be very suitable. If BER or BLER approach is used, device needs to decode enough number of bits to measure 1% BER. Success rate is used to test integrator’s REFSENS in [2], it may be a candidate for device 1 REFSENS requirement.

Proposal 9: Success rate can be considered as the criteria for device 1 REFSENS requirement.

2.2.2 Maximum input power

For device 1, there’re two kinds of input power, i.e. R2D signal and CW signal. For CW signal, it may not be very necessary that the maximum radiated power requirement is assumed that the input CW signal is the maximum input CW signal. For R2D signal, there’s no AGC issue for device 1, we’re not sure if it’s very necessary. Generally, we think device 1 is very low-cost device, so only defining necessary requirements bring benefits to the commercial success. For RFED circuit, small signal is relatively more challenge than large signal. So we have the following proposal.

Proposal 10: Maximum input power requirement is not defined for device 1.

- Conclusion

This contribution provides our views for the device 1 RF requirements. We have the following proposals,

For Tx requirements,

Proposal 1: Device 1 output power is defined as EIPR not TRP.

Proposal 2: Peak EIRP can be considered as the requirement for device 1. If more than one peak EIRP directions are defined can be discussed further.

Proposal 3: The maximum input CW power to calculate device 1 radiated power can be assumed as 3 dBm.

Proposal 4: Radiated output power uses BPSK modulation.

Proposal 5: If OBW needs to be defined according to regulation, it can be defined as following,

The occupied bandwidth for all transmission bandwidth configurations shall be less than the channel bandwidth.

Proposal 6: EVM requirement is defined for Device 1.

Proposal 7: Device 1 Tx signal SNR is defined [10] dB higher than receiver DEMOD SNR threshold.

Observation: EVM can be tested for small frequency shift mode if it needs to be tested.

Proposal 8: Emission requirement is only tested for no small frequency shift mode.

For Rx requirements,

Proposal 9: Success rate can be considered as the criteria for device 1 REFSENS requirement.

Proposal 10: Maximum input power requirement is not defined for device 1.

- References

[1] R4-2502858, “WF on requirements for A-IoT deviceR4-2420470”, RAN4#114, CMCC

[2] ETSI EN 302 208