3GPP TSG-RAN WG4 Meeting #114bis																		R4-2503317

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	Xiaomi

Title: 	Discussion on AIoT CW RF requirements

Agenda item:	7.24.2.3

Document for:	Discussion

- Introduction

After one meeting discussion for CW requirement, many issues are still open [1]. This contribution provides our analysis for the remaining issues for CW node requirements.

- Discussion

2.1 ON/OFF power

For the OFF power requirement, there was offline comment that the interference to other systems. Technically, it’s a valid argument. If it’s agreed to be defined, it seems NR BS off power can be a reference. For AIoT, reader off power requirement is not decided yet, our understanding is that CW off power requirement can be defined following reader off power.

Proposal 1: Define CW off power requirement following reader off power value.

There may be another discussion point that if transient period requirement needs to be discussed. RAN1 spec may have some definition for device like RFID specification [2], Tag Transmit-to-Receive Turn-Around Time and Receive-to-Transmit Turn-Around Time. Our understanding is that it’ll be much longer than the CW node transient period time capability. So it seems no need to define the transient period requirement for CW node.

Tpri = 1 / BLF, BLF is R times of data rate, so Tpri is at least tens of us. For Receive-to-Transmit Turn-Around Time, device decode and procession time should be considered, so it can be very short. Based on this analysis, we think it’s not necessary to define CW transient period requirement.

Proposal 2: It’s not necessary to define CW transient period requirement.

2.4 Phase noise

For phase noise, it’s not decided if it should be defined as a requirement. Usually, phase noise is a LO performance which is not defined as a requirement. In TN OFDM signal, phase noise performance impacts Tx EVM performance, phase noise should be in a certain level to meet EVM requirement. However, for AIoT system, there’s no detail study on phase noise impact yet. In our understanding, phase noise impact may be separated to two parts, first to reader side receiver noise, second to device Tx signal quality. Reader receiver noise impact is also related to CW cancellation capability, it’s hard to say what’s the performance requirement for CW phase noise. The severe impact from CW is at the carrier wave frequency point not the phase noise frequency range. For device Tx signal quality, phase noise may not have big impact for OOK and BPSK signal, but the conclusion may need some theory analysis, or simulation. So in our understanding, there’re many aspects not being studied seriously and clearly, it’s hard to draw the conclusion if phase noise requirement should be defined and what’s the exact requirement.

Observation 1: There’re many aspects related to phase noise not being studied seriously and clearly, it’s hard to draw the conclusion if phase noise requirement should be defined and what’s the exact requirement.

Proposal 3: Leave the phase noise part to implementation in R19. Whether it needs be defined for D2T2 scenario should be studied in R20 WI.

2.3 Unwanted emission and spurious emissions

It’s not clear how to define unwanted emission for CW signal because there’s no channel bandwidth for CW. We have the following proposal,

Proposal 4: Unwanted emission requirement is not defined.

For spurious emission requirement, it was agreed that NR requirement is baseline. For the spurious emission boundary, D2R CBW may be considered to make the emission to other systems under control.

Proposal 5: The boundary for spurious emission requirement can consider D2R CBW if there’s no implementation difficulty.

2.5 Tx Intermodulation

In our understanding, CW node can be considered as a NW similar node not a device because it’s fixed and is deployed in some place by a plan. So BS Tx intermodulation requirement motivation can be taken as a reference [3]. BS Tx intermodulation is defined for co-location scenario. R19 AIoT deployment is very simple, our understanding is no co-location deployment with other BS for CW node, so Tx intermodulation requirement is not needed.

Observation: No co-location deployment assumption with other BS for CW node is foreseen in R19 AIoT WI.

Proposal 6: Tx intermodulation requirement for CW node is not defined.

- Conclusion

This contribution provides our further analysis for AIoT CW requirement. We have the following proposals,

For ON/OFF power,

Proposal 1: Define CW off power requirement following reader off power value.

Proposal 2: It’s not necessary to define CW transient period requirement.

For phase noise,

Observation 1: There’re many aspects related to phase noise not being studied seriously and clearly, it’s hard to draw the conclusion if phase noise requirement should be defined and what’s the exact requirement.

Proposal 3: Leave the phase noise part to implementation in R19. If it needs be defined for D2T2 scenario should be studied in R20 WI.

For unwanted emission and spurious emissions,

Proposal 4: Unwanted emission requirement is not defined.

Proposal 5: The boundary for spurious emission requirement can consider D2R CBW if there’s no implementation difficulty.

For Tx intermodulation,

Observation: No co-location deployment assumption with other BS for CW node is foreseen in R19 AIoT WI.

Proposal 6: Tx intermodulation requirement for CW node is not defined.

- References

[1] R4-2502859, “WF on requirements for A-IoT BS and CW”, CMCC, RAN#114.

[2] EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard, Specification for RFID Air Interface Protocol for Communications at 860 MHz – 960 MHz

[3] TR 38.817-2