3GPP TSG-RAN4 Meeting # 114bis	R4-2503319

# Wuhan, China, 7th – 11nd April, 2025

Title:	NTN and IoT-NTN HPUE co-existence study and calibration

Source:	Xiaomi

Agenda item:	7.10.2.1

Document for:	Approval

- Introduction

In the RAN plenary meeting RAN#104, there was an agreement for a new Rel-19 RAN4 led WI Enhanced requirements and test methodology for NR and IoT NTN [1]. The baseline assumptions agreed in RAN4#112bis meeting have been captured in WF [2]. In this document, Xiaomi provides the simulation study results of the follow scenarios:

- NR-NTN PC1 Scenario 4

- NR-NTN PC1.5 Scenario 4

- NR-NTN PC2 Scenario 4

- IoT-NTN PC1 Scenario 4

- IoT-NTN PC2 Scenario 4

- NR-NTN and IoT-NTN HPUE co-existence study

In this clause, Xiaomi provides the coexistence simulation results for NR-NTN and IoT-NTN high power UE.

2.1 Simulation results for NR-NTN HPUE

NR-NTN PC1 Scenario 4:

|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | ACLR/ACIR | Requirement | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | ACIR | 18.5 |  |  |  |  | 7.54 | 5.33 | 3.9 | 2.8 |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 17.3 |  |  |  | 8.06 | 5.98 | 4.51 | 3.21 |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | ACIR | 18.8 |  |  |  | 10.42 | 7.61 | 5.58 | 4.12 | 2.94 |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | ACIR | 17.6 |  |  | 10.9 | 8.34 | 6.31 | 4.65 | 3.48 | 2.59 |
|  |  |  |  |  |  |  |  |  | 26 | 28 | 30 | 32 |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 29.3 | 7.27 | 5.8 | 4.57 | 3.57 |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | ACIR | 29.2 | 7.11 | 5.73 | 4.54 | 3.59 |  |  |  |  |
|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | ACLR/ACIR | Requirement | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 28 |
| 5%-tilt | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | ACIR | 23.3 |  |  |  | 11.18 | 6.69 | 4.02 | 2.52 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 22.7 |  |  |  |  | 8.78 | 5.67 | 3.59 | 2.29 |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | ACIR | 22.4 |  |  |  | 9.3 | 5.48 | 3.31 | 2.07 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | ACIR | 25.1 |  |  |  |  | 9.76 | 6.3 | 3.98 | 2.52 |
|  |  |  |  |  |  |  |  |  | 34 | 36 | 38 | 40 |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 37.3 | 10.05 | 6.5 | 4.14 | 2.65 |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | ACIR | 37.2 | 9.94 | 6.31 | 4.07 | 2.6 |  |  |  |  |

NR-NTN PC1.5 Scenario 4:

|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | ACLR/ACIR | Requirement | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | ACIR | 15.8 |  |  | 9.84 | 6.79 | 4.83 | 3.5 |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 15.3 |  |  | 8.08 | 6.07 | 4.46 | 3.26 |  |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | ACIR | 16.3 |  |  |  | 7.45 | 5.23 | 3.66 | 2.22 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | ACIR | 16.0 |  |  | 8.59 | 6.6 | 4.96 | 3.72 |  |  |
|  |  |  |  |  |  |  |  |  | 26 | 28 | 30 | 32 | 34 | 36 | 38 |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 32.5 | 10.31 | 8.33 | 6.68 | 5.26 | 4.11 | 3.26 | 2.5 |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | ACIR | 32.4 |  |  | 6.61 | 5.21 | 4.07 | 3.12 |  |  |
|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | ACLR/ACIR | Requirement | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 26 |
| 5%-tilt | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | ACIR | 20.2 |  |  | 14.99 | 8.22 | 5.25 | 3.23 |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 22.8 |  |  |  | 13.98 | 9.01 | 5.8 | 3.72 | 2.35 |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | ACIR | 20.5 |  |  |  | 9.22 | 5.5 | 3.5 | 2.19 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | ACIR | 23.2 |  |  |  |  | 9.94 | 6.36 | 4.03 | 2.57 |
|  |  |  |  |  |  |  |  |  | 36 | 38 | 40 | 42 | 44 |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 40.4 | 13.03 | 8.39 | 5.43 | 3.41 | 2.1 |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | ACIR | 40.1 | 12.33 | 7.99 | 5.08 | 3.25 | 2.07 |  |  |  |

NR-NTN PC2 Scenario 4:

|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | ACLR/ACIR | Requirement | 6 | 8 | 10 | 12 | 14 | 16 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | ACIR | 12.9 |  |  | 8.03 | 5.71 | 4.06 | 2.95 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 12.6 |  |  | 7.15 | 5.44 | 3.99 | 2.88 |  |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | ACIR | 12.8 |  |  | 8.05 | 5.74 | 3.96 | 2.76 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | ACIR | 14.3 |  |  |  | 6.96 | 5.18 | 3.84 | 2.77 |
|  |  |  |  |  |  |  |  |  | 12 | 14 | 16 | 18 |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 14 | 6.28 | 5.04 | 4.05 | 3.24 |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | ACIR | 14.5 | 6.73 | 5.42 | 4.35 | 3.48 |  |  |  |
|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | ACLR/ACIR | Requirement | 10 | 12 | 14 | 16 | 18 | 20 | 22 |
| 5%-tilt | 1.5 | Y | TN cluster | Urban | non-AAS | LEO-1200 | ACIR | 16.8 |  |  | 9.65 | 5.85 | 3.69 | 2.35 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 19.6 |  |  |  | 11.07 | 7.21 | 4.52 | 2.88 |
|  | 1.5 | Y | TN cluster | Urban | non-AAS | GEO | ACIR | 17.5 |  |  | 11.52 | 7.16 | 4.26 | 2.59 |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | ACIR | 19.6 |  |  |  | 10.74 | 6.96 | 4.44 | 2.83 |
|  |  |  |  |  |  |  |  |  | 20 | 22 | 24 | 26 |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | ACIR | 23.8 | 10.35 | 6.95 | 4.55 | 2.94 |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | ACIR | 24.5 | 12.62 | 8.53 | 5.64 | 3.67 |  |  |  |

3.2 Simulation results for IoT-NTN HPUE

IoT-NTN PC1 Scenario 4:

|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | SCS / UE# | ACLR/ACIR | Requirement | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 15kHz / 9 | ACIR | 0 | 3.57 | 2.6 | 1.88 | 1.35 |  |  |  |  |  |  | 0.12 |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 2 | 6.39 | 4.67 | 3.38 | 2.42 |  |  |  |  |  |  | 0.18 |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 15kHz / 9 | ACIR | 2 | 6.74 | 4.97 | 3.61 | 2.58 |  |  |  |  |  |  | 0.18 |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 3.75kHz / 36 | ACIR | 9.6 |  |  |  | 8.87 | 6.53 | 4.75 | 3.99 |  |  |  | 0.84 |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | 15kHz / 9 | ACIR | 14.5 |  |  |  |  |  |  | 7.09 | 5.59 | 4.38 | 3.4 | 2.62 |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 21.2 |  |  |  |  |  |  |  |  |  | 6.91 | 5.55 | 4.44 | 3.53 |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | 15kHz / 9 | ACIR | 19.1 |  |  |  |  |  |  |  |  | 6.82 | 5.4 | 4.25 | 3.32 |  |
|  |  |  |  |  |  |  |  |  |  | 20 | 22 | 24 | 26 | 28 | 30 | 32 |  |  |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | 3.75kHz / 36 | ACIR | 29.8 | 14.77 |  |  | 7.76 | 6.17 | 4.87 | 3.81 |  |  |  |  |  |  |
|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | SCS / UE# | ACLR/ACIR | Requirement | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 28 |
| 5%-tilt | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 15kHz / 9 | ACIR | 6.7 | 8.38 | 5.57 | 3.64 | 2.35 |  |  |  |  | 0.24 |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 8.8 |  | 10.04 | 6.69 | 4.39 | 2.88 |  |  |  | 0.46 |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 15kHz / 9 | ACIR | 10.4 |  |  | 8.19 | 5.44 | 3.55 | 2.28 |  |  | 0.59 |  |  |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 3.75kHz / 36 | ACIR | 17.3 |  |  |  |  |  | 9.86 | 6.58 | 4.3 | 2.78 |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | 15kHz / 9 | ACIR | 25.1 |  |  |  |  |  |  |  |  | 12.4 | 8.44 | 5.57 | 3.62 | 2.33 |
|  |  |  |  |  |  |  |  |  |  | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 31.0 | 35.31 |  |  |  | 9.31 | 6.2 | 4.05 | 2.62 |  |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | 15kHz / 9 | ACIR | 28.0 | 22.84 |  |  | 7.65 | 5.02 | 3.25 | 2.09 |  |  |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | 3.75kHz / 36 | ACIR | 39.5 |  |  |  |  |  |  |  |  | 10.45 | 6.98 | 4.59 | 2.98 |  |

IoT-NTN PC2 Scenario 4:

|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | SCS / UE# | ACLR/ACIR | Requirement | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 15kHz / 9 | ACIR | 0 | 1.92 | 1..4 | 1.01 | 0.73 |  |  |  |  |  |  | 0.06 |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 0 | 2.88 | 2.02 | 1.41 | 0.98 |  |  |  |  |  |  | 0.08 |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 15kHz / 9 | ACIR | 0 | 3.47 | 2.54 | 1.85 | 1.34 |  |  |  |  |  |  | 0.13 |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 3.75kHz / 36 | ACIR | 5.8 |  | 8.11 | 5.96 | 4.32 | 3.1 |  |  |  |  |  | 0.39 |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | 15kHz / 9 | ACIR | 9.1 |  |  |  | 7.39 | 5.9 | 4.66 | 3.65 |  |  |  | 1.24 |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 14.8 |  |  |  |  |  |  | 7.31 | 5.74 | 4.5 | 3.49 | 2.69 |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | 15kHz / 9 | ACIR | 14.0 |  |  |  |  |  | 7.86 | 6.26 | 4.95 | 3.89 |  | 2.38 |  |  |
|  |  |  |  |  |  |  |  |  |  | 20 | 22 | 24 | 26 | 28 |  |  |  |  |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | 3.75kHz / 36 | ACIR | 24.3 | 8.19 | 6.52 | 5.16 | 4.05 | 3.16 |  |  |  |  |  |  |  |  |
|  | Isolation | Wrap-around | Statistics | Deployment | NR BS Type | SAN Orbit | SCS / UE# | ACLR/ACIR | Requirement | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 |
| 5%-tilt | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 15kHz / 9 | ACIR | 2.2 | 7.82 | 5.17 | 3.37 | 2.17 |  |  |  |  |  |  | 0.09 |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 4.4 |  | 8.09 | 5.37 | 3.49 | 2.25 |  |  |  |  |  | 0.14 |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 15kHz / 9 | ACIR | 6.0 |  |  | 7.61 | 5.05 | 3.29 | 2.12 |  |  |  |  | 0.22 |  |  |
|  | 1.5 | Y | TN cluster | Urban | AAS | GEO | 3.75kHz / 36 | ACIR | 13.2 |  |  |  |  |  | 8.44 | 5.53 | 3.61 | 2.32 |  | 0.95 |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | 15kHz / 9 | ACIR | 19.8 |  |  |  |  |  |  |  |  | 10.47 | 7.06 | 4.66 | 3.03 |  |
|  |  |  |  |  |  |  |  |  |  | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | LEO-1200 | 3.75kHz / 36 | ACIR | 24.7 | 12.57 | 8.56 | 5.69 | 3.72 | 2.4 |  |  |  |  |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | 15kHz / 9 | ACIR | 23.4 | 10.43 | 7.01 | 4.61 | 2.99 |  |  |  |  |  |  |  |  |  |
|  | 0 | Y | TN cluster | Urban | AAS | GEO | 3.75kHz / 36 | ACIR | 33.6 | 48.56 |  |  |  |  | 10.59 | 7.1 | 4.68 | 3.04 |  |  |  |  |

Proposal 1: Propose RAN4 consider the simulation results in this document during the discussion of the ACIR requirement.

- Conclusion

Proposal 1: Propose RAN4 consider the simulation results in this document during the discussion of the ACIR requirement.

- Reference

[1]	RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair (Huawei)

[2]	R4-2417122,”WF on co-existence study and HPUE for NR-NTN”, 3GPP RAN4#112bis, October 2024

[3]	TR38.863-i30