# 3GPP TSG-RAN WG4 Meeting #114 bis	R4-2503321

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:			7.10.2.3.1

Source: 	Xiaomi

Title: 						RF requirements for IoT-NTN

Document for:	Approval

- Introduction

Rel-19 NTN agreed to introduce HPUE for NR-NTN/Iot-NTN operation:

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In this contribution, views on remaining open issues were provided.

- Discussion

ACLR requirements

In last RAN4 meeting, initial conclusion was reached for co-existence study as following:

Issue 1-1-2: ACLR for eMTC based Iot-NTN for PC1/2; Agreement: o Option 1: Re-use PC3 eMTC based Iot-NTN ACLR 30dBc. o Option 2: 31dBc ACLR

On isolation distance, we prefer to align with previous assumption the deployment scenario and restriction shall be consistency independent to device power classes which belongs to UE capability.

On NTN operation, the critical issue is coverage due to long distance between satellite over the space to terminal. With reasonable ACLR, it will bring benefits to improve uplink coverage with less MPR and or A-MPR.

Based on above consideration, we suggest to specify following ACLR values:

Proposal 1: Specify eMTC based Iot-NTN HPUE ACLR as following

- PC2: 30 dBc

- PC1: 30 dBc

Target device types and device type assumption

Issue 1-2-1: Discuss the unsettled requirements applicability for Iot-NTN: - Agreement (Ad Hoc): - No 2Tx PC1/PC2 requirement will be specified for IoT NTN in Rel-19; - Only consider non-handheld for PC1 1Tx eMTC based with full duplex Iot-NTN - FFS whether requirements should be defined for PC1 1Tx handheld UE for NB-Iot based Iot-NTN Updated Status: (For Information) Table: Current status for requirements consideration and feasibility for Iot-NTN Power Class; Antenna Numbers; Handheld and/or non-handheld. NB-Iot based; PC1; 1Tx; FFS. PC2; 1Tx; Both. eMTC based with half duplex; PC1; 1Tx; FFS. PC2; 1Tx; Both. eMTC based with full duplex; PC1; 1Tx; Non-handheld. PC2; 1Tx; Both.

In general, we prefer to leverage existing TN network design on FDD bands as much as possible with following thinking:

- Harmonize the common design between TN and NTN operation for echo-system

- Shared RF chain between NTN and TN operation on similar frequency range for handheld device for cost and form factor limitations

- Reduce RAN4 workload on leverage the conclusion from TN to NTN

Some companies proposed to consider GSM PA for supporting PC1. However, IoT devices may not support GSM operation and using GSM PA with bring additional cost for IoT device types.  We suggest to focus on non-hand held device types on supporting PC1 on IoT NTN operation.

Proposal 2: Only focus on non hand-held device type for PC1 IoT NTN including NB-IoT, eMTC with half-duplex, and eMTC with full duplex in Rel-19

Issue 1-1-5: MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC1/2 Agreement: - For MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC1/2 - For NB-IoT PC2, for A-MPR evaluations assume 25 dBc IQ-image and LO-leakage and 60 dBc CIM3 - For NB-IoT PC1, for MPR and A-MPR evaluations assume [28~34]dBc IQ-image and LO-leakage and 60 dBc CIM3. - For NB-IoT PC2, as a starting point consider re-use of PC3 MPR - Editor’s Note: In RAN4#113 there was also a related agreement: “As the starting point, consider the center single tone allocation as MPR=0 reference point, and introduce MPR for edge single tone allocation.”

For simulation of MPR and A-MPR evaluations assumption for the NB-IoT PC1, IQ-image value was not decided in last meeting.

Proposal 3：Assume 28dBc IQ-image for NB-IoT PC1 MPR and A-MPR evaluations.

Issue 1-1-6: MPR assumptions for eMTC based Iot-NTN for PC1/2 Agreement: - For MPR/A-MPR assumptions for eMTC based Iot-NTN for PC1/2 - For Cat M1, in simulations calibrate PA to just meet ACLR with [30~31] dBc ACLR and 1 dB MPR with 100 RB DFT-s-OFDM waveform.

For the MPR/A-MPR assumptions for eMTC based Iot-NTN for PC1/2, it should be aligned the ACLR assumption with the previous ACLR issue. Therefore:

Proposal 4： For Cat M1, in simulations calibrate PA to just meet ACLR with 30dBc ACLR and 1 dB MPR with 100 RB DFT-s-OFDM waveform.

Issue 1-1-7: P-MPR definition for NB-Iot based Iot-NTN. - Proposals - For NB-Iot based Iot-NTN, discuss the proposal, confirm it is applicable for all the power classes, and discuss whether it can be defined from Rel-18. - Discuss whether P-MPR should be applied to handheld UEs only and whether this needs to be specified or not.

In TS 36.101, P-MPR isn't exclusive to handheld UEs. It's a power management mechanism ensuring the UE's transmit power meets electromagnetic radiation safety and regulations across various scenarios.

While P-MPR is common in handheld devices, especially when users hold them close to their heads or bodies due to SAR issue, it also applies to other UE types like Vehicle-mounted UEs and Fixed-installed UEs.

Thus, P-MPR is a general power management parameter for various UE devices, not just handheld ones.

Proposal 5: RAN4 should consider P-MPR for both handheld UEs and non-handheld UEs

Issue 1-1-8: The Basic MOP tolerance (Including eMTC and NB-Iot based) - Proposals - For PC2: - Option 1: +2 / -3dB - Option 2: +2 / -2dB - For PC1: - Option 1: +2 / -3dB

During the recent RAN4 meetings, MOP tolerance specifications have been extensively reviewed. Notably, NR HPUE has achieved widespread commercial adoption, demonstrating both technological maturity and reliable performance in deployments. Given that the existing NR-NTN framework already defines MOP tolerance as +2 / -3dB, there is significant merit in reusing these specifications for IoT-NTN HPUE.

Proposal 6: Reusing +2/-3dB as MOP tolerance for IoT-NTN for PC2.

Proposal 7: Reusing +2/-3dB as MOP tolerance for IoT-NTN for PC1.

SAR Related issues:

Issue 2-1-1: Whether and how to assume some scenarios like satellite orbits and traffic model etc. for the study of SAR issue. - Proposals - Option 1: No need. - [E.g. The scheme has to be general enough to cover various scenarios;] - Option 2: Needed. - [E.g. If typical scenarios do not have an issue may be the discussion is not needed.] - Option 3: Others

Proposal 8: It is suggested RAN4 to consider some different scenarios for the study of SAR issue.

- Conclusion

In this contribution, views on remaining open issues were provided.

Proposal 1: Specify eMTC based Iot-NTN HPUE ACLR as following

- PC2: 30 dBc

- PC1: 30 dBc

Proposal 2: Only focus on non hand-held device type for PC1 IoT NTN including NB-IoT, eMTC with half-duplex, and eMTC with full duplex in Rel-19

Proposal 3：Assume 28dBc IQ-image for NB-IoT PC1 MPR and A-MPR evaluations.

Proposal 4： For Cat M1, in simulations calibrate PA to just meet ACLR with 30dBc ACLR and 1 dB MPR with 100 RB DFT-s-OFDM waveform.

Proposal 5: RAN4 should consider P-MPR for both handheld UEs and non-handheld UEs

Proposal 6: Reusing +2/-3dB as MOP tolerance for Iot-NTN for PC2.

Proposal 7: Using +2/-3dB as MOP tolerance for Iot-NTN for PC1.

Proposal 8: It is suggested RAN4 to consider some different scenarios for the study of SAR issue.

Reference

[1] R4-2503006, WF on HPUE UE requirement for IoT-NTN