3GPP TSG-RAN WG4 Meeting #114bis				                              R4-2503324

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.10.3.5

Title: 	Discussion on the performance part requirements for less than 5MHz NTN

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In the previous RAN4 meeting, the RRM core requirements [1] related to the less than 5MHz NTN were almost completed [2]. Accordingly, RAN4 is required to define the corresponding test cases to verify the RRM requirements in TS38.133 [3].

In this paper we firstly provide the considerations on the basic test configurations for the less than 5MHz NTN measurement. And the list of the test cases to verify the corresponding core requirements is proposed.

- Discussion

- Issue 1. Numerology

As RAN4 agreed RRM requirements for less than 5MHz NTN is considered for 15KHz SC only, we can propose that

R4#112 Agreement: - To confirm only 15KHz SC needs to be considered.

Proposal 1: It is necessary to define the test cases for 15KHz SC only.

- Issue 2. Channel bandwidth configuration

For NTN less than 5MHz, for the channel bandwidth and other relevant fundamental parameters were agreed in the previous meeting.

R4#112bis agreements Issue 1-1-1-1: Channel bandwidth assumption Agreements - 3MHz with 15 PRBs Issue 1-1-1-2: SSB/PBCH assumption Agreements - 12 PRBs Issue 1-1-1-3: PDCCH transmission bandwidth assumption Agreements - 15 PRBs

Proposal 2: For the test case configuration, only 3MHz channel bandwidth with 12PRBs SSB/PBCH and 15PRBs PDCCH needs to be defined.

- Issue 3: Methodology to introduce the test cases for NTN less than 5MHz

According to RAN4’s discussion on the core part, for NTN less than 5MHz, only part of new requirements were introduced in Rel19. For examples, for the requirements in IDLE/inactive mode, the core requirements for NTN in Rel18 can be reused.

Table 1. RRM impacts summary due to spectrum less than 5MHz

| RRM requirements | NTN requirements in TS38.133 v18.5.0 | Need the new requirements in Rel19 |
| --- | --- | --- |
| IDLE/inactive mode mobility | 4.2C Cell Re-selection for NR UE for Satellite Access | None |
| RRC connection mobility control | 6.2C RRC Connection Mobility Control for Satellite Access | None |
| Handover | 6.1C Handover for SAN | Yes |
| UE Tx timing, MTTD/ MRTD, timer accuracy, TA accuracy | 7.1C, 7.2C, 7.3C | None |
| RLM | 8.1C Radio Link Monitoring for Satellite Access 8.5C Link Recovery Procedures for Satellite Access | Yes |
| Measurement | 9.2C NR intra-frequency measurements for SAN 9.3C NR inter-frequency measurements for SAN | Yes |

However, for the test cases themselves, in our views in Rel18 test case for NTN [3, TS38.133 v18.6.0] the test configuration for 10MHz bandwidth was introduced only.

Observation 1: in Rel18 test case for NTN [3, TS38.133 v18.7.0] the test configuration for less than 5MHz was not included.

Therefore, in order to verify the performance of NTN less than 5MHz in which the legacy requirements were leveraged (e.g. the requirements for IDLE/Inactive mode, timing) the additional test configuration for NTN less than 5MHz needed be introduced.

Proposal 3: RAN4 can FFS on the additional testing configuration with less than 5MHz for NTN test cases in which the same requirements can be applied to all channel bandwidth supported.

On the other hand, for the new requirement specific for less than 5MHz NTN, e.g. handover, SSB index reading, RLM a, it is straightforward to define the new test cases

Proposal 4: The new test cases for NTN less than 5MHz with the new core requirements shall be defined in Rel19.

In summary, the following test cases for NTN less than 5MHz core requirement were proposed.

Proposal 5: The test cases for core requirement of NTN less than 5MHz are summarized below.

| NTN requirements in TS38.133 v19.x.x | Description |
| --- | --- |
| 6.1C.1.2 NR SAN FR1 – NR SAN FR1 Handover | Test the intra-frequency handover from NR SAN FR1 cell to NR SAN FR1 cell |
| 6.1C.2.2 NR SAN FR1 – NR SAN FR1 conditional handover | Test the intra-frequency conditional handover from NR SAN FR1 cell to NR SAN FR1 cell |
| 6.1C.3 NR SAN Satellite switching with re-synchronization | FFS |
| 8.1C Radio Link Monitoring for Satellite Access | Test SSB based RLM OOS/IS with 15RB PDCCH, non-DRX with 3MHz bandwidth |
| 8.5C Link Recovery Procedures for Satellite Access | Test SSB based BFD with 15RB PDCCH, non-DRX with 3MHz bandwidth |
| 9.2C NR intra-frequency measurements for SAN | Test SSB based intra-frequency NTN measurement with 12PRB SSBs, non-DRX with 3MHz bandwidth |
| 9.3C NR inter-frequency measurements for SAN | Test SSB based inter-frequency NTN measurement with 12PRB SSBs, DRX with 3MHz bandwidth |

- Summary

This paper proposes the main configurations parameters and test cases list to verify the RRM requirements related to NTN less than 5MHz.

Proposal 1: It is necessary to define the test cases for 15KHz SC only.

Proposal 2: For the test case configuration, only 3MHz channel bandwidth with 12PRBs SSB/PBCH and 15PRBs PDCCH needs to be defined.

Observation 1: in Rel18 test case for NTN [3, TS38.133 v18.7.0] the test configuration for less than 5MHz was not included.

Proposal 3: RAN4 can FFS on the additional testing configuration with less than 5MHz for NTN test cases in which the same requirements can be applied to all channel bandwidth supported.

Proposal 4: The new test cases for NTN less than 5MHz with the new core requirements shall be defined in Rel19.

Proposal 5: The test cases for core requirement of NTN less than 5MHz are summarized below.

| NTN requirements in TS38.133 v19.x.x | Description |
| --- | --- |
| 6.1C.1.2 NR SAN FR1 – NR SAN FR1 Handover | Test the intra-frequency handover from NR SAN FR1 cell to NR SAN FR1 cell |
| 6.1C.2.2 NR SAN FR1 – NR SAN FR1 conditional handover | Test the intra-frequency conditional handover from NR SAN FR1 cell to NR SAN FR1 cell |
| 6.1C.3 NR SAN Satellite switching with re-synchronization | FFS |
| 8.1C Radio Link Monitoring for Satellite Access | Test SSB based RLM OOS/IS with 15RB PDCCH, non-DRX with 3MHz bandwidth |
| 8.5C Link Recovery Procedures for Satellite Access | Test SSB based BFD with 15RB PDCCH, non-DRX with 3MHz bandwidth |
| 9.2C NR intra-frequency measurements for SAN | Test SSB based intra-frequency NTN measurement with 12PRB SSBs, non-DRX with 3MHz bandwidth |
| 9.3C NR inter-frequency measurements for SAN | Test SSB based inter-frequency NTN measurement with 12PRB SSBs, DRX with 3MHz bandwidth |

- References

- RP-240857

- R4-2502589

- 3GPP TS 38.133 v18.7.0