3GPP TSG-RAN WG4 Meeting #114bis				                                         R4-2503327

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.25.2.1

Title: 	Discussion on OD-SSB requirements

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1] for R19 NES was agreed and the objectives for OD-SSB are self-contained as below.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify the corresponding core requirements, for the above features [RAN4].

And in the last RAN4 meeting, the agreements for NES RRM were achieved in [2]. Therefore, in this discussion, we provided more analysis on the potential RRM impacts due to these aspects in Rel19 NES.

- Discussion

  - General On-demand SSB(OD-SSB) requirements

In the previous RAN4 meetings, whether the OD-SSB based cell identification before deactivated SCell measurement can be skipped were discussed but without any agreements.

R4#114 discussion Main proposals: - Recommended WF - Moderator suggests the group to check the following options(except the Case 1 Option 2). - The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios: - Case 1: No always-on SSB on the cell - Option 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band - Option 2-1: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [Xs] and then the OD-SSB is transmitted again with the same spatial reception parameter. - Other detail condition is FFS. (Note: this option has already covered in issue 1-2-3) - Option 2-2: UE has sent a valid measurement report based on OD-SSB for the SCell being activated [5]s before, and then the OD-SSB is transmitted again with the same spatial reception parameter same transmission power, and on the same frequency carrier. - Other detail condition is FFS. - Option 3: Extend referenceCell to OD-SSB SCell - Other option is not precluded - Case 2: Always-on SSB on the cell - Option 1: Cell identification can be always skipped - Option 1a: Cell identification can be always skipped in scenario 2 - Option 2: follow SCell activation known condition - Other option is not precluded

Firstly, for case1, given RAN1 agreements below, we can conclude that：

Observation 1: Since OD-SSB are not located at synchronization raster, UE can’t perform the cell identification with OD-SSB for the initial access.

R1#118 discussion Agreement: For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

On the other hand, OD-SSB based measurements for the deactivated SCell can be feasible with the valid timing and cell information. That is the pre-required information can be obtained from the serving cells associated with the SCells with OD-SSB if they can meet the specific conditions (e.g. adjacent together in both frequency domain and spatial domain).

Observation 2: For measurements on the deactivated SCell with OD-SSB UE can know the correct timing and other SSB index information from the other serving cell (e.g. PCell) if their timing and spatial domain information are associated together.

Also the specific known condition for the cell identification can be feasible. But if such known condition can’t be satisfied, how UE can obtain the necessary timing information shall be considered as well. In our view, in case of unknown condition, UE can also rely on the timing of associated serving cell.

Observation 3: UE can measure the OD-SSB without the cell identification by:

- The known conditions whether the valid synchronization timing can be obtained based on the previous measurements results or

- other serving cell relationship

Therefore, in order to simplify our specification, we prefer to define such condition for case 1by Option 1 directly.

Secondly, for case2, since there accompanied always-on SSB is available, it is quite simple for UE implementation to detect/identify SCell to be measured with the always-on SSB indeed.

Thus, we can propose that:

Proposal 1: The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios:

- Case 1: No always-on SSB on the cell

  - Option 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

- Case 2: Always-on SSB on the cell

  - Case 1- OD-SSB based deactivated SCell L3 measurement

When the OD-SSB was triggered for SCell activation, the periodicity of OD-SSB is expected to be short as possible to enable UE can complete the measurement quickly which can save the network power. That is the exact meaning of “fast L3 measurement” in RAN4 agreements [4]. However, for the deactivated SCell, the periodicity of OD-SSB can be much sparser than these for the SCell activated or to be activated. As a result, UE need not to measure the OD-SSB so frequently to minimize UE’s power consumption. This is what “slow measurement” mean in comparison with the former one.

Thus, in the previous RAN4 meeting, the harmonized solution to balance the power consumption and the reliable measurement results via OD-SSB occasions is to restrict the periodicity of measurement based OD-SSB in the fast mode window (“Tfast”) and after that (e.g.“Tslow”) UE can relax the measurement with “measCycleSCell” or stop the measurement as agreements below.

Figure 1. OD-SSB measurement stages [4]

And in the last RAN4 meeting, the agreements on how to define the time window (Tfast) and corresponding UE measurement behavior was achieved in [4].

- Sub-issue: Sub-issue 1 –Lower bound for measurement in FMW

Sub-issue 3-1: Lower bound for measurement in FMW - Background - The agreement in last meeting is as follow. Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.. Wayforward: - Option 1: RAN4 not to define lower bound to measurement in FMW. - Option 2: Define a lower bound with OD-SSB periodicity as [10ms]. - The lower bound is only applied to measurement in FMW.

In principle, according to RAN4 agreement in the last meeting above, within Tfast UE is expected to measure the deactivated SCell as fast as possible. Otherwise，NW needs to transmit more SSBs over the longer duration. This is not beneficial to NW power saving. In other words, such assumption will revoke the intention to introduce OD-SSB.

Figure 2. OD-SSB Scenario 2A measurement

Observation 4: the lower bound (e.g. the one longer than the OD-SSB periodicity itself) for the measurement interval within Tfast may have impacts the network power saving expected.

Therefore, in our view from the network power saving perspective which is the key objective of OD-SSB, it is unnecessary to define the lower bound of measurement periodicity within the fast measurement window.

Proposal 2: RAN4 needs NOT to define lower bound to measurement in FMW.

- Sub-issue 2: T2 related

Agreement: RAN4 to define measurement requirement based on T2. - When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. - Option 3: UE is required to perform FMW after T2. - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to meet fast cell identification and measurement. - The start point of T2 is based on - Option 1: the time point Tend - Option 2: the latter point of L3 measurement reporting for FMW and Tend - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend - Option 4: the time point of OD-SSB deactivation - Other options are not precluded Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3. - The duration is - Option 1: 5*max(measCycleSCell, DRX cycles) [– OD-SSB periodicity] - Option 2: 5* measCycle when measCycle =160 ms - Other options are not precluded - The activation and deactivation of OD-SSB is fully up to NW. What’s the UE’s behaviour when OD-SSB further activation/reconfiguration after T2? - WF/Agreements RAN4 to define measurement requirement based on T2. - When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. Option 3: UE is required to perform FMW after T2. - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to meet fast cell identification and measurement.

In our view, the minimum T2(also denoted as Tsleep below) is up to the different purpose or UE behavior after Tfast and Tslow. For an example, as discussed in the last RAN4 meeting, there are several ways to define UE behaviors.

- When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. - Option 3: UE is required to perform FMW after T2.

UE with Option 1’s behavior is the typical usage after the fast measurement window. In other words, UE behavior within T2 is same as that within Tslow. It seems unnecessary to define additional behavior for UE within T2.

For Option2’s behavior, it is similar as known/unknown conditions for intra/inter-frequency measurements in RAN4. That is if UE performing the deactivated SCell measurement within a specific duration, the synchronization timing can be assumed valid. UE needs only performing RRM measurements but not the cell detection.

Observation 5: How to define UE’s behavior within “T2” is highly dependent with the other sequential stages before it.

Moreover, as discussed in the last meeting, there are too diverse understandings on the definition of T2. For an example, if the start point of T2 can’t be aligned, the exact requirement on T2 and corresponding UE behavior is unclear.

Figure 3.

Observation 5: There are too diverse understandings on the definition of T2 in RAN4 which results in the ambiguous UE behavior during T2 and the stages after T2.

Therefore, according to the previous RAN4 discussion and agreements above, we can clarify the UE behavior during the different stages under OD-SSB measurement:

- Within Tfast, UE needs perform the SCell measurement with OD-SSB per the OD-SSB periodicity

- Within Tslow, UE can perform the SCell measurement with OD-SSB per the legacy deactivated SCell measurement requirements (e.g. based on “measSCellCycle”)

- Within T2(Tsleep) from the deactivation OD-SSB(t3) to the next OD-SSB activation(t4), UE can stop any measurements

- Within Tre-act, the UE behavior can be upon the duration of Tsleep, which can be FFS.

Proposal 3: The overall procedure for deactivated SCell with OD-SSB shall be clarified especially after the initial OD-SSBs were deactivated.

Since after OD-SSB deactivation, there is no any OD-SSB available during T2, UE can stop its measurement on SCell in case 1 from that point. In our views, the it is better to define the start point of T2 window as the time point of OD-SSB deactivation (Option 4). It is more clear to distinguish the different stage of UE behaviors.

Proposal 4: The start point of T2 can be the time point of OD-SSB deactivation.

Proposal 5: UE can stop its measurement during T2. And from RAN4 perspective, there is no any requirements for UE within T2.

But after T2, when the OD-SSB was re-triggered, in order to further reduce UE power consumption, UE can follow the sparser measurements in comparison with the measurement in Tfast if the previous measurement result is still valid. This is similar with the known condition for the SCell activation in TS38.133 [3]. Otherwise, UE can follow the same UE behavior as that within Tfast.

Proposal 6: When the OD-SSB was re-activated, within Tre-activate (after T2) UE can follow the sparer measurements instead of OD-SSB cycle under the specific conditions.

- FFS on the conditions by which UE can relax its measurement with OD-SSB after T2

  - Case 2 - OD-SSB based deactivated SCell L3 measurement

In the last RAN4 meeting, on how to define the necessary requirement on SCell (de)activation with OD-SSB was discussed also with some high level agreements achieved.

Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered Agreement: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), - Option 1: The same approach in Case 1 can be reused. - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell). - Option 2: - If the serving cell is unknown, the same approach in Case 1 can be reused. - If the serving cell is known, UE follows legacy requirements based on measCycleSCell. - The known condition reuses the principle of legacy known condition. - This scenario applies at least when UE sends the meas. reporting. - FFS the case if the L3 meas. report cannot be triggered. - The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

For case 2, the motivation to trigger OD-SSB is same as that for Case 1. That is UE can perform the measurement based on the OD-SSB in the fast window and the other sparer measurement in the slow window.

One difference with Case 1 is there always-on SSB could be available for UE’s measurement. However, according to the latest RAN1 agreements below, from RAN4 RRM perspective, the requirements can define base on a single SSB instead of hybrid ones (e.g. only OD-SSB).

Observation 6: For Case 2, the requirements can define base on a single SSB instead of hybrid ones (e.g. only OD-SSB).

That is the conclusion for OD-SSB based L3 measurement for case 1 can be reused for case 2. Thus, we can support Option 1 as well.

R1#120 Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

Proposal 7: OD-SSB based deactivated SCell L3 measurement for Case 2 can follow the conclusions for Case 1.

  - OD-SSB based activation

In the last RAN4 meeting, on how to define the necessary requirement on SCell (de)activation with OD-SSB was NOT discussed. And the open issue left for this in R4#113 wad captured below.

Issue 1-4-1: General Agreement: - RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios. Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded.

For the detailed RRM requirements updates, in our view it needs more discussion under the different conditions and scenarios. For an example, as illustrated in Figure 4 below, for the case of “Scenario #2 and Case #1“ and known Scell activation case, if the first SSB transmitted after OD SSB trigger command can located before the time “n + THARQ+5ms+TfirstSSB +margin”, UE can follow the current behavior for Scell activation and corresponding requirements defined in TS38.133 for known cases can be reused. Otherwise, the requirements in current spec needs to be updated. However, for the unknown case since the more than one SSB needed for the SCell timing and frequency synchronization, the current requirements could not be reused with the condition above for known cases indeed.

Observation 7: When leveraging the current requirements in TS38.133 to the SCell activation/deactivation requirement with OD-SSB, whether SCell is known or unknown needs to be updated.

Figure 4.

Proposal 8: For the SCell activation/deactivation requirements with OD SSB, the UE known or unknow conditions shall be redefined as  :

During the period equal to max([5]*max(measCycleSCell, ODSSB_cycle), 5*DRX cycles), or max([5]*max(160ms, ODSSB_cycle), 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise, SCell in FR1 is unknown.

- Conclusion

In this contribution, serval issues related to RRM in Rel19 NES are discussed. The observations and proposals can be summarized as:

Observation 1: Since OD-SSB are not located at synchronization raster, UE can’t perform the cell identification with OD-SSB for the initial access.

Observation 2: For measurements on the deactivated SCell with OD-SSB UE can know the correct timing and other SSB index information from the other serving cell (e.g. PCell) if their timing and spatial domain information are associated together.

Observation 3: UE can measure the OD-SSB without the cell identification by:

- The known conditions whether the valid synchronization timing can be obtained based on the previous measurements results or

- other serving cell relationship

Proposal 1: The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios:

- Case 1: No always-on SSB on the cell

  - Option 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

- Case 2: Always-on SSB on the cell

Observation 4: the lower bound (e.g. the one longer than the OD-SSB periodicity itself) for the measurement interval within Tfast may have impacts the network power saving expected.

Proposal 2: RAN4 needs NOT to define lower bound to measurement in FMW.

Observation 5: How to define UE’s behavior within “T2” is highly dependent with the other sequential stages before it.

Proposal 3: The overall procedure for deactivated SCell with OD-SSB shall be clarified especially after the initial OD-SSBs were deactivated.

Proposal 4: The start point of T2 can be the time point of OD-SSB deactivation.

Proposal 5: UE can stop its measurement during T2. And from RAN4 perspective, there is no any requirements for UE within T2.

Proposal 6: When the OD-SSB was re-activated, within Tre-activate (after T2) UE can follow the sparer measurements instead of OD-SSB cycle under the specific conditions.

- FFS on the conditions by which UE can relax its measurement with OD-SSB after T2

Observation 6: For Case 2, the requirements can define base on a single SSB instead of hybrid ones (e.g. only OD-SSB).

Proposal 7: OD-SSB based deactivated SCell L3 measurement for Case 2 can follow the conclusions for Case 1.

Observation 7: When leveraging the current requirements in TS38.133 to the SCell activation/deactivation requirement with OD-SSB, whether SCell is known or unknown needs to be updated.

Proposal 8: For the SCell activation/deactivation requirements with OD SSB, the UE known or unknow conditions shall be redefined as  :

During the period equal to max([5]*max(measCycleSCell, ODSSB_cycle), 5*DRX cycles), or max([5]*max(160ms, ODSSB_cycle), 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise, SCell in FR1 is unknown.

- References

- RP-240830

- R4-2420098 WF on RRM requirements for Netw_Energy_NR_enh_Part1

- TS38.133 v18.6.0

- R4-2420098	WF on RRM requirements for Netw_Energy_NR_enh_Part1

- R4-2418282	Topic summary for [113][224] Netw_Energy_NR_enh_Part1