3GPP TSG-RAN WG4 Meeting #114bis				                                         R4-2503328

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.25.2.2

Title: 	Discussion on RRM requirements for adaptation of common signal/channel in NES

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1] for R19 NES was agreed and one of the objectives below was discussed in the last meeting [4].

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this meeting, we provide our further considers on open issues.

- Discussion

Firstly, the agreements made in RAN1 on adaptation mechanism of SSB and scenarios was summarized.

R1#116 Agreement Agreement For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study - Adaptation of SSB burst periodicity - Adaptation based on two SSB configurations where up to two configurations can be active - Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration - Adapting the transmitted number of SSBs within a SSB burst - Cell DTX for SSB adaptation - Whether to support new SSB burst periodicity value(s) - Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst) - New compact SSB burst(s) - Adapting the position of SSBs within a SSB burst - Other mechanisms/combinations are not precluded Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration R1#116bis Agreement Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized R1#117 Agreement Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB R1#118bis Agreement Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 R1#119 Agreement Agreement Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Agreement Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Agreement Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Further update to be made based on RAN1#119 progress if any. Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) R1#120 Agreement Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

  - L3 measurement

From RAN4 RRM perspective, in legacy RRM requirement, both intra and inter-frequency measurements requirement in TS38.133 [2] section 9.2 and 9.3 can be summarized as:

Intra-frequency measurements: Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1) DRX cycle; T SSB_measurement_period_intra. No DRX; max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil( 5 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Inter-frequency measurements: Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Observation 1: In current spec, the total L3 measurement delay requirements depend on the SMTC periodicity which is configured by NW RRC message can be reused for L1/L3 measurement

Thus, UE can follow the indicated SSB adaptation was indicated by NW RRC message or other SIB message as well.

Observation 2: UE’s measurement requirements can be little impacted because SMTC period in TPSS/SSS_sync and T SSB_measurement_period can be explicitly indicated by NW.

Furthermore, according to RAN2 LS [5] on the new SMTC for L3 measurement based on the adaptation SSB, we can conclude that:

Agreements on SSB adaptation

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

Proposal 1: Requirements on L3 measurements based on the adaptation SSB can reuse the current L3 RRM requirements framework in TS38.133 by replacing SMTC configuration with the additional one for the adaptation SSB.

For an examples, the L3 measurement requirements for SSB adaptation can be:

Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) DRX cycle; TPSS/SSS_sync_intra. No DRX; max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1,5 x CSSFintra. DRX cycle≤ 320ms; max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(5 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 5: In case of measurement with SSB adaptation, the different SMTC period configuration will be used..

  - SCell activation requirement for SSB adaptation

In the last meeting, for the SCell activation requirement for SSB adaptation was agreed as [4]:

Agreement: For SCell activation: - Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - For the case SSB adaptation command comes together with SCell activation command. The requirement can be updated based on further progress in other WG

Figure 1a. SCell activation when UE receiving SCell activation command earlier than SSB adaptation

Figure 1b. SCell activation when UE receiving SCell activation command later than SSB adaptation

However, as illustrated in Figure 1a above, no matter whether SSB adaptation is triggered simultaneously with SCell activation command, if the processing time for SSB adaptation command (MAC) is larger than that for SCell activation command, UE receiving time for adaptation SSB command can be after that for SCell activation command. As a result, the start point of SCell activation delay defined TS38.133 can be not reused. That is the current SCell activation delay requirements in TS38.133 can be reused for the SCell activation procedure with the adaptation SSB under some conditions.

8.3 SCell Activation and Deactivation Delay 8.3.1 Introduction This clause defines requirements for the delay within which the UE shall be able to activate at least one deactivated SCell and deactivate at least one activated SCell in EN-DC, or in standalone NR carrier aggregation, or in NE-DC, or in NR-DC. The requirements shall apply for EN-DC, standalone NR carrier aggregation, NE-DC, and NR-DC. 8.3.2 SCell Activation Delay Requirement for Deactivated SCell The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated. The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions. Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

Therefore, we can propose that:

Proposal 2: Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - The start point can be the slot UE receiving SCell activation command if UE receiving SSB adaptation command (MAC) is not later than it

- Conclusion

In this contribution, serval issues related to RRM in Rel19 NES are discussed. The observations and proposals can be summarized as:

Observation 1: In current spec, the total L3 measurement delay requirements depend on the SMTC periodicity which is configured by NW RRC message can be reused for L1/L3 measurement.

Observation 2: UE’s measurement requirements can be little impacted because SMTC period in TPSS/SSS_sync and T SSB_measurement_period can be explicitly indicated by NW.

Proposal 1: Requirements on L3 measurements based on the adaptation SSB can reuse the current L3 RRM requirements framework in TS38.133 by replacing SMTC configuration with the additional one for the adaptation SSB.

Proposal 2: Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - The start point can be the slot UE receiving SCell activation command if UE receiving SSB adaptation command (MAC) is not later than it

- References

- RP-240830

- TS38.133 v18.6.0

- TS38.331 v17.1.0

- R4-2416860

- R2-2501484