3GPP TSG-RAN WG4 Meeting #114bis				                                         R4-2503329

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.7.3

Title: 	Discussion on LB-CA RRM requirements

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP#105, a new WI was agreed to enable LB-LB CA with swicthing in [1]. And the detailed WID is self-contained here for reference:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In the last RAN4 meeting, the discussion on RRM impacts due to LBCA operation was initiated and some agreements and WF was achieved [2]. In this contribution, we provided further analyze on the open issues below.

- Assumptions for R19 LB CA via switching

- RRM impacts:

  - Interruption requirement (if needed) for carrier switching between FDD CC and SDL CC

  - SDL SCell related requirement.

- Discussion

  - Assumptions for R19 LB CA via switching

In the last meeting, the following agreements/WF were achieved.

Issue 1-1-2: Assumptions for R19 LB CA via switching - (1) FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Agreement: - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting.

In WID, the scenarios to be studied was highlighted below:

Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers

For UE which is capable for the SSB-less SCell measurement, the co-located synchronized carriers deployments and same TAG are sufficient conditions for SSB-less SCell. That is the SDL operation can be taken under SSB-less. But for UE which is not capable for SSB-less SCell measurement, the SSB-based measurements are still needed even these SCells are co-located and within a same TAG.

Observation 1: SSB-less SCell operation for SDL needs UE to support SSB-less capability.

Thus we can propose that:

Proposal 1：RAN4 shall discuss and define requirement for SDL SCell operation for both SSB-based and SSB-less.

  - Interruption requirement for carrier switching between FDD CC and SDL CC

In the last meeting, the following agreements/WF were achieved.

- Interruption requirement - Alt 1: Needs interruption requirement for carrier switching between FDD CC and SDL CC (Apple, Xiaomi, CATT, HW, Samsung) - Capabilities for UEs with and without interruption (Samsung) - Alt2: RRM to reuse the RF defined switch delay and time mask. RAN4 RRM not to define additional interruption or scheduling restriction in RRM - Session Chair (RRM session): check the progress in RF session, and discuss if any aspects are not covered by the RF discussion.

Based on the agreed WID, it’s agreed to focus on the scope as following:

- Focus on FDD-SDL band with semi-static switching pattern

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

  - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier

  - Focus on collocated scenario with single TAG and aligned SCS 15kHz only

RAN4 target to specify time mask and required switching delay for the transition between two carriers as case 1 to case 2 or vice versa w/o simultaneous Tx/Rx between the PCell and the SCell.

Figure 1. LB-CA scenarios

For the required time on RF returning among two carriers and location of switching period, existing switching period are specified for UL Tx switching can be reused as starting point. In Rel-16 UL Tx switching between two carriers for inter-band UL CA, the required switching period is specified as UE capability with candidate values {35us, 140us, 210us} to accommodate different UE implementation.

During the transient period, RF returning will interrupt both downlink reception and uplink transmission across two carriers which corresponding to RAN4 uplink transmission time mask requirements in RF specification TS 38.101-1 and DL interruption requirements specified in RRM specification TS 38.133.

Observation 2: The switching period X is pending on UE RF session decision.

That is, the exact switching delay and time mask will be discussed in RF session. But the interruption due to such RF operations (e.g. antenna switching) shall be defined in RRM parts.

Proposal 2: Interruption requirements can be specified in TS38.133 separately based on RF conclusion on the carrier switching delay.

For the detailed DL interruption requirements, we can leverage the work from Rel-16 UL Tx switching as much as possible meanwhile some differences are highlight below:

- On FDD-SDL band combination, no uplink transmission on SDL carrier: the switching happed between carrier 1 (Pcell, FDD carrier Tx/Rx) and carrier 2 (Scell, SDL carrier RX only)

- No con-current transmission and reception between these 2 carriers (for UL Tx switching, simultaneous reception and/or transmission across two carriers allowed pending on band combinations)

- Switching scheme: In Rel-19 only focus on semi-static switching pattern based on RRC configuration

For the analysis simplicity, here we take carrier 1 as FDD carrier (Pcell) and carrier 2 as SDL carrier (Scell). Depending on switching time location on carrier 1 or carrier 2, the corresponding impact to time mask and DL interruption requirements is different (the analysis on time mask impact in companion contribution [2]). The expected interruption on two carriers as described in below figures:

Figure 2. LB-CA antenna switching

Refer to Rel-16 UL transmission discussion, the interruption length is specified following:

- Consider the interruption uncertainty due to MRTD and TA adjustment accuracy, where MRTD=3us.

- Interruption length is expressed by: ceil(switching period+2*TA adjustment uncertainty+2*MRTD-CP length)/symbol duration)+1

- 1 additional symbol was added due to timing offset between UL carrier and DL carrier

As explained above, in LB (FDD) -LB CA(SDL) via switching scenario, only Pcell has UL carrier with single TAG and co-located assumption.

Then above Rel-16 UL transmission DL interruption equation can be further simplified as following:

Proposal 3: Interruption length of LB(FDD)-LB(SDL) via switching can be specified based on following equation

- Interruption length of LB(FDD)-LB(SDL) via switching = ceil((switching period+TA adjustment uncertainty+ MRTD-CP length)/symbol duration)+1

- TA adjustment uncertainty = 130ns, for collocated scenario MRTD = 3us

Proposal 4: The DL interruption starts from the first OFDM symbol which fully or partially overlap with the UL switching period.

Assuming the switching period same as Rel-16 UL Tx switching [3], then expected interruption as shown in below table for example.

DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching between two carriers (LB (FDD) – LB(SDL))

|  | NR Slot length (ms) | Carrier switching period |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35µs | 140µs | 210µs |
| 0 | 1 | 2 | 3 | 4 |

  - SDL SCell related requirement

In the last RAN4 meeting , the following agreements on SDL SCell related requirements were achieved.

Agreement: - FFS: Whether and how - RAN4 to define a new SDL SCell activation/deactivation delay and related interruption for R19 LB CA - the existing SCell activation/deactivation delay and related interruption requirement can be used as baseline for R19 LB CA - FFS: Whether and how - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA

Regarding SDL SCell activation, there are some difference between legacy SCell activation procedure and the one for LBCA,e.g.

- If the antenna switching happened before the Scell activation, the antenna switching time needs to be added into the total SCell activation delay.

- The time for frequency and timing tracking can be skipped because of co-located deployments

- for SDL after antenna switching, the periodicity of RS will be different

- RF warming up time can be further reduced

Figure 3. SCell activation procedure for LB-CA

Observation 3: For SCell activation with LB-CA, UE’s behavior can be different with these for the legacy CA in NR.

Therefore, regarding to RAN4 requirements below, the further SCell activation delay requirements in case of LBCA can be considered.

Proposal 5: RAN4 shall define a new SDL SCell activation/deactivation delay requirements for R19 LB CA.

- Conclusion

In this contribution, serval issues related to RRM in Rel19 LB-CA are discussed. The observations and proposals can be summarized as:

Observation 1: SSB-less SCell operation on SDL needs UE to support SSB-less capability.

Proposal 1：RAN4 shall discuss and define requirement for SDL SCell operation for both SSB-based and SSB-less.

Observation 2: The switching period X is pending on UE RF session decision.

Proposal 2: Interruption requirements can be specified in TS38.133 separately based on RF conclusion on the carrier switching delay.

Proposal 3: Interruption length of LB(FDD)-LB(SDL) via switching can be specified based on following equation

- Interruption length of LB(FDD)-LB(SDL) via switching = ceil((switching period+TA adjustment uncertainty+ MRTD-CP length)/symbol duration)+1

- TA adjustment uncertainty = 130ns, for collocated scenario MRTD = 3us

Proposal 4: The DL interruption starts from the first OFDM symbol which fully or partially overlap with the UL switching period.

Observation 3: For SCell activation with LB-CA, UE’s behavior can be different with these for the legacy CA in NR.

Proposal 5: RAN4 shall define a new SDL SCell activation/deactivation delay requirements for R19 LB CA.

- References

- RP-240830

- R4-2502588

- TS38.133 v18.7.0

- TS38.331 v17.1.0