3GPP TSG-RAN WG4 Meeting #114bis			                                R4-2503332

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.14.2.2

Source:	Xiaomi

Title:			Discussion on OTA test for FR1 NTN devices

Document for:	Discussion

- Introduction

The test methodology and radiated performance metric for FR1 NTN devices has been extensively discussed in previous meetings. The latest progress was captured in the WF [1]. This contribution continues to provide our views on these remaining issues.

### Sub-topic 3-1 UE type, usage scenarios and performance metric for NTN (NR-NTN and IoT-NTN) Issue 3-1-1: UE manufacturer declaration of supported usage scenario for different NTN UE Agreements: - OEM declaration of scenario applicability can be used to declare the supported usage scenario. - Details on declaration and spec language can be further discussed. Issue 3-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2 Agreements: - RAN4 further discuss the performance metric for UE declared only support scenario 2. - Details on performance metric data processing should also be discussed.

- Discussion

Performance metric for scenario 2

For those UEs supporting scenario 2 only, from our perspective, in GEO scenario, the user is more likely to hold the UE in a certain position to guarantee the UL coverage and antenna efficiency due to the UL link-budget, it is reasonable to consider the directional antenna design in real implementation to concentrate energy in a certain direction. RAN4 should consider the partial sphere metrics within a certain angular range which is declared by OEMs for both IoT-NTN and NR-NTN UEs supporting Scenario 2 only. And in [2], two options were proposed as the performance metric, and our preference is option 2, e.g. Outage value corresponding to the Nth percentile of the CDF of EIRP/EIS over the specified range of angles.

Proposal 1: RAN4 to consider defining the outage value corresponding to the Nth percentile of the CDF of EIRP/EIS over the declared range of angles as the performance metric for IoT-NTN and NR-NTN UEs supporting Scenario 2 only.

3	Conclusions

In this paper, we provided our views on the remaining issues on test methodology and radiated performance metric for FR1 NTN devices and our proposal as follows:

Proposal 1: RAN4 to consider defining the outage value corresponding to the Nth percentile of the CDF of EIRP/EIS over the declared range of angles as the performance metric for IoT-NTN and NR-NTN UEs supporting Scenario 2 only.

4 References

[1] R4-2419808, WF for [113][325] TRP_TRS_Ph3, Vivo

[2] R4-2500323, On NTN TRP TRS test methodology, Apple