# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503335

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.11.4

Source: 	Xiaomi

Title: 	RF requirements for Ku band(s) VSAT

Document for:	Discussion

1		Introduction

In RAN#107, WF on check point for Ku bands were agreed that:

- Two sets of bands with FR1 numerologies or FR2 numerologies confirmed based on the response LS from RAN1/RAN2

- In additional to 30kHz SCS, 15kHz SCS was added into WID which target for GSO and low mobility scenario

The update WID with major changes as following:

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only. ### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. The objective of the performance part is to update as appropriate for Ku band: - RRM performance requirements and test cases - UE demodulation and CSI reporting requirements - For 15 kHz subcarrier spacing, the requirements are only applicable for GSO and low mobility scenarios. - Satellite access node demodulation requirements - Satellite access node conformance tests

In this contribution, we discussed VSAT type and VSAT RF requirements.

2		Discussion

- Issue 1-1-1: VSAT types to be considered for NTN Ku band after RAN#106 update

In last RAN4 meeting, following agreements reached for VSAT Types:

- Issue 1-1-1: VSAT types to be considered for NTN Ku band after RAN#106 update - VSAT type 1-3 will not be specified for Ku bands based on the revised WID - The mobile VSAT support of the Ku bands will be added based on the existing mobile VSAT type 4 & 5 - The following changes can be the starting point NTN VSAT class; NTN VSAT type; Type description. Mobile VSAT; 43; Mobile VSAT communicating with GSO and LEO with mechanical steering antenna.. 52,3; Mobile VSAT communicating with GSO and LEO with electronic steering antenna.. NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. Note 3: LEO support is only defined for Ku bands (TBD).

According to the agreed WID, RAN4 target to leverage existing VSAT requirements and VSAT type definition as much as possible meanwhile RAN-P also agreed to introduce mobile VSAT supporting both NGSO scenario and GSO scenario based on existing Type 5 requirements. In our view, VSAT types were introduced based on regulation difference, and RF requirements dependency. RAN4 shall strive to harmonize device types to minimize specification work, and market fragmentation.

From UE capability perspective, separate UE capability IEs are introduced for VSAT on supporting satellite types, mobility (fixed or mobile) and antenna type (electronic or mechanical steering antenna).

From RF requirements perspective, the major difference between mobile VSAT supporting both NGSO and GSO and supporting NGSO only are Minimum Peak EIRP and EIS.  The requirements for device type supporting NGSO only are more relaxed that UE supporting GSO.

Observation 1: From RAN4 UE RF requirements perspective, only Minimum Peak EIRP and EIS requirements are depending on VSAT capability on supporting satellite types.  The requirements for device type supporting NGSO only are more relaxed than UE supporting both GSO only or both.

Based on above observations, we didn’t see any RF requirements differentiation required on extending Mobile VSAT type 4 and type 5 to supporting LEO in additional to GSO.

Observations 2: From UE RF requirements perspective, no differentiation required on extending Mobile VSAT type 4 and type 5 to supporting LEO in additional to GSO.

Proposal 1: No need to restrict LEO supporting only for Ku bands for mobile VSAT type 4/5.

- Issue 1-1-4: General update to support NTN VSAT in FR1-NTN

- Proposals: - The content in the draft CR R4-2501358 (Ericsson) to prepare TS 38.101-5 to support NTN VSAT in FR1-NTN can be considered

In last RAN4 meeting, companies also discussed how to incorporate NTN VSAT into FR1-NTN for RF requirements. In existing TS 38.101-5, radiated requirements are applicable for FR2-NTN bands and conductive requirements are applicable for FR1-NTN bands. Separate Ku bands set with FR1-numerologies or FR2 numerologies will be specified under FR1-NTN or FR2-NTN.  On the other hands, for VSAT only supporting radiated requirements regardless FR1-NTN bands or FR2-NTN bands.

Regarding requirements set applicability rules, we suggest to update requirements appliable rules based on device types instead of operating frequency ranges.

Table 4.2-1: Requirement set applicability

| Requirement | Requirement set |  |
| --- | --- | --- |
|  | FR1-NTN bands | FR2-NTN bands |
| Conducted transmitter characteristics | Clause 6 | NA |
| Conducted receiver characteristics | Clause 7 | NA |
| Conducted performance requirements | Clause 8 | NA |
| Radiated transmitter characteristics | NA | Clause 9 |
| Radiated receiver characteristics | NA | Clause 10 |
| Demodulation performance requirements (Radiated requirements) | NA | Clause 11 |

Proposal 2: Update requirement applicable rules (conductive vs radiated) for NTN terminal based on device types instead of operating frequency ranges.

- Issue 1-1-5: General RF requirements impact on numerologies dependency

- Only radiated requirements shall be updated with the new channel bandwidths to be supported. - It is confirmed the requirements in section 9.2 Transmitted power, 9.4 Transmitter signal quality, 10.8 Receiver antenna off-axis are numerology independent. - The requirements in section 9.5 Output RF spectrum emission, 10.3 OTA reference sensitivity level, 10.4 Maximum input level, 10.5 ACS & 10.6 blocking needs to be updated due to the new channel bandwidth at least.

9.3 Output power dynamics

Proposal 3: No update needed on 9.3.1 Minimum output power, 9.3.3 transmit off power and 9.3.4 Power control for supporting Ku bands.

Proposal 4:  Update 9.3.2 transmit on/off time mask based on FR1-NTN and FR2-NTN.

- FR1-NTN Ku band(s) reusing FR1 requirements

- FR2-NTN Ku band(s) reusing FR2 requirements

9.4 Transmit signal quality

Proposal 5: No update on 9.4 Transmitter signal quality

9.5 Output RF spectrum emission & 10.5 ACS

ACLR and ACS requirements are subject to co-existence study conclusion.

10.3 OTA reference sensitivity level

For OTA reference sensitivity requirement, we can divide as FR1-NTN and FR2-NTN with different reference EIS as EISREFSENS_50MHz and EISREFSENS_10MHz

Proposal 6: Introduce EISREFSENS_50MHz and EISREFSENS_10MHz for FR2-NTN Ku bands and FR1-NTN Ku bands.

3		Conclusion

In this contribution, views on RF requirements for Ku band were provided.

Observation 1: From RAN4 UE RF requirements perspective, only Minimum Peak EIRP and EIS requirements are depending on VSAT capability on supporting satellite types.  The requirements for device type supporting NGSO only are more relaxed than UE supporting both GSO only or both.

Observations 2: From UE RF requirements perspective, no differentiation required on extending Mobile VSAT type 4 and type 5 to supporting LEO in additional to GSO.

Proposal 1: No need to restrict LEO supporting only for Ku bands for mobile VSAT type 4/5.

Proposal 2: Update requirement applicable rules (conductive vs radiated) for NTN terminal based on device types instead of operating frequency ranges.

Proposal 3: No update needed on 9.3.1 Minimum output power, 9.3.3 transmit off power and 9.3.4 Power control for supporting Ku bands.

Proposal 4:  Update 9.3.2 transmit on/off time mask based on FR1-NTN and FR2-NTN.

- FR1-NTN Ku band(s) reusing FR1 requirements

- FR2-NTN Ku band(s) reusing FR2 requirements

Proposal 5: No update on 9.4 Transmitter signal quality

Proposal 6: Introduce EISREFSENS_50MHz and EISREFSENS_10MHz for FR2-NTN Ku bands and FR1-NTN Ku bands.

Reference

[1] RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

[2] R4-2502288, Way Forward for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL