# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503341

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	On RRM requirements for UE-to-UE CLI handling

Source: 	OPPO

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

In last meeting, a WF on RRM requirements for NR_duplex_evo was approved [1].

In this contribution, we will provide our views on RRM requirements for UE-to-UE CLI handling.

Discussion

- Issue 1-1-3: Measurement resources - Agreements - Define measurement requirements assuming wideband reporting for L1-SRS-RSRP and L1-CLI-RSSI, and do not need to consider subband reporting. FFS if UE is able to measure both DL subbands simultaneously in case of DUD and how such measurement capability affects measurements period and accuracy requirements. - RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement for semi-static, aperiodic and periodic resource types. - Agreements - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion. - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion. - Option 3: consider the UE measurement capability

Firstly, RAN4 needs to continue the discussion on whether UE is able to measure both DL subbands simultaneously in case of DUD and how such measurement capability affects measurements period and accuracy requirements.

For CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands), we think the measurement performance including measurement delay and accuracy should consider different UE implementation. For the worst case, UE is not able to measure wideband RSSI from 2 DL subbands from a single resource occasion, but measure one DL sub-band on each CSI-RS resource occasion in TDM manner, considering IMD and other negative impact due to intra-band non-contiguous simultaneous reception.

Proposal 1: RAN4 to define the minimum requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement assuming UE cannot measure both DL sub-bands simultaneously. Further discuss additional UE capability as optional.

#### Issue 1-1-5: Scheduling restriction - Agreements (from Thu AH session) - For L1-SRS-RSRP, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restriction does not apply to the symbols with SRS if UE is able to support FDM-ed DL reception and SRS measurement if RAN1 will define optional UE capability for FDMed DL reception and SRS measurement; otherwise (if UE does not support the capability or RAN1 does not define the capability), scheduling restriction applies. - The restricted symbols at least include symbols for measurement resource and Y symbols before symbols for measurement resource and Y is re-using same number as R16 requirements, - Note: this time has accounted for the UL/DL switching time. - FFS whether there is restriction on symbols after the symbols for measurement resource considering UL/DL transition. - For L1-CLI-RSSI Methods #1, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restrictions do not apply except when CLI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2. - The restricted symbols at least include symbols for measurement resource, FFS whether restriction on additional symbols is needed. - For L1-CLI-RSSI Method #3, scheduling restriction requirements are FFS

For L1 UE-to-UE CLI measurement and reporting, 3 Methods are supported:

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- Method #3: UE measures RSSI within UL subband

For L1-SRS-RSRP and L1-CLI-RSSI Method #1, RAN4 has almost achieved agreements. The remaining open issues is about whether to consider additional restricted symbols after measurement resources.

In our view, it is similar to the case before the measurement resources because of UL/DL transition.

Proposal 2: For L1-SRS-RSRP and L1-CLI-RSSI Method #1, consider the additional restricted symbols after the symbols for measurement resource.

For L1-SRS-RSRP and L1-CLI-RSSI Method #3, we are fine that R16 requirements are re-used as baseline for both FR1 and FR2, and exact number of restricted symbols is FFS. And also the worst cases should be considered, and the number is also suggested to align with other scenarios.

Proposal 3: For L1-CLI-RSSI Method #3, reuse R16 scheduling restriction requirements as baseline.

Conclusions

In this paper we provided our views on RRM requirements for UE CLI measurement in SBFD.

Proposal 1: RAN4 to define the minimum requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement assuming UE cannot measure both DL sub-bands simultaneously. Further discuss additional UE capability as optional.

Proposal 2: For L1-SRS-RSRP and L1-CLI-RSSI Method #1, consider the additional restricted symbols after the symbols for measurement resource.

Proposal 3: For L1-CLI-RSSI Method #3, reuse R16 scheduling restriction requirements as baseline.

Reference

- R4-2502699, WF on RRM requirements for NR_duplex_evo