# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503346

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	On event triggered L1 measurement reporting

Source: 	OPPO

Agenda item:	7.27.2.1

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility [1].

In this contribution, we provide our views on remaining issues of measurements related enhancements for LTM, including event trigger L1 measurement reporting.

Discussion

Event triggered L1 measurement reporting

RAN4 agreed in last meeting that event triggered L1 reporting starts from the time when a condition exists at the UE which will trigger the reporting, and ends at the point when the UE starts to transmit the first UL transmission to report measurement result over the air interface.  One leftover issue is that when defining upper bound of the event triggered measurement reporting delay, “the corresponding Cell(s)” of the L1-RSRP measurement period should be the specific cell triggers the event or all related cells corresponding to the event.

- Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - Except for periodic MR: - Option 1: The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - Option 2: The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - For periodic MR: - Option 1: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR. - Option 2: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR. <Recommended WF> - Continue discussion.

In our view, the safe solution to UE can be “the maximum L1-RSRP measurement period on the cells corresponding to the event”. But since RAN4 did not clarify this issue for event triggered L3 reporting from R15, it does not broke RRM requirements and related implementation of both UE and network. So we think the ambiguity here seems not a big issue. Either option is ok to UE. If we need to clarify this, the better way could be to solve it for both event triggered L3 reporting and event triggered L1 reporting.

Observation 1: Without clarification on “the corresponding Cell(s)” of the L1-RSRP measurement period for event triggered L3 reporting, it does not broke RRM requirements and related implementation of both UE and network.

Proposal 1: Prefer to follow the solution of L3 reporting, and can compromise that the event triggered measurement reporting delay shall be no larger than “the maximum L1-RSRP measurement period on the cells corresponding to the event” if RAN4 decides to clarify.

Besides, the measurement period on the LTM candidate cells is also related to Treport. Both event and Treport are configured by network and need to be considered in the reporting delay requirement. We would like to know the reason why to assume Treport = 0. RAN4 needs to clarify it.

Proposal 2: Clarify Treport in L1-RSRP measurement period when defining the event triggered measurement reporting delay

- Issue 2-4: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Yes (Xiaomi, CMCC, CATT, MTK, ZTE, OPPO, CTC, E///) - Option 1a: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO) - Option 1b: Number of supported L1 events is to be added on top of the currently supported L3 events. Details FFS. (Nokia) - Option 2: Open to let RAN1 discuss the number of events supported by UE at first. (MTK) - Option 2a: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo) - Issue 2-5: whether to define a separate capability for supported number of events triggered L1 measurement reporting Candidate options: - Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. ([Xiaomi?], ZTE, CMCC, CTC) - Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///) - Issue 2-6: how to define requirements for reporting criteria per measurement category Candidate options: - Proposal 1: Regarding whether to count LTM event in a per-layer basis, RAN4 shall wait for further RAN1/2 conclusions regarding the detailed resource configuration for event-triggered L1-RSRP reporting in R19 LTM. (vivo)

The three issues above are talking about the same thing that whether to define separate Ecat capability for LTM L1 measurements or share with Ecat capability for L3 measurements.

We support to additionally include L1 reporting criteria per measurement category into current Ecat. UE can share the UE capability between L1 and L3 reporting. And UE is going to use the same value of Ecat for L1+L3 intra-frequency and inter-frequency measurement category, and update the note for each of events description, as the table below for example:

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |

Proposal 3: RAN4 to additionally include SSB and CSI-RS based L1-RSRP for intra-frequency and inter-frequency measurement reporting criteria in the same table, and not change current number of Ecat.

Conclusion

In the contribution we provide the following proposals:

Observation 1: Without clarification on “the corresponding Cell(s)” of the L1-RSRP measurement period for event triggered L3 reporting, it does not broke RRM requirements and related implementation of both UE and network.

Proposal 1: Prefer to follow the solution of L3 reporting, and can compromise that the event triggered measurement reporting delay shall be no larger than “the maximum L1-RSRP measurement period on the cells corresponding to the event” if RAN4 decides to clarify.

Proposal 2: Clarify Treport in L1-RSRP measurement period when defining the event triggered measurement reporting delay

Proposal 3: RAN4 to additionally include SSB and CSI-RS based L1-RSRP for intra-frequency and inter-frequency measurement reporting criteria in the same table, and not change current number of Ecat.

Reference

- R4-2402601, WF on NR mobility enhancements Phase 4_part1