# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503347

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	On CSI-RS based L1 measurements

Source: 	OPPO

Agenda item:	7.27.2.2

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility [1].

In this contribution, we provide our views on remaining issues of CSI-RS based L1 measurements.

Discussion

- Issue 4-1: Known cell definition in CSI-RS based L1 measurement Issue 4-1-1: Whether to use detectable or measurable? Tentative agreement: - Use measurable and specify the side condition for RS being measurable. Issue 4-1-2: Side condition for CSI-RS based L1 measurement for neighbor cell Candidate options: - Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (Nokia) - Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Xiaomi, MTK, OPPO, Huawei, Ericsson)

About the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement, we prefer to follow CSI-RS based L3 measurement. And due to the cell detection also relies on SSB, the CSI-RS from the target cell configured for L1 measurement remains “measurable”, which is more suitable than “detectable”. About the side condition, we prefer to reuse the legacy SINR=-3dB. It is not reasonable to relax the condition for CSI-RS.

Proposal 1: Reuse the legacy side condition of SINR=-3dB for CSI-RS based L1 measurement for neighbor cell.

- Issue 4-2: Measurement procedure - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.] The following is concluded in RAN4#113 meeting. Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 - Option A: Whether step 2 can be skipped or not can be based on UE capability reporting. (Apple, CATT, Nokia, Ericsson, Qualcomm, CMCC - compromise, ZTE, China Telecom, vivo, Xiaomi, OPPO) - Option B: Always keep step 2 for FR2-1 (MTK, HW) - Option C: Remove L1 measurement for FR2-1 in the 2nd bullet. (CMCC, OPPO)

About measurement procedure for CSI-RS based LTM candidate cell measurements in FR2-1, We prefer to remove step 2 to reduce the total measurement delay. But it does not mean SSB-based L1 measurements and reports on the same candidate cell are not allowed, because it depends on UE implementation. However, optional UE capability can also be an acceptable compromise to us.

Proposal 2: Prefer to skip step 2 for FR2 CSI-RS based LTM measurement, and also can compromise to introduce optional UE capability reporting.

- Combined issues: Measurement type and RTD assumption - Option 1: CMCC, ZTE, CATT, China Telecom - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - Define requirements for RTD > CP, as optional UE feature. FFS details - Option 2: E///, HW, CATT, Apple - compromise, CMCC - compromise - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - No Ran4 requirements for RTD > CP. - Option 3: - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - Define requirements for support RTD > CP, as optional UE feature. FFS details - Option 4: QC, Apple, vivo, Xiaomi, Samsung, MTK, OPPO, Nokia - compromise, E/// - compromise - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - No Ran4 requirements for RTD > CP.

We also discuss whether to deprioritize some scenarios as list above in last meeting. As highest priority, we prefer option 4 not to Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, and no Ran4 requirements for RTD > CP. Because SSB based LTM requirements have been already defined in R18, and most UE and network vendor will complete them if needed. Now CSI-RS based LTM is going to be a supplementary but not an alternative. The similar situation happens for R15 SSB based L3 measurements and R16 CSI-RS based L3 measurement. To be realistic, we prefer the scope or requirements of additional CSI-RS based LTM can be controllable.

Observation 1: To be realistic, the scope for requirements of additional CSI-RS based LTM should be controllable.

Proposal 3: Prefer not to Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, and no Ran4 requirements for RTD > CP.

Conclusion

In the contribution we provide the following proposals:

Proposal 1: Reuse the legacy side condition of SINR=-3dB for CSI-RS based L1 measurement for neighbor cell.

Proposal 2: Prefer to skip step 2 for FR2 CSI-RS based LTM measurement, and also can compromise to introduce optional UE capability reporting.

Observation 1: To be realistic, the scope for requirements of additional CSI-RS based LTM should be controllable.

Proposal 3: Prefer not to Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, and no Ran4 requirements for RTD > CP.

Reference

- R4-2402601, WF on NR mobility enhancements Phase 4_part1