# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503350

# Wuhan, China, 7th – 11st April, 2025

Agenda Item:	7.21.1

Source: 	OPPO

Title: 	Discussion on general aspects for AI mobility

Document for:	Approval

- Introduction

In last meeting, the general aspects for AI mobility were discussed and the agreements were captured in [1]. In this contribution, we will further discuss the open issues captured in [2] and give our proposals. In addition, considering we only have two meetings left for this SI, we will discuss the framework of RAN4 part in TR 38.744 and give some text proposals to capture the RAN4 agreements.

- Discussion

Scenarios and use cases

According to the previous RAN4 agreements and TR 38.744, scenario 2, 3, 4 are prioritized in this study phase.

Issue 1-1-2: Scenario/sub-use-case priorities Agreement: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. The prioritization among evaluation scenarios is captured in table 5.2.1-1 Table 5.2.1-1: Prioritization of evaluation scenarios scenario number; Priority; Evaluation scenario; Target study goal; Methodology. 1; Low; FR1 to FR1 intra-frequency temporal domain case A; 2nd goal; TBD. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. 5; Low; FR2 to FR2 intra-frequency temporal domain case B; 1st goal; TBD. 6; Middle; FR2 to FR2 intra-frequency spatial domain; 1st goal; Intra-cell.

But in last meeting, there are still proposals to discuss the other scenarios:

#### Issue 1-1-3: Scenario priorities - Proposals and Observations: - Proposal 1 (Huawei): RAN4 to start the study of other scenarios than scenarios 2,3,4 if RAN2 achieves sufficient progress. - Proposal 2 (vivo): RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case. For scenario 6, the study can be considered after enough progress is made in AI/ML beam management. - Proposal 3 (Apple): RAN4 should evaluate the combined benefits of RRM measurement predictions across multiple domains (such as spatial beam/cell, frequency, and temporal) and dimensions (across various cells and frequencies). Additionally, a working plan should be established to investigate scenarios that maximize these potential advantages. - Proposal 4 (Samsung): RAN4 to deprioritize studying the impacts on requirements for beam level spatial domain prediction (i.e., FR2 to FR2 intra-frequency spatial domain scenario 6) in AI mobility

Considering RAN2 only evaluated the scenario 2, 3, 4 and there are only two meetings left for this SI, we think we can focus on the three scenarios in this release and deprioritize other scenarios. And since this is the first release of AI for mobility, we prefer to consider the basic approach, i.e., consider the single domain prediction and deprioritize the combined cases.

Proposal 1: RAN4 to only study scenarios 2,3,4 in this release.

Proposal 2: RAN4 not to consider the combined prediction across multiple domains in this release.

#### Issue 1-1-4: Sub-use-case priorities - RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model) At RAN2#125bis, the following sub-use cases were agreed for RRM measurement prediction: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Proposals and Observations: - Proposal 1 (Huawei): RAN4 to consider all three sub-use cases until RAN2 gives the priority. - Proposal 2 (vivo): Before performing enough evaluation and reaching an aligned conclusion prediction performance in terms of three sub-use cases, RAN4 not to have any deprioritization given to any sub-use cases. - Proposal 3 (CMCC): it is proposed to consider all three sub-use cases. - Proposal 4 (QC): RAN4 to consider all three sub-use cases, with higher prioritization given to sub-use case 2 and 3. - FFS on whether and how to differentiate Sub-use case 2 and 3, e.g. one of or both sub-use case can be considered in defining RRM core and performance requirements. - Proposal 5 (Ericsson): RAN4 to prioritize sub case 2, where cell level measurements are used for the cell level RRM measurement prediction, for the requirement discussion for the temporal prediction cases. - Proposal 6 (Samsung): RAN4 to consider the Sub-use-case 2 as a baseline for the study of scenarios 2, 3, and 4 for cell-level prediction

Regarding the sub-use case, since all the three cases are evaluated in RAN2 and no prioritization is defined, we think there is no need to deprioritize any case in RAN4.

Proposal 3: RAN4 to consider all three sub-use cases with no prioritization.

TR framework

Since only two meetings are left for this SI, it is time to consider the framework of TR 38.744 to capture the RAN4 agreements. In the existing Draft TR [3], the following section is reserved for RAN4 requirements and testability aspects.

## 6.2 Interoperability, testability, and RRM requirements Editor Note: This section intends to capture the spec impact on testability, interoperability, and RRM requirements and performance [RAN4] based on SID.

Based on the existing RAN4 discussions and agreements, we think the following aspects can be captured in the TR:

- Scenarios and use cases

- RRM requirements impact for measurement prediction

  - Core requirements

  - Performance requirements

- RRM requirements impact for measurement event prediction

  - Core requirements

  - Performance requirements

- Testability and Interoperability

  - Testing setup for FR1 and FR2 (including prediction consistency in time domain, cell number, channel model and prediction specific parameters etc.)

  - Testing principle (including FR2 aspects, inter-carrier aspects and ground truth derivation)

  - Testing goal

  - Generalization

According to the above aspects, we suggest adopting the text proposals for TR 38.744 in [4] and some of the contents still depend on further discussion and consensus.

Proposal 4: Adopt the text proposal for TR 38.744 in [4] to capture the RAN4 study.

- Conclusions

In this contribution, we discussed the general aspects for AI mobility and got the following observations and proposals:

Proposal 1: RAN4 to only study scenarios 2,3,4 in this release.

Proposal 2: RAN4 not to consider the combined prediction across multiple domains in this release.

Proposal 3: RAN4 to consider all three sub-use cases with no prioritization.

Proposal 4: Adopt the text proposal for TR 38.744 in [4] to capture the RAN4 study.

- References

R4-2502594 WF on FS_NR_AIML_Mob

R4-2500533 Topic summary for [114][219] FS_NR_AIML_Mob

R2-2500287 Text proposal on TR 38.744

R4-2503645 TP on RAN4 aspects for TR 38.744