# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503351

# Wuhan, China, 7th – 11st April, 2025

Agenda Item:	7.21.2.1

Source: 	OPPO

Title: 	Study of impacts on RAN4 requirements for RRM measurement prediction

Document for:	Approval

- Introduction

In RAN4#114 meeting, the RRM requirements for AI mobility were discussed. The agreements and open issues were captured in the WF [1] and topic summary [2] respectively.

In this contribution, we will further evaluate the impacts on RRM requirements for measurement prediction based on RAN2 progress and current RAN4 discussions, and some background information derived from RAN2 SI is provided in Annex.

- Discussion

Performance Metrics/KPIs for Measurement Prediction use case

In RAN4#112bis meeting, the following agreements were reached:

Issue 1-1-2: Scenario/sub-use-case priorities Agreement: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. Agreement: - Scenario 2, 3 and 4, the 3 cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed. Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

In last meeting, the reference point for defining requirements, i.e., the impacts on beam/cell level measurements, was not discussed due to the limited time. Based on our contribution in [3], from the point view of AI/ML model implementation, each of sub-use cases utilizes L1-level measurements as the origin and aims for L3-level measurement predictions as their destinations, the entire process or a portion thereof being replaced by AI/ML modules or solutions.

For RAN4 requirements and testing, all three sub-use cases should use L3 cell level measurements as target (i.e., Point C in the figure 1), rather than check L1/L3 beam-level results(e.g. point A1 or E in figure 1). Concerning sub-use case 1 and sub-use case 2, some additional parameters(if needed) may be included as part of the test conditions in the form of side conditions (e.g., L3 filter). Regarding the possible different performance at point C due to the three implementation methodologies, RAN4 can evaluate it and decide whether to define separate requirements or unified requirements during WI stage.

Figure 1: L1/L3 measurement framework

Proposal 1: For all three sub-use cases, RAN4 to define prediction accuracy requirements and testing for L3 cell level measurements at point C.

Proposal 2: RAN4 to evaluate and decide whether to define separate or unified requirements for different sub-use cases during WI stage.

Regarding the definition of relative prediction accuracy, the following agreements were reached in RAN4#114 meeting:

Issue 2-1-1a: Relative RSRP Prediction Accuracy Agreement: - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) - FFS: Beam 1 and Beam 2 can be from same cell or different cells. - FFS: whether ground truth can be reported or ideal - FFS: whether the reported RSRP of cell2/beam2 can be predicted or measured or a combination of both predicted and measured samples.

In legacy measurement requirements, as cited below, for intra-frequency measurement, the relative accuracy is defined as the RSRP measured from one cell compared to the RSRP measured from another cell on the same frequency, or between any two RSRP levels measured on the same cell. For inter-frequency measurement, the relative accuracy is defined as the RSRP measured from one cell on a frequency compared to the RSRP measured from another cell on a different frequency.

##### 10.1.2.1.2 Relative SS-RSRP Accuracy The relative SS-RSRP accuracy is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 is configured. ##### 10.1.4.1.2 Relative SS-RSRP Accuracy in FR1 The relative SS-RSRP accuracy in inter-frequency case is defined as the RSRP measured from one cell on a frequency in FR1compared to the RSRP measured from another cell on a different frequency in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasInterFreq-r17 is configured.

Based on the agreements in last meeting, the similar definition is reused for AI based mobility, i.e., the relative accuracy can be defined as the accuracy of RSRP difference between different cells/beams. But we still need to clarify how to handle the beam level report and whether to consider the difference between the reported predicted results and reported measured results. According to the proposal 1 above, we think we only need to define requirements for L3 cell level measurements. And in this release, we prefer not to combine the measurement prediction on time domain and spatial domain. So for the relative accuracy, only the RSRP difference between two cells needs to be considered for both intra-frequency and inter-frequency measurement prediction.

Regarding the difference between predicted results and measured results, since we are discussing prediction accuracy, we prefer not to introduce the impact of legacy measurement, i.e., not to include the measured results in the relative accuracy. In summary, the relative accuracy is defined as below, where the definition of ground truth can refer to the discussion for absolute accuracy.

- Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1– reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1– ground truth of RSRP of cell 2)

Proposal 3: For both intra and inter frequency prediction, the relative accuracy is defined as below, where the definition of ground truth can refer to the definition for absolute accuracy.

- Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1– reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1– ground truth of RSRP of cell 2)

Regarding the definition of absolute prediction accuracy and ground truth, the following agreements were reached in previous meeting:

RAN4#113 meeting: Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2 RAN4#114 meeting: Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2. Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

It was agreed that the absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP and the remaining issue is the definition of ground truth of L3-RSRP. Based on the discussions in last meeting, there are two approaches to define the ground truth, where alt1 is to use the expected measurement results calculated based on transmission power and channel modelling and alt2 is to use the reported measurement results under certain conditions.

  - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

  - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

For alt1, the advantage is that it is easily calculated and TE side can clearly know the expected results at each timing point, but the issue is that TE does not know when the measurement is performed or predicted. Therefore, TE doesn’t know which one to be compared which may request additional report from UE on the time stamp. The other issue not mentioned in last meeting is that the performance requirements may be used in model monitoring in LCM procedure, we understand it may cause problem since UE cannot acquire the expected measurement results in real LCM implementation. In addition, we understand the model input and training data is based on the actual measurement results, thus the output is usually close to the UE measurement.

For alt2, according to the measurement model in figure 1, from inputs at point A, UE needs to experience L1 filter, beam consolidation/selection and L3 filter to get L3 measurement results at point C in which the latter two parts are RRC configured. Based on the legacy procedure, when the SINR is high enough, the UE measurement results are approximate to the ideal values. The advantage is that this can accommodate different UE implementation e.g., L1 filtering, and can be aligned with model training and inference procedure. To define the prediction accuracy and testing, certain channel modelling, beam consolidation configuration and L3 filter configuration can be considered as possible side conditions. The issue for alt2 is that UE may not perform actual measurement on certain occasions in some of the prediction cases, but we think this can be resolved by setting additional procedure in the testing. The other concern is that both ground truth and report are from UE which makes UE possible to cheat in testing. But we understand the actual measurement used as ground truth still needs to meet the legacy requirements, and cheating in the testing is not benefit for UE since it may lead bad performance in real practice. So we do not need to assume that UE will cheat in the testing.

Based on the comparison of the two approaches above, the advantages and disadvantages are summarized as below and we prefer to use alt2 as baseline. And considering the limited time of SI, if we cannot make decision in this meeting, we are also fine to list the issues and possible solutions in the TR and down-select in WI stage.

|  | Advantages | Disadvantages | Possible solutions |
| --- | --- | --- | --- |
| Alt 1 | - Easily to acquire and to be known by TE side | - TE does not know when the measurement is performed or predicted - May not be used in model monitoring in LCM procedure - May not be aligned with model training - Not consider the UE implementation inaccuracy | - UE report on time stamp |
| Alt 2 | - The reported results are approximate to the ideal values under high SINR - Can be used in model monitoring in LCM procedure - Aligned with model training - Accommodate the UE implementation inaccuracy | - Condition setting (e.g., continuous or time varying high SINR) - Acquire UE actual measurement for reduction case - Avoid UE cheating | - Control transmit power to adapt to the channel modeling - Additional procedure under same testing environment to acquire actual measurement |

Proposal 4: Use Alt2(i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy.

Proposal 5: If the definition of ground truth cannot be decided in this meeting, list the advantages/disadvantages in the TR and down-select in WI stage.

Proposal 6: RAN4 to consider certain channel model, beam consolidation configuration and L3 filter configuration as possible side conditions when defining the accuracy requirements and testing.

For intra-frequency measurement prediction, it was agreed to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy.

Issue 2-1-9: Impacts from observation and prediction windows Agreement: - RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions. - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

Based on the agreements/observations reached in TR 38.744, the prediction accuracy for inter-frequency prediction can be impacted by the correlation coefficient between two frequency layers and cluster approach.

For FR1 inter-frequency predictions in co-located scenario, the following observations are made: - The prediction accuracy is comparable between higher-to-lower frequency and lower-to-higher frequency case. - The UE speed has minor impact on the prediction accuracy. - The higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy - The cluster approach can improve the prediction accuracy compared to single cell approach - AI algorithm with cluster approach shows better performance compared to pathloss offset-based algorithm. But AI algorithm with single cell approach achieves limited gain compared to pathloss offset based algorithm without the help of neighbour cell measurement results.

We understand these parameters can be considered as the side conditions when defining the accuracy requirements in RAN4. Regarding whether and how to define different sets of requirements, we can evaluate it during WI stage.

Proposal 7: RAN4 to consider the impact of correlation coefficient between two frequency layers and the cluster approach on inter-frequency measurement prediction accuracy requirements.

Impacts of measurement errors on the prediction accuracy

In RAN4# 113 meeting, 4 options have been proposed for discussion on the impacts of measurement errors

Issue 2-1-4: Impacts of measurement error on prediction accuracy Agreement: For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

In last meeting, it was agreed that the RF and baseband errors on the measurements would impact the prediction accuracy. We understand this impact should be considered in WI stage when defining the accuracy requirements in RAN4 rather than to consider in SI for system simulation. On the other hand, even if the measurement error is considered in the system simulation, no much difference on the accuracy based on our simulation results below:

|  | FR1 case B | FR2 case A |
| --- | --- | --- |
|  | Assumptions: UE speed = 30km/h, MRRT = 50%, OW = 600ms, PW=200ms, sub case 2, non-sliding filtering | Assumptions: UE speed = 60km/h OW = 800ms, PW=400ms, sub case 2, non-sliding filtering, |
| method | Average L3 RSRP difference [dB] | Average L3 RSRP difference [dB] |
| AI without meas. error | 0.26 | 1.21 |
| AI with meas. error | 0.95 | 1.46 |

Observation 1: Even with a relatively large measurement error, for temporal case A and case B, the degradation due to measurement error is not significant.

According to the experience of AI/ML for NR Air Interface, the feasibility study is RAN1/2 work. They do not need to consider the impact of measurement error, and they have their own evaluation approach on the feasibility. For study of AI/ML for NR Air Interface, there are the simulation results with and without measurement error, and different companies have different results, but that does not impact the conclusion on feasibility. Similarly for AI mobility, RAN4 does not need to interpose the feasibility discussion in RAN2 but can consider the impact of measurement error on accuracy requirements in WI.

Proposal 8: RAN4 not to interpose the feasibility discussion in RAN2 but can consider the impact of measurement error on accuracy requirements in RAN4 during WI stage similar as AI/ML for NR Air Interface.

Proposal 9: RAN4 to focus on the possible requirements to be defined and the testability discussion for AI mobility during SI stage.

Impacts on Core requirements

In RAN4#112bis meeting, RAN4 agreed to start the discussion for scenario 2, 3, 4 as listed below:

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Based on the following conclusions in RAN2, UE measurement behaviour can be different from legacy, e.g., UE can skip some measurement based on different MRRT values or OW/PW values for intra-frequency measurement prediction or UE can skip some measurement on certain frequency for inter-frequency measurement prediction.

Post email discussion after RAN2#127 meeting: In case A, continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either one or more sampling periods (with sliding L1/L3 filtering option) or one or more measurement periods (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding. For temporal domain case A, Company can report the observation window and prediction window with following limitation: 1, The ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5} 2, The window length should be multiple times of sampling period or measurement period of corresponding frequency range. In case B, measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) and measurement result(s) in previous prediction window is/are skipped during window sliding To agree following recommended value range for FR1 intra-frequency temporal domain case B: - MRRT: {50%~80%} - OW: {40ms ~ 2000ms} - PW: {40ms~800ms} RAN2#126 meeting: 5 Definition of measurement reduction for intra-frequency scenario is defined as:(20/20) Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: MRRT= skipped measurement time instances / total measurement time instances Measurement reduction rate in spatial domain (MRRS): MRRS = skipped beams to be measured/ total beams to be measured

From RAN4 perspective, the measurement period requirements can be different according to the skipped measurements, e.g., the skipped SMTC occasions may not cause scheduling restrictions and the skipped frequencies may not need to be counted in the CSSF calculation.  The impact of skipped measurement instances and the corresponding measurement period requirements can be discussed in WI stage.

Proposal 10: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A considering the impact of OW or PW length.

Proposal 11: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values.

Proposal 12: RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation.

Proposal 13: RAN4 to investigate the possibility of updates on scheduling restrictions, measurement capabilities or definition of known conditions due to measurement prediction.

- Conclusions

In this contribution, we discussed the possible RAN4 requirements for RRM measurement prediction and got the following observations and proposals:

Proposal 1: For all three sub-use cases, RAN4 to define prediction accuracy requirements and testing for L3 cell level measurements at point C.

Proposal 2: RAN4 to evaluate and decide whether to define separate or unified requirements for different sub-use cases during WI stage.

Proposal 3: For both intra and inter frequency prediction, the relative accuracy is defined as below, where the definition of ground truth can refer to the definition for absolute accuracy.

- Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1– reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1– ground truth of RSRP of cell 2)

Proposal 4: Use Alt2(i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy.

Proposal 5: If the definition of ground truth cannot be decided in this meeting, list the advantages/disadvantages in the TR and down-select in WI stage.

Proposal 6: RAN4 to consider certain channel model, beam consolidation configuration and L3 filter configuration as possible side conditions when defining the accuracy requirements and testing.

Proposal 7: RAN4 to consider the impact of correlation coefficient between two frequency layers and the cluster approach on inter-frequency measurement prediction accuracy requirements.

Observation 1: Even with a relatively large measurement error, for temporal case A and case B, the degradation due to measurement error is not significant.

Proposal 8: RAN4 not to interpose the feasibility discussion in RAN2 but can consider the impact of measurement error on accuracy requirements in RAN4 during WI stage similar as AI/ML for NR Air Interface.

Proposal 9: RAN4 to focus on the possible requirements to be defined and the testability discussion for AI mobility during SI stage.

Proposal 10: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A considering the impact of OW or PW length.

Proposal 11: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values.

Proposal 12: RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation.

Proposal 13: RAN4 to investigate the possibility of updates on scheduling restrictions, measurement capabilities or definition of known conditions due to measurement prediction.

- References

R4-2502594 WF on FS_NR_AIML_Mob

R4-2500533 Topic summary for [114][219] FS_NR_AIML_Mob

R4-2501133 Study of impacts on RAN4 requirements for AI mobility

RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

RP-240082 Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

Annex: Background information

In RAN#102 meeting, the SID on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR is approved. In RAN#105 meeting, the related work plan in RAN4 has been further checked and updated as follows:

Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

Some background information derived from RAN2 SI is as follows:

To evaluate potential benefits and gains of AI/ML aided mobility, following aspects are agreed in SID[1] for study:

- Measurement prediction (including UE sided and NW sided model)

- HO failure/RLF prediction (only UE sided model)

- Measurement events prediction (only UE sided model)

And as clarified in RAN2 on TR 38.744, the study of the use cases is driven mainly by two study goals.

- The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements.

- The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

Regarding the RRM measurement prediction

- 3 sub-use cases are considered for cell-level RRM measurement prediction

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Temporal/frequency/spatial domain measurement prediction

- For temporal domain measurement prediction,

In RAN2#125bis meeting, it has been agreed as:

- For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal predication.

In addition, based on the post email discussion after RAN2#127 meeting, temporal domain case A and case B have been further clarified as:

Intra-frequency temporal domain case A: continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either one or more sampling periods (with sliding L1/L3 filtering option) or one or more measurement periods (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding.

Figure 1

Intra-frequency temporal domain case B: measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) and measurement result(s) in previous prediction window is/are skipped during window sliding

Figure 2-1 temporal domain case example 1

Figure 2-2 temporal domain case example 2

- For frequency domain measurement prediction, typical feature is to use the measurement result from one frequency to predict the measurement result at another frequency, which can be applicable to FR1 to FR1 cases in R19 evaluation.

- For spatial domain measurement prediction, typical feature is to predict cell-level measurements through measurements of partial beams, which can be applicable to sub-use case 1 and 3 in R19.