# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503352

# Wuhan, China, 7th – 11st April, 2025

Agenda Item:	7.21.2.2

Source: 	OPPO

Title: 	Study of impacts on RAN4 requirements for measurement event prediction

Document for:	Approval

- Introduction

In RAN4#114 meeting, the RRM requirements for AI mobility were discussed. It was agreed to start discussion on measurement event prediction from this meeting.

Issue 1-1-1: Measurement event prediction use case Agreement: - RAN4 will start the discussion on measurement event prediction use case in RAN4#114bis meeting.

In this contribution, we will evaluate the impacts on RRM requirements for measurement event prediction based on RAN2 progress, and the agreements derived from RAN2 is provided in Annex for information.

- Discussion

Based on RAN2 discussion and evaluation, intra-frequency temporal case A and case B are considered for measurement event prediction. And the input/output for direct and indirect event prediction are defined as below:

For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. And the final output is the expected occurrence time of a certain measurement event (ex. event A3). The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed. For direct measurement event prediction, the model output is the probability of event occurrence within a time window.

For direct event prediction, the model output is the probability of event occurrence within a time window. We understand how to use this probability to define the event conditions is within RAN2 scope. When the event is triggered, the procedure e.g., handover is almost same as legacy, and from requirements perspective, we think no updates or enhancements are needed. On the other hand, considering there are not much evaluation for direct event prediction, we prefer to deprioritize direct prediction and start from indirect prediction.

Proposal 1: For measurement event prediction, RAN4 to discuss RRM requirements impact starting from indirect prediction case.

In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A or case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model.

For this case, the AI/ML model is used for providing RRM measurement requirements, and after that, legacy procedure can be used to derive measurement event. To verify the AI/ML functionality, we understand the requirements of intermediate output can be defined and the requirements for measurement prediction can be used as baseline.

Proposal 2: For indirect prediction, the requirements for intermediate output are needed and the requirements for measurement prediction can be used as baseline.

Regarding the event conditions and handover procedure, according to the following objective for case A and case B, the difference with legacy handover is the earlier handover preparation upon A3 event prediction. But the execution after handover command, e.g., cell searcher, fine time tracking and SSB processing are still needed and same as legacy.

Objective in Case A (FR2): Predict A3 events to enhance HO execution via earlier HO Prep upon A3 event prediction (as shown in Option 2 or Option 3), i.e., HO performance enhancement. Objective in Case B (FR1): Predict A3 events in the presence of skipped measurements, i.e., overhead reduction while maintaining similar HO performance.

Taking the existing handover delay requirements as baseline, the components are not reduced or enhanced by event prediction. And RAN4 to discuss whether the existing requirements can be reused.

##### 6.1.1.2.1 Handover delay When the UE receives a RRC message implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover msec from the end of the last TTI containing the RRC command. Where: Dhandover equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.1.2.2. ##### 6.1.1.2.2 Interruption time The interruption time is the time between the end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay. When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. T∆ is time for fine time tracking and acquiring full timing information of the target cell. Tprocessing is time for UE processing. Tprocessing can be up to 20 ms. Tmargin is time for SSB post-processing. Tmargin can be up to 2 ms. TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in table 8.1-1 of TS 38.213 [3].

At the meantime, RAN2 are still discussing the condition for handover command triggering and KPI for event prediction. In current evaluation, F1 score is used for indirect prediction and the following counters are used:

Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa

Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD]

Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD]

Based on further RAN2 progress, RAN4 can discuss whether to define new quantities, requirements or applicability for indirect prediction.

Proposal 3: For indirect prediction, RAN4 to discuss whether the existing handover delay requirements can be reused.

Proposal 4: For indirect prediction, RAN4 to discuss the possibility of updates on event conditions or requirements applicability.

Proposal 5: For indirect prediction, RAN4 to discuss whether new quantities or requirements are needed based on further RAN2 progress.

- Conclusions

In this contribution, we discussed the possible RAN4 requirements for measurement event prediction and got the following proposals:

Proposal 1: For measurement event prediction, RAN4 to discuss RRM requirements impact starting from indirect prediction case.

Proposal 2: For indirect prediction, the requirements for intermediate output are needed and the requirements for measurement prediction can be used as baseline.

Proposal 3: For indirect prediction, RAN4 to discuss whether the existing handover delay requirements can be reused.

Proposal 4: For indirect prediction, RAN4 to discuss the possibility of updates on event condition or requirements applicability.

Proposal 5: For indirect prediction, RAN4 to discuss whether new quantities or requirements are needed based on further RAN2 progress.

- References

R4-2502594 WF on FS_NR_AIML_Mob

RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

RP-240082 Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

Annex RAN2 agreements

RAN2#127bis meeting:

Agreements

Measurement event prediction simulations will at least focus on intra-frequency FR2, case A, and second study goal (i.e. HO KPI improvement).   FFS what is KPI.

Companies can bring simulation results for intra-frequency measurement reduction for FR1 and report what they are doing.  Focus on temporal case B.

Companies will prioritize simulations on indirect method.   Companies can bring simulations on direct method and should report what method is being used.

Measurement event prediction results are expected in RAN2#129

Agreements on simulation assumptions

The Simulation assumption of RRM measurement prediction can be reused unless otherwise specified.

Companies can pick and report what they are using for filtering options (similar to RRM prediction)

Companies will focus on sub-case 2 for measurement event prediction.   Companies can simulate other sub-cases if they wish and report what they are using.

Leave the simulation parameter discussion for email discussion.  Pick only one value for A3.

It is up to company implementation how to model UE behavior after A3 event is trigger.  Focus on intermediate KPIs for this exercise.  System level KPI is FFFs

For measurement event prediction, traffic is not simulated.

Agreements on inputs/outputs and KPIs - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection, - Continue discussion over email discussion to see if there is a difference.

- RAN2#128 meeting:

For System level simulation 1 For measurement event prediction for temporal domain case B: UE reports when A3 event is satisfied with actual measurements and predicted results. And handover command will be received after handover preparation. Remove T0 from the picture 2 For measurement event prediction for temporal domain case A, company focus on option 2 or option 3. Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1)) Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT. 3 Handover Preparation time is 40ms and handover execution time: 40ms 4 RAN2 focus on indirect prediction methodology.

Agreements on direct prediction for RLF and event predictions 1 The time window for direct prediction goes for interpretation 2 2 For direct prediction, following values are agreed and company can report probability threshold for corresponding case (other values are allowed) Measurement event prediction; RLF prediction. Time window length (Interpretation 2); PW length as indirect case (FR1:200ms; FR2:400ms); PW length as indirect case FR1/FR2: 400ms.

Agreements on event prediction 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model 2 The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed. 3 For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional. 4 To agree following definition for true event prediction, false event detection and missed event detection for indirect measurement event prediction Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD] 5 To agree following definition for true event prediction, false event detection and missed event detection for direct measurement event prediction Counter n3’ (true event prediction): it increases by 1 when a real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n1’ (false event detection): it increases by 1 when no real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n2’ (missed event detection): it increases by 1 when a real event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold 6 Agree following definition of F1 score: F1 score = 2*Precision*Recall/(Precision + Recall) Formula_3 (13/14) 7 In addition to F1 score, precision and recall in following formula are optional metrics to report. (12/14) Precision = n3/(n1+n3) Formula_1 Recall =n3/(n2+n3) Formula_2 Note: for direct prediction, the counter should be n1’,n2’,n3’

- RAN2#129 meeting:

Objective in Case A (FR2): Predict A3 events to enhance HO execution via earlier HO Prep upon A3 event prediction (as shown in Option 2 or Option 3), i.e., HO performance enhancement. - Objective in Case B (FR1): Predict A3 events in the presence of skipped measurements, i.e., overhead reduction while maintaining similar HO performance.

Agreements on Measurement event case A - Most of the results show that the F1 score for prediction of measurement event is very good. - For handover decision option 3 and 2 AI/ML performs better than baseline (legacy) in terms of HO failure rate.

Agreements on Measurement event case B - Measurement event case B can have very good F1 score depending on filtering approach/PW - F1 score is good for the PW window for case B (i.e. low PW) - F1 score decreases with an increasing MRRT value - We will focus on finding cases that bring benefits to the system rather than trying to compare - With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism.

Other agreements Inter-frequency measurement event prediction will be considered for the specification impact study but no explicit simulations will be required.

Agreements on direct measurement event prediction - F1 score for direct measurement is very good based on the current simulation results.