# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503354

# Wuhan, China, 7th – 11st April, 2025

Agenda item:	7.26.4.1

Source:	OPPO

Title:	Simulation assumption and results for LP-WUS/WUR

Document for:	Approval

- Introduction

Previous RAN4 meetings discussed simulation assumptions for LP-SS measurements, and reached some agreements on the parameters as well as the target baseband accuracy requirements, i.e., {3.5, 2.5} dB, in [1][2][3][4]. This contribution will give our simulation results for LP- RSRP based on the updated simulation assumptions.

Issue 2-1-4: Accuracy baseline for simulation Agreement: Candidate absolute baseband accuracy to derive the number of samples for simulation alignment purpose only: - For FR1: - For OOK based LP-WUR, {3.5, 2.5} dB for LP-RSRP and {3.5, 2.5} dB for LP-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - For OFDM based LP-WUR, {3.5, 2.5} dB for SS-RSRP and {3.5, 2.5} dB for SS-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - Note 1: the numbers can be updated if needed based on the simulation results and/or the progress in RF session - Note 2: the numbers for accuracy requirement is a separate discussion, and different numbers for OOK based and OFDM based LP-WUS for accuracy requirement are not precluded.

- Discussion

In last meeting, the LP-SS sequence for simulation was agreed as below:

Agreements in RAN4#114 meeting: Issue 2-1-5: On ideal LP-RSRP/RSRQ in simulation Agreement: Use on method 1 in P2 as a base further check the difference between method 1 and method 2 in P2 and check the channel estimation assumption in ideal RSRP for OOK based LR Issue 2-1-6: On SCS in simulation Agreement: Using SCS = 30KHz as the base in the simulation Issue 2-1-7: On OOK interference Agreement: Define a specific LP-SS binary sequences of the serving and interference cell for simulation alignment. Issue 2-1-4: On LP-SS sequence for simulation purpose Agreement: For RAN4 simulation alignment purpose: For M=1, L=4 (if supported), the set of LP-SS binary sequences is: Serving cell: [0 1 0 1] Interference cell: [0 1 1 0] For M=1, L=6 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 1 0 1 0] Interference cell: [0 1 0 1 0 1] For M=1, L=8 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 1 0 0 1 0 1] Interference cell: [1 0 1 0 1 0 0 1] For M=2, L=8 (if supported), the set of LP-SS sequence is: Serving cell: [0 1 0 1 1 0 0 1] Interference cell: [0 1 1 0 0 1 0 1] For M=2, L=12 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 0 1 1 0 0 1 1 0 0 1] Interference cell: [0 1 1 0 1 0 0 1 1 0 0 1] For M=2, L=16 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1] Interference cell: [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1] For M=4, L=16 (if supported), the set of LP-SS sequence is down-selected between: Serving cell: [0110100110101010] Interference cell: [0110101010011010] For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between: Serving cell: [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] Interference cell: [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1] Issue 2-2-1: On simulation campaign Agreement: delta RSRP and delta RSRQ (the delta RSRP is the largest value of the difference between real RSRP and ideal RSRP at 5% and 95% point) For simulation alignment purpose, interested companies can also report the “mean” value. For the number of samples: It is up to companies whether to provide result for all or some of the numbers of samples.

And we simulated the following candidate combinations {M, L} for LP-SS sequences:

- OOK-4 M=1, LP-SS sequence length L=4, i.e. 4 OFDM symbols

  - Serving cell:    [0 1 0 1]

  - Interference cell: [0 1 1 0]

- OOK-4 M=1, LP-SS sequence length L=8, i.e. 8 OFDM symbols

  - Serving cell:    [1 0 1 0 0 1 0 1]

  - Interference cell: [1 0 1 0 1 0 0 1]

- OOK-4 M=2, LP-SS sequence length L=8, i.e. 4 OFDM symbols

  - Serving cell:    [0 1 0 1 1 0 0 1]

  - Interference cell: [0 1 1 0 0 1 0 1]

- OOK-4 M=2, LP-SS sequence length L=16, i.e. 8 OFDM symbols

  - Serving cell:    [1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

  - Interference cell: [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

- OOK-4 M=4, LP-SS sequence length L=16, i.e. 4 OFDM symbols

  - Serving cell:    [0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0]

  - Interference cell: [0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0]

- OOK-4 M=4, LP-SS sequence length L=32, i.e. 8 OFDM symbols

  - Serving cell:    [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

  - Interference cell: [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

In our simulation, the SINR = -3dB is considered, with both 9dB and 6dB power difference between the serving cell and the interference cell, and the other simulation parameters can be found in Annex A. The simulation results are summarized in Table-1 and Table-2 respectively.

Table 1: RSRP accuracy @ Es/Iot = -3dB with 9dB power difference (SNR = [-2.7, -11.7] dB)

| Channel | AWGN |  |  |  | TDL-C |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. of samples | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 |
| OOK-4 M=1 length L=4 | 2.16 | 1.8 | 1.6 | 1.5 | 3.47 | 2.68 | 2.43 | 2.12 |
| OOK-4 M=1 Length L=8 | 2.10 | 1.81 | 1.68 | 1.61 | 3.01 | 2.61 | 2.53 | 2.39 |
| OOK-4 M=2 length L=8 | 1.36 | 1.12 | 1.02 | 0.95 | 1.91 | 1.51 | 1.31 | 1.26 |
| OOK-4 M=2 length L=16 | 1.23 | 1.05 | 0.96 | 0.92 | 1.53 | 1.27 | 1.15 | 1.09 |
| OOK-4 M=4 length L=16 | 1.57 | 1.30 | 1.18 | 1.12 | 2.82 | 2.37 | 2.14 | 2.04 |
| OOK-4 M=4 length L=32 | 1.17 | 0.98 | 0.89 | 0.84 | 1.96 | 1.58 | 1.40 | 1.27 |

Table 2: RSRP accuracy @ Es/Iot = -3dB with 6dB power difference (SNR = [-2.4, -8.4] dB)

| Channel | AWGN |  |  |  | TDL-C |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. of samples | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 |
| OOK-4 M=1 length L=4 | 2.13 | 1.71 | 1.55 | 1.45 | 3.43 | 2.60 | 2.32 | 2.03 |
| OOK-4 M=1 Length L=8 | 2.27 | 1.99 | 1.87 | 1.81 | 3.62 | 3.38 | 3.22 | 3.14 |
| OOK-4 M=2 length L=8 | 1.34 | 1.10 | 1.00 | 0.93 | 1.83 | 1.42 | 1.21 | 1.18 |
| OOK-4 M=2 length L=16 | 1.22 | 1.04 | 0.94 | 0.90 | 1.56 | 1.24 | 1.14 | 1.07 |
| OOK-4 M=4 length L=16 | 1.74 | 1.48 | 1.38 | 1.30 | 3.26 | 2.89 | 2.66 | 2.63 |
| OOK-4 M=4 length L=32 | 1.17 | 0.98 | 0.90 | 0.84 | 1.90 | 1.52 | 1.33 | 1.20 |

Observation 1: With at least 4 OFDM symbols for LP-SS, the RSRP measurement could achieve 3.5 accuracy at SINR = -3dB.

Observation 2: With at least 2 samples for LP-SS, the RSRP measurement could achieve 3.5 accuracy at SINR = -3dB.

- Conclusion

This contribution gave some simulation results for LP-RSRP measurement and the following observations.

Observation 1: With at least 4 OFDM symbols for LP-SS, the RSRP measurement could achieve 3.5 accuracy at SINR = -3dB.

Observation 2: With at least 2 samples for LP-SS, the RSRP measurement could achieve 3.5 accuracy at SINR = -3dB.

References

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR, vivo, RAN4 #111

- R4-2413899, WF for RRM core requirements for LP-WUSWUR, vivo, RAN4 #112

- R4-2416861, WF on RRM requirements for NR_LPWUS, vivo, RAN4 #112bis

- R4-2502600, WF on RRM requirements for NR_LPWUS, vivo, RAN4 #114

Annex Simulation assumption

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | Agreement: 2.6 GHz initially |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | [Data, SSB and LP-SS have the same SCS] |
| Subcarrier spacing | 30KHz initially TBD for 15 KHz |
| Measurement period (in number of measurement samples) | [4, 5, other number could be studied upon a need] 1-4 samples |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS initially (11PRB, 5MHz) TBD for 15KHz SCS |
| SSS | 30KHz for SSS, TBD for 15KHz |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-1; or OOK-4 with M = [2,4] Note: M value [2 4] are up to company selection | when Cell 1 uses OOK-1; OOK-1 or NR signal is used for Cell 2 when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 or NR signal is used for Cell 2 |
| LP-SS pattern |  | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm (13)kHz OOK based receiver [5 10] ppm (13, 26)kHz | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: - T= 5us for OOK-1 - T= 2us for OOK-4 with M=2 - T= 1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
|  |  | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
| Ês/Iot | dB | -3; -0.5dB; 2dB | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [3th/5th Order Butterworth with 3.96MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4 or 8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [3.84 or 7.68MHz] |