# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503355

# Wuhan, China, 7th – 11st April, 2025

Agenda item:	7.26.4.2

Source:	OPPO

Title:	Discussion on RRM requirements for LP-WUSWUR

Document for:	Approval

- Introduction

Previous meeting discussed the RRM requirements regarding LP-WUS/WUR and reached some agreements in [1] [2]. This contribution will provide our further considerations on the open issues captured in [3].

- Discussion

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Background: Case #1 and #3 have been agreed RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. RAN4 to further discuss case #2: RRM measurement case index; MR serving cell measurement; MR neighbouring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. Case #4 is covered by issue for higher priority frequency layer: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #4 Relaxed case c; Off; On, FFS the condition and the details; ON. Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading - Proposals Regarding case 2: P1: Support (CMCC vivo); P2: De-prioritize (xiaomi oppo); P3: Not consider (ZTE Ericsson Huawei MTK Apple); P4: Up to RAN2’s decision (LG vivo Nokia) Issue 1-1-10: Considerations on higher priority frequency layer - Proposals - P1: Support relaxed higher priority frequency layer search at case 1 and case 3 (xiaomi CT ZTE Huawei vivo Ericsson CT) - P1-1: For case 1 and case 3, when the serving cell is above threshold for inter-frequency neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search. (Ericsson vivo) - P1-2: If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #4), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed. If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements. (Huawei) - P2: Higher priority layer neighbour cell measurement shall also be OFF for case #1. (Apple vivo) - P3: When LR and MR operates at different band, whether to turn on higher priority measurement depends on the solution for issue 1-1-8. If Solution 1 for issue 1-1-8 is adopted, then the higher priority measurement can be OFF. If Solution 2 or 3 for issue 1-1-8 is adopted, the higher priority measurement should be ON (CMCC)

In last meeting, the handling of high priority frequency layer in case #1 was discussed but no consensus was reached. According to the previous discussion, case #1 has a clear definition as fully offloading case which means MR is turned off and LR is turned on. It is not reasonable to perform MR measurement and define measurement requirements on neighbor cell even if they are high priority frequency layers. And this would also cause collision and confusion to RAN2 discussion on the criteria. If it is agreed to consider high priority frequency layer in case #1, we need to redefine the case and send LS to RAN2 to clarify the cases.

For case #2, as we discussed in previous meeting, it is not reasonable to only measure serving cell but skip neighbor cell as the MR is already turned on. We think we do not need to consider case #2 in this release.

Proposal 1: Do not consider case #2 in Rel-19.

Proposal 2: Higher priority layer neighbor cell measurement shall also be OFF for case #1.

Issue 1-1-3-1: On RRM requirements to be specified for OOK-based LP-WUR based on LP-SS with overlaid OFDM sequence Agreement: - In Rel-19, the same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS regardless of whether overlaid OFDM sequence is configured or not. Issue 1-1-3-2: On RRM requirements to be specified for SSB-based LP-WUR based only on LP-SS with and without overlaid OFDM sequence Agreement: - Define the requirements for OFDM-based LP-WUR serving cell measurement based only on LP-SS - Reuse the same requirement for LP-SS OOK based

In last meeting, it was agreed to define the requirements for OFDM-based LP-WUR serving cell measurement based only on LP-SS and to reuse the same requirement for OOK based LP-WUR serving cell measurement on LP-SS. But after checking the WID, we found the agreement is conflicted with the following note in the WID.

- Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point.

We think RAN4 agreements should be aligned with the WID, and we need to handle the collision by discarding the agreement or removing the note.

Proposal 3: RAN4 to resolve the conflict between RAN4 agreements on OFDM-based LP-WUR serving cell measurement based only on LP-SS and the note in WID.

Issue 1-2-1: LR Accuracy requirements - Proposals - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (ZTE Apple xiaomi Ericsson Huawei CT vivo MTK) - P2: Option 1: define dedicated accuracy requirement in the performance section but no test case; Option 2: don’t define the accuracy requirement for LR-WUR based RRM measurement in Idle/inactive states, but use it as the simulation target when evaluate the measurement delay requirement simulation. (CMCC) - P3: To enable reliable return to MR from LR in a timely manner, it is necessary to have absolute measurement accuracy requirements defined for LR; RAN4 should develop LP-SS based and SSB based measurement accuracy requirements (Nokia)

Since there is no measurement report in IDLE/INACTIVE state, we are fine to not define dedicated accuracy requirements but to reflect the performance as a margin in the core requirements. And the margin are based on the simulation results. After the margin is decided, we need to decide whether it should be captured in RAN2 or RAN4 spec and send LS to RAN2 to inform the margin.

Proposal 4: Do not define dedicated accuracy requirements but to reflect the performance as a margin in the core requirements.

Proposal 5: Send LS to RAN2 to inform the existence of the margin that reflects accuracy.

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements Agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. Agreement: When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity. Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity - Proposals - P1: In case the periodicity of LP-SS can be configured to small value like 80ms or 160ms (xiaomi) - Option 1: define the lower limit for measurement delay requirements; - Option 2: define the measurement delay requirements based on max(LP-SS periodicity, LO periodicity). - P2: Periodicity for measurement and evaluation delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode is 320ms if LP-SS periodicity 80ms and 160ms are determined to be included. (Huawei vivo) - P3: No need for lower bound, before RAN1 agrees other periodicities. (Nokia) Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity - Proposals - P1: Do not define upper bound for SSB-based LP-WUR measurement periodicity. (xiaomi CMCC oppo Huawei) - P2: Prefer the periodicity for measurement and evaluation delay requirements for LP-WUR based on SSB in IDLE/Inactive mode is 1.28s if the LO periodicity is 2.56s. (vivo)

In last meeting, the periodicity(interval) for SSB based LP-WUR measurement delay requirements was discussed and it was agreed to define requirements based on LO periodicity, but there are still open parts to be decided.

The first one is whether to define upper bound when LP-SS is not configured. If the upper bound is defined, the requirements would be defined as min(N*LO periodicity, Tupper bound). And it means when DRX cycle is large, UE is required to perform measurement based on an interval smaller than DRX cycle. We think it is not reasonable or necessary, and there were not any such requirements in legacy.

The second one is whether to use LP-SS periodicity when it is larger than LO periodicity. We think it is not needed to define specific requirements for this case and it is better to apply same requirements by using LO periodicity. On the other hand, since we are defining SSB based measurement requirements, it is not preferred to use LP-SS periodicity which may cause confusion. It is possible that only OFDM based receiver is implemented to measure SSB and UE may not differentiate the cases when LP-SS is configured or not configured, then the result is that UE will still follow the requirements only based on LO periodicity.

For LP-SS based measurement requirements, it has been agreed to use LP-SS periodicity as baseline, but it is still open whether to define lower bound on the delay requirements. Since the LP-SS periodicity is still under discussion in RAN1, we prefer to postpone the decision. If RAN1 decide that LP-SS periodicity can be small, e.g., smaller than 160ms, to guarantee the power saving gain, RAN4 can consider to define lower bound for delay requirements.

Proposal 6: No need to define upper bound when LP-SS is not configured for SSB based LP-WUR measurement requirements.

Proposal 7: For SSB based LP-WUR measurement requirements, when LP-SS is configured, apply the requirements based on LO periodicity regardless of LP-SS periodicity.

Proposal 8: Postpone the decision on lower bound of LP-SS based LP-WUR measurement requirements. If the LP-SS periodicity can be configured with small value, RAN4 can consider to define the lower bound for delay requirements.

Issue 1-2-4-2-2: On single or multiple LR evaluation requirements Agreement: Define a single LR evaluation requirement for both LR entry and exit threshold evaluation for LP-WUS monitoring, case 1 and case 3, i.e., LR evaluation requirement is not differentiated on entry or exit; or on different cases. Note: RAN2’s discussion on timer will not impact on RAN4’s evaluation requirement. Issue 1-2-4-2-3: On how to define LR evaluation requirements Agreement: LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. Number x and y are defined as measurement requirement as [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

In last meeting, it was agreed that LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. Based on the agreement, x and y can be determined by simulation results. For x1 or y1, similar as legacy MR evaluation requirements, they can be N times of x or y.

Proposal 9: For LR evaluation requirements, x and y can be determined by simulation results, and x1 or y1 can be N times of x or y.

For MR evaluation requirements, the same approach as LR can be reused, i.e., single requirements can be defined for both entry and exit threshold evaluation and not to differentiate different cases. For detailed requirements, the existing Nsev can be reused.

Proposal 10: Define a single MR evaluation requirement for both entry and exit threshold evaluation and not to differentiate different cases. The existing Nsev can be reused.

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR - Proposals - P1: MR wake-up time is pending on UE capability which can be 400ms and 800ms based on RAN1 agreement; MR sync time, need to be further discuss with different side conditions (LG) - Condition 1: MR and LR well sync, and LR sync info can be fully reused e.g. 1~2 SSB sufficient - Condition 2: MR and LR not well synced and MR need to apply sync procedure e.g. at least 10 SSBs required - P2: Wait for RAN1 further progress. (CMCC CATT oppo Huawei Ericsson) - P3: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. (Apple) - P4: After MR is triggered to be waken up and during the waking up duration, MR should follow SSB periodicity instead of DRX periodicity to perform measurement (vivo) - P5: Defining MR wake up requirement for the normal case and the “wake up delay” duration for the normal is: “wake up delay” duration = Ramping duration + [5] samples * SSB periodicity; Whether to further consider a worst case where a cell reselection is needed after ME is wake up needs FFS. (vivo) - P6: RAN4 shall define the minimum required time for MR synchronisation and related side conditions (Nokia)

LS R4-2503110_R1-2501624: In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120: Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

Based on the latest RAN1 agreements and the LS [4], RAN1 has defined the UE capability report for wake-up delay which indicates the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. With this agreement, we understand RAN4 does not need to define additional delay or interruption requirements.

In addition, RAN1 assume 3 and 5 SSB samples for synchronization when defining the candidate values of capability. Based on the PSS/SSS detection requirements in section 9 in 38.133, we understand the assumption is reasonable and no differentiation on SSB periodicities if only PSS/SSS detection is needed. But additional 3 SSB samples are assumed for SSB index acquisition, we can inform RAN1 about this assumption and it depends on RAN1 whether to consider it in the wake-up delay.

Proposal 11: RAN4 not define additional delay or interruption requirements on top of UE capability report of wake-up delay defined in RAN1.

Proposal 12: The assumption of 3 and 5 SSB samples in wake-up delay definition in RAN1 is reasonable if only PSS/SSS detection is needed. Inform RAN1 that 3 SSB samples are assumed for SSB index acquisition.

Proposal 13: The assumption on the number of SSB samples has no differentiation on different SSB periodicities.

Issue 1-4-1: LP-WUR at CONNECTED mode - Proposals - P1: No RRM objectives is expected for connected mode in this WI. (CATT xiaomi oppo ZTE Apple CT MTK) - P3: Postponed until more progress in other groups (Ericsson Huawei vivo) - P4: RAN4 to introduce interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. The interruption on WUS monitoring follows the current interruption requirements on DL data during MGs. (Qualcomm)

According to the objectives description in the WID, the measurement is performed by MR at connected mode and no new RAN4 RRM requirements are needed.

Proposal 14: No RRM objectives is expected for connected mode in this WI.

- Conclusion

This contribution gave the following proposals on the core requirements for LP-WUR/WUS.

Proposal 1: Do not consider case #2 in Rel-19.

Proposal 2: Higher priority layer neighbor cell measurement shall also be OFF for case #1.

Proposal 3: RAN4 to resolve the conflict between RAN4 agreements on OFDM-based LP-WUR serving cell measurement based only on LP-SS and the note in WID.

Proposal 4: Do not define dedicated accuracy requirements but to reflect the performance as a margin in the core requirements.

Proposal 5: Send LS to RAN2 to inform the existence of the margin that reflects accuracy.

Proposal 6: No need to define upper bound when LP-SS is not configured for SSB based LP-WUR measurement requirements.

Proposal 7: For SSB based LP-WUR measurement requirements, when LP-SS is configured, apply the requirements based on LO periodicity regardless of LP-SS periodicity.

Proposal 8: Postpone the decision on lower bound of LP-SS based LP-WUR measurement requirements. If the LP-SS periodicity can be configured with small value, RAN4 can consider to define the lower bound for delay requirements.

Proposal 9: For LR evaluation requirements, x and y can be determined by simulation results, and x1 or y1 can be N times of x or y.

Proposal 10: Define a single MR evaluation requirement for both entry and exit threshold evaluation and not to differentiate different cases. The existing Nsev can be reused.

Proposal 11: RAN4 not define additional delay or interruption requirements on top of UE capability report of wake-up delay defined in RAN1.

Proposal 12: The assumption of 3 and 5 SSB samples in wake-up delay definition in RAN1 is reasonable if only PSS/SSS detection is needed. Inform RAN1 that 3 SSB samples are assumed for SSB index acquisition.

Proposal 13: The assumption on the number of SSB samples has no differentiation on different SSB periodicities.

Proposal 14: No RRM objectives is expected for connected mode in this WI.

References

- R4-2502577, Ad-hoc minutes for NR_LPWUS WI, vivo, RAN4#114

- R4-2502600, WF on RRM requirements for NR_LPWUS, vivo, RAN4#114

- R4-2500539, Topic summary for [114][225] NR_LPWUS, vivo, RAN4#114

- R4-2503110 (R1-2501624), LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode, Apple(RAN1), RAN4#114bis

Annex

3GPP TSG-RAN WG4 Meeting #114bis	                                              R4-25XXXX

Wuhan, China, 7th – 11st April, 2025

Title:	Reply LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode

Response to:		R4-2503110 (R1-2501624)

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

#### Name:	Qiuge Guo

E-mail Address:	guoqiuge@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1. Overall Description:

RAN4 thanks RAN1 for the LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode (	R4-2503110 (R1-2501624)). RAN4 has discussed assumptions in RAN1 LS and reached the following conclusion:

- The assumption of 3 and 5 SSB samples in wake-up delay definition in RAN1 is reasonable if only PSS/SSS detection is needed.

- RAN4 would also like to inform RAN1 that 3 SSB samples are assumed for SSB index acquisition.

- The assumption on the number of SSB samples has no differentiation on different SSB periodicities.

# 2. Actions:

To RAN1

ACTION: 	RAN4 kindly asks RAN1 to take the above information into account.

# 3. Date of Next RAN4 Meetings:

3GPP TSG RAN WG4#115	19 – 23 May, 2025		Malta

3GPP TSG RAN WG4#116	25 – 29 August, 2025		India