# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503356

# Wuhan, China, 7th – 11st April, 2025

Title: 	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Source: 	OPPO

Agenda item:	7.17.2.1

Document for:	Approval

Introduction

In last meeting, FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor was discussed and the agreements were captured in the approved WF[1].

In this contribution, we will provide our views on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor.

Discussion

In our view, two issues should be discussed for UE power consumption due to FBS with multi-Rx operation: how to define entering and exiting rules of FBS, and how UE can meet the conditions and corresponding requirements.

In RAN4#113 meeting, RAN4 agreed to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement, but not to support UE mobility status based triggering condition in R19. And it was agreed not to consider additional conditions to activate FBS-based L3 measurements.

RAN4#113 meeting: Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF (including activation condition and de-activation conditions): Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19. RAN4#114 meeting: Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF Agreement in ad-hoc session: - Discuss if additional conditions are needed to activate FBS-based L3 measurements. - Do not consider additional conditions beside the agreements from RAN4#113 meeting.

Also in last meeting, the usage of FBS trigger condition was agreed, i.e., the FBS-based L3 measurements can be triggered when serving cell quality is worse than a threshold and the FFS part is whether to limited the serving cell to FR2 serving cell which is for CA/DC case and whether to add hysteresis and timeToTrigger parameters.

Agreement in RRM session (Monday): - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. - Further discuss whether to limit the serving cell to FR2 serving cell. - Further discuss whether to add the hysteresis and timeToTrigger parameters. - Note: the scenario related to single carrier or CA/DC is discussed in another issue. - The signaling design is up to RAN2.

For CA/DC case, since we do not have enough to finish all the related requirements, we prefer not to touch it, i.e., not to define part of requirements. So we think the first FFS is not needed to be discussed. And if we must make the decision, we think it should be limited to FR2 serving cell.

For the second FFS, we understand the threshold as well as the hysteresis and timeToTrigger parameters is within RAN2 scope and should be left to RAN2.

Proposal 1: The serving cell used in the FBS trigger condition should be limited to FR2 serving cell.

Proposal 2: The threshold of FBS trigger condition as well as the hysteresis and timeToTrigger parameters should be left to RAN2.

For exiting condition, it was discussed in last meeting with the options in [2] but did not reach any consensus.

- Recommended WF - Discuss the following proposed quit conditions: - UE indication to network, e.g., Rel-18 UAI ‘multiRx-PreferenceFR2’. - UE deactivates FBS based L3 measurements when UE has reported qualified measurement results. - UE does not performing fast L3 measurements when threshold is not configured to UE. - Combination of serving cell’s signal quality threshold and a validity timer, including condition to enter FBS mode is no longer met, etc. - Discuss the following principles and check if they are agreeable: - UE autonomous quitting should be avoided. - Send LS to RAN2 about the criteria/thresholds for triggering/deactivating Multi-Rx operation on L3 measurements and the corresponding UE behaviours after the whole framework is decided.

Considering the UE power consumption is an important issue for Multi-Rx operation for L3 measurement, we prefer to have this quit condition. As discussed in the last meeting, if UE stays in Multi-Rx mode for a long time but not meets the handover condition, the power consumption would be high and may cause overheating issue. There are three types of conditions discussed for quitting Multi-Rx based L3 measurement including UAI indication, event trigger and timer based. We understand all the three approaches can achieve the goal but using UE indication is the simplest one and can be aligned with Rel-18 Multi-Rx. For the detailed signaling, we think either the legacy signalling like the overheating indication, or new UAI indication of UE preference is feasible. We can leave it to RAN2 decision.

Regarding the quit condition when UE has reported qualified measurement results, we think it can be used for handover or RRC Connection Release with Redirection but not to be applicable for intra- and inter-frequency measurement. And also the definition of “qualified” is not clear. We prefer not to use the measurement report to deactivate the FBS mode, and UE can deactivate the FBS after the procedure delay.

Proposal 3: For scenario 1/2/3/4, UE indication to the NW can be used to deactivate FBS based L3 measurements. The detailed signaling can leave to RAN2 decision.

Proposal 4: For scenario 5/7, UE deactivate FBS based L3 measurements after the delay for handover or RRC Connection Release with Redirection.

- Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF - Proposals - Option 1a: (ZTE, CATT, Ericsson, CT, CMCC) - Introducing the UE capability of reduced N, the candidates could be 2, 4 or 6. - Option 1b: (QC) - At least, support reduction of beam sweeping factor from 8 to 4 for multi-RX capable UEs in Rel-19. Support of other beam sweeping factors can be FFS. - Option 1c: (MTK) - On the solution to achieve reduced beam sweeping factor (BSF) as UE capability, RAN4 to decide to use either: - Single value for reduced BSF (e.g., 6 or 4) - Multiple candidate values for BSF (e.g., 6 and 4) - Option 2: (vivo) - RAN4 to consider the following approaches as solutions: - Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc. - Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc. - Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4. - Option 3: (HW) - The UE capability design shall allow enough flexibility on supporting this feature considering: - Different BC - Number of Carriers/MOs to measured - Intra-frequency/inter-frequency - Option 4: (LGE) - Introduce scaling value (Pmulti-Rx) < 1 to reduce measurement delay

For the solutions to specify Multi-Rx based L3 measurement delay, it was agreed that the Rx BSF can be reduced according to the UE capability. Then for the agreed scenarios, the measurement requirements can be updated by replacing beam sweeping factor (i.e., 8) with UE capability. The candidate values of this capability can be 2, 4 or 6.

Proposal 5: For scenario1/2/3/4/5/7, the measurement requirements can be updated by replacing existing beam sweeping factor (i.e., 8) with UE capability.

Conclusions

In this paper, we provided our views on FR2-1 SSB based L3 measurement delay reduction for optimizing BSF. And the following proposals are provided.

Proposal 1: The serving cell used in the FBS trigger condition should be limited to FR2 serving cell.

Proposal 2: The threshold of FBS trigger condition as well as the hysteresis and timeToTrigger parameters should be left to RAN2.

Proposal 3: For scenario 1/2/3/4, UE indication to the NW can be used to deactivate FBS based L3 measurements. The detailed signaling can leave to RAN2 decision.

Proposal 4: For scenario 5/7, UE deactivate FBS based L3 measurements after the delay for handover or RRC Connection Release with Redirection.

Proposal 5: For scenario1/2/3/4/5/7, the measurement requirements can be updated by replacing existing beam sweeping factor (i.e., 8) with UE capability.

Reference

- R4-2502593 WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2500532, Topic summary for [114][218] NR_RRM_Ph5_Part2, CATT