3GPP TSG-RAN4 Meeting #114BIS	R4-2503361

# Wuhan, Hubei, China, 7th – 11th April, 2025

Agenda item:			7.1.1.1.1

Source:	MediaTek

Title:	Discussion on HPUE intra-band/inter-band CA/DC with 2Tx/3Tx

Document for:	Approval

# Introduction

In RAN4#114 meeting, the WF[1] has been approved and recommended to further discuss and encourage more companies’ input for each open issues i.e., SAR compliance solution, Pcmax,c modification for intra-band non-contiguous CA UE (with Dual PA) and so on for HP CA/DC UE.

Way forward [1]: ## 1.3 MPR values for intra-band non-contiguous ULCA (with dualPA) Agreement: Further discussion in upcoming meetings ## 1.5 PCMAX,c and PCMAX for intra-band NCCA (with dual PA) Agreement: - PCMAX,C is upper bounded by 26dBm - Detailed spec change is discussed based on dCR provided in future meeting, CR leader is encouraged to take companies’ input into account - For PCMAX, no change of current spec ## Configurations Agreement: - RAN4 to develop general requirements based on at least Table 1, Table 2, Table 3, and Table 4 in Rel-19 - Table 1, Table 2, Table 3, and Table 4 are for information purpose, and are not expected to be captured into RAN4 TS. Table 1 Potential new UL configurations for PC2 inter-band UL CA with 2Tx or 3Tx 1st band; 2nd band; 2Tx or 3Tx. PC3 TDD 1Tx; PC2 FDD 1Tx; 2Tx. PC3 TDD 1Tx; PC2 FDD 2Tx; 3Tx. PC3 FDD 1Tx; PC2 FDD 1Tx; 2Tx. PC3 FDD 1Tx; PC2 FDD 2Tx; 3Tx. PC5 TDD 1Tx; PC2 TDD 1Tx; 2Tx. PC5 TDD 1Tx; PC2 TDD 2Tx; 3Tx. PC5 TDD 1Tx; PC2 FDD 1Tx; 2Tx. PC5 TDD 1Tx; PC2 TDD 2Tx; 3Tx. Table 2 Potential new UL configurations for PC2 inter-band EN-DC with 2Tx or 3Tx E-UTRA band; NR band; 2Tx or 3Tx. PC3 TDD 1Tx; PC2 FDD 1Tx; 2Tx. PC3 TDD 1Tx; PC2 FDD 2Tx; 3Tx. PC3 FDD 1Tx; PC2 FDD 1Tx; 2Tx. PC3 FDD 1Tx; PC2 FDD 2Tx; 3Tx. PC2 FDD 1Tx; PC3 TDD 1Tx; 2Tx. PC2 FDD 1Tx; PC3 FDD 1Tx; 2Tx. Note: Per the WID, only PC3 is considered for LTE FDD band in EN-DC configuration in this Rel-19 HPUE work under this WI. Table 3 Potential new UL configurations for PC1.5 inter-band UL CA with 2Tx or 3Tx 1st band; 2nd band; 2Tx or 3Tx. PC2 TDD 1Tx; PC2 TDD 1Tx; 2Tx. PC2 TDD 1Tx; PC2 TDD 2Tx; 3Tx. PC3 TDD 1Tx; PC1.5 TDD 2Tx; 3Tx. PC2 TDD 1Tx; PC1.5 TDD 2Tx; 3Tx. PC2 TDD 1Tx; PC2 FDD 1Tx; 2Tx. PC2 TDD 1Tx; PC2 FDD 2Tx; 3Tx. PC2 FDD 1Tx; PC2 TDD 2Tx; 3Tx. PC2 FDD 1Tx; PC2 FDD 1Tx; 2Tx. PC2 FDD 1Tx; PC2 FDD 2Tx; 3Tx. PC2 FDD 1Tx; PC1.5 TDD 2Tx; 3Tx. PC5 TDD 1Tx; PC1.5 TDD 2Tx; 3Tx. Table 4 Potential new UL configurations for PC1.5 EN-DC with 2Tx or 3Tx E-UTRA band; NR band; 2Tx or 3Tx. PC3 TDD 1Tx; PC1.5 TDD 2Tx; 3Tx. PC3 FDD 1Tx; PC1.5 TDD 2Tx; 3Tx. PC2 TDD 1Tx; PC2 TDD 1Tx; 2Tx. PC2 TDD 1Tx; PC2 TDD 2Tx; 3Tx. PC2 TDD 1Tx; PC2 FDD 1Tx; 2Tx. PC2 TDD 1Tx; PC2 FDD 2Tx; 3Tx. PC2 TDD 1Tx; PC1.5 TDD 2Tx; 3Tx. PC2 FDD 1Tx; PC2 TDD 1Tx; 2Tx. PC2 FDD 1Tx; PC2 TDD 2Tx; 3Tx. PC2 FDD 1Tx; PC2 FDD 1Tx; 2Tx. PC2 FDD 1Tx; PC2 FDD 2Tx; 3Tx. PC2 FDD 1Tx; PC1.5 TDD 2Tx; 3Tx. Note: Per the WID, only PC3 is considered for LTE FDD band in EN-DC configuration in this Rel-19 HPUE work under this WI. ## Re-organize 2Tx/3Tx Notes in clause 5 and clause 6 Agreement: - Companies are encouraged to continue the discussion on the necessity and possibility to re-organize the Notes pertaining to power class in future meetings. ## 4.2 Duty cycle solution for SAR compliance Agreement: - Detailed spec changes are checked based on corresponding dCRs

In this paper, we we share our view on the HP CA/DC UE related to configured Tx power requirements and SAR compliance solutions for 3Tx UE in Rel-19.

# PCMAX,C and MPR for intra-band non-contiguous CA UE

In RAN4 #114 Athens Meeting, The candidate Pcmax,c are proposed as below

  - Proposal 1: For PCMAX, no change of current spec (ZTE, vivo)

  - Proposal 2: For PC1.5 intra-band non-contiguous UL CA, consider up to 6dB PSD imbalance in configured transmitted power. (LGE)

  - Proposal 3: (Huawei)

- It’s proposed to set PPowerClass,c = 26dBm (not PCMAX,c = 26dBm) for the component carriers of UL NC CA for PC1.5 with dualPA-architecture.

- Under the condition of MPRc = MPR, a UE is not required to meet the configured maximum power per CC PCMAX,c during intra-band ULCA operations, and PCMAX,c is calculated only for PHR purpose.

  - Proposal 4: (Otion2 of last meeting’s WF)

  - PPowerClass,CA = min{29,26+10*log(1+4*min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))} dBm assuming 6dB PSD difference ,

or,

26+10*log(1+min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2)) assuming equal PSD, can be used as a starting point.

For the Pcmax for total configured maximum output power, RAN4 already agreed not to change the current specification. Then, Pcmax,c is can be followed the single carrier Pcmax,c per carrier of PC1.5 intra-band non-contiguous CA UE. The PPowerClass,c shall be 26dBm for PC1.5 dualPA-Architecture UE. However, we do not prefer to indicate the detail PSD different between CC1 and CC2 in specification.

So, we propose as follow for PC1.5 intra-band NC CA UE with dualPA-Architecture UE

Proposal 1: Following PCMAX,C equation will be considered for PC1.5 intra-band NC CA UE without any indicate with detail PSD level. RAN4 only assume the equal PSD for MPR requirements.

=========================== TS38.101-1 Clause 6.2A.4.1.2 ===============================

##### 6.2A.4.1.2	Configured transmitted power for Intra-band non-contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4.

For Power class 1.5 UE indicating dualPA-Architecture supported, the configured maximum output power PCMAX,c  on serving cell c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass,c – ΔPPowerClass,c + ΔPPowerBoost,c) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PPowerClass,c, PEMAX,c,  PPowerClass,c – ΔPPowerClass,c + ΔPPowerBoost,c}

========================================================================

For the MPR requirements of PC1.5 intra-band non-contiguous CA UE, RAN4 can estimate the delta MPR values between the MPR requirements of PC2 single carrier with 2Tx UE and the MPR requirements of PC1.5 single carrier with 2Tx UE. For the Edge RB allocations cases, the delta MPR values is 2.5dB~3dB for both DFT-S-OFDM and CP-OFDM waveforms since the MPR simulation assumptions of the dual PA configurations, ACLR, SEM and other parameters are same for those MPR evaluation. Only difference point is 3dB power offset between PC2 and PC1.5 MPR.

TS38.101-1 MPR requirements for PC2 and PC1.5 of single carrier (2Tx)

Table 6.2D.2-1: Maximum power reduction (MPR) for power class 2 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.52 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |

Table 6.2D.2-2: Maximum power reduction (MPR) for power class 1.5 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Based on the observation, the following MPR requirements for PC2 with dualPA-architecture to meet the -30dBm/MHz can be adjusted with delta MPR value. In other word, based on the PC2 MPR requirements, RAN4 can estimate the PC1.5 MPR requirements with dualPA-architecture UE

TS38.101-1 MPR requirements for PC2 for intra-band NC-CA UE with -30dBm/MHz emission (2Tx)

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15.5; 	0 ≤ B < 1.44

15.0; 	1.44 ≤ B < 2.88

14.0;    	2.88 ≤ B < 5.76

12.0; 	5.76 ≤ B < 10.8

10.5; 	10.8 ≤ B < 23.04

9.0; 	 	23.04 ≤ B

=========================== TS38.101-1 Clause 6.2A.2.2.1 ===============================

##### 6.2A.2.2.1.5	PC1.5 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 1.5 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15.5 + [2.5~3.0]; 	0 ≤ B < 1.44

15.0 + [2.5~3.0]; 	1.44 ≤ B < 2.88

14.0 + [2.5~3.0];   	2.88 ≤ B < 5.76

12.0 + [2.5~3.0]; 	5.76 ≤ B < 10.8

10.5 + [2.5~3.0]; 	10.8 ≤ B < 23.04

9.0 + [2.5~3.0]; 	23.04 ≤ B

##### 6.2A.2.2.1.6	PC1.5 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 1.5 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	19.5 + [2.5~3.0]; 	0 ≤ B < 1.08

19.0 + [2.5~3.0]; 	1.08 ≤ B < 2.16

18.0 + [2.5~3.0]; 	2.16 ≤ B < 5.04

16.5 + [2.5~3.0]; 	5.04≤ B < 10.08

16.0 + [2.5~3.0]; 	10.08 ≤ B < 36

12.0 + [2.5~3.0];  	36 ≤ B < 56.88

10.5 + [2.5~3.0]; 	56.88 ≤ B

=============================================================================

Proposal 2: To derive MPR requirements for PC1.5 intra-band NC CA UE with dualPA-Architecture, RAN4 consider 2.5~3dB delta MPR value from MPR of PC2 intra-band NC CA UE with dualPA-Architecture.

# 2-band Inter-band UL NR-CA/EN-DC with 3Tx

## SAR compliance with Duty cycle ratio between 2-band inter-band UL CA/EN-DC

For the 3 Tx inter-band CA with UL-MIMO UE, RAN4 need to update the SAR compliance solution in the clause 6.2H.3.1 UE maximum output power for inter-band UL CA with UL-MIMO and 6.2L.3.1 UE maximum power for inter-band UL CA with TxDiversity as follow.

=========================== TS38.101-1 Clause 6.2H.3.1.1 ===============================

If a UE supports power class 2 for the band combination listed in Table 6.2H.3.1-1:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is 23dBm or lower;

–	shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2H.3.4;

–	else if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38. 306 (The exact evaluation period is no less than one radio frame); or

–	 if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is 26dBm or lower;

–	shall apply all requirements for the power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2H.3.4;

–	else;

–	shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2H.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

–	else if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

If a UE supports power class 1.5 for the band combination listed in Table 6.2H.3.1-1:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or

–	if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is 23dBm or lower;

–	shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2H.3.4;

–	else if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 as defined in TS38.306 (The exact evaluation period is no less than one radio frame) but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or

–	if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is between 23dBm and 26dBm;

–	shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2H.3.4;

–	else;

–	shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified in clause 6.2H.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability 0.5maxUplinkDutyCycle-PC2-FR1 or maxUplinkDutyCycle-PC1dot5-MPE-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one band within the band combination is power class 1.5

–	if the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 are both absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 25%;

–	else if only one of the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 is reported;

–	the corresponding maxDutyNR,x or maxDutyNR,y is according to the reported capability;

–	else

–	the corresponding maxDutyNR,x or maxDutyNR,y is the smaller of maxUplinkDutyCycle-PC1dot5-MPE-FR1 and 0.5*maxUplinkDutyCycle-PC2-FR1;

–	if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

================ Unchanged sections are omitted ===========

=========================== TS38.101-1 Clause 6.2L.3.1.1 ===============================

If a UE supports power class 2 for the band combination listed in Table 6.2L.3.1-1:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2L.3.4 is 23dBm or lower;

–	shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2L.3.4;

–	else if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38. 306 (The exact evaluation period is no less than one radio frame); or

–	 if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2L.3.4 is 26dBm or lower;

–	shall apply all requirements for the power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2L.3.4;

–	else;

–	shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2L.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

–	else if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

If a UE supports power class 1.5 for the band combination listed in Table 6.2L.3.1-1:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or

–	if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2L.3.4 is 23dBm or lower;

–	shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2L.3.4;

–	else if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 as defined in TS38.306 (The exact evaluation period is no less than one radio frame) but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or

–	if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2L.3.4 is between 23dBm and 26dBm;

–	shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2L.3.4;

–	else;

–	shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified in clause 6.2L.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 or maxUplinkDutyCycle-PC1dot5-MPE-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one band within the band combination is power class 1.5

–	if the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 are both absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 25%;

–	else if only one of the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 is reported;

–	the corresponding maxDutyNR,x or maxDutyNR,y is according to the reported capability;

–	else

–	the corresponding maxDutyNR,x or maxDutyNR,y is the smaller of maxUplinkDutyCycle-PC1dot5-MPE-FR1 and 0.5*maxUplinkDutyCycle-PC2-FR1;

–	if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

=============================================================================

Proposal 3: For the 3Tx inter-band CA HP UE, RAN4 update the SAR compliance solutions in Clause 6.2H.3.1.1 and 6.2L.3.1.1 based on the agreed RAN4 CR approach (R4-2502865, R4-2502866) in TS38.101-1 and TS38.101-3.

## Re-organization 2Tx/3Tx Note in Clause 5 and 6

For the 3Tx Notes in clause 6 with suffix H (CA+UL-MIMO) and suffix L (CA+Tx Diversity), RAN4 can move detail device type information (FWA/CPE/other device type) or detail Tx/Rx antenna configuration in clause 4 general part to remove detail notes in clause 5 and 6.

Proposal 4: RAN4 add the relevant information of the target device type (FWA/CPE/other device type for 3Tx) and detail Tx Chain configuration into the general part of clause 4 to remove some sprinkled notes in clause 5 and 6.

- Conclusion

In this contribution, we share our view how to define the Pcmax,c and MPR requirements for intra-band non-contiguous CA UE (with Dual PA) and SAR compliance solution for HP inter-band CA with 3Tx UE and so on.

Based on our observation and analysis, we can propose as follow

Proposal 1: Following PCMAX,C equation will be considered for PC1.5 intra-band NC CA UE without any indicate with detail PSD level. RAN4 only assume the equal PSD for MPR requirements.

Proposal 2: To derive MPR requirements for PC1.5 intra-band NC CA UE with dualPA-Architecture, RAN4 consider 2.5~3dB delta MPR value from MPR of PC2 intra-band NC CA UE with dualPA-Architecture.

Proposal 3: For the 3Tx inter-band CA HP UE, RAN4 update the SAR compliance solutions in Clause 6.2H.3.1.1 and 6.2L.3.1.1 based on the agreed RAN4 CR approach (R4-2502865, R4-2502866) in TS38.101-1 and TS38.101-3.

Proposal 4: RAN4 add the relevant information of the target device type (FWA/CPE/other device type for 3Tx) and detail Tx Chain configuration into the general part of clause 4 to remove some sprinkled notes in clause 5 and 6.

## References

- R4-2502863, WF on TN HPUE, Samsung, Skyworks, OPPO, LGE, Apple, RAN4 #114

- R4-2420329, WF on HPUE RF requirements, Samsung, RAN4#113

- R4-2419699, 1UL HPUE NR-CA MSD Simplifications for Rel-19, Skyworks Solutions, Inc. RAN4#113.

- R4-2502865, Draft CR on duty cycle solution for PC1.5 of CA, vivo, LGE, China Telecom, Huawei, Hisilicon #114.

- R4-2502865, Draft CR on duty cycle solution for ENDC, vivo, LGE, China Telecom, Huawei, Hisilicon #114.