3GPP TSG-RAN4 Meeting #114bis	R4-2503367

# Wuhan, China, 7th-11th April, 2025

Source:	Samsung, KT, [Murata, Skyworks, Apple, Qualcomm]

Title:	TP for TR 37.719-11-11 DC_(n)3CA

Agenda item:			6.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.719-11-11 to include DC_(n)3CA with BCS0 according to the request in [1].

2. Reference

[1] RP-250143 Revised WID for Rel-19 Dual connectivity (DC) of x LTE band(s) y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands

# 3. Text Proposal

<Start of Text Proposal>

## 6.3	Intra-band contiguous DC

### 6.3.x	DC_(n)3

#### 6.3.x.1	Channel bandwidths per operating band for DC

Table 6.3.x.1-1: Supported channel bandwidths per DC configuration for intra-band contiguous EN-DC

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)3CA | DC_(n)3AA4 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 10, 20, 25, 30 |  | 50 | 0 |
|  |  |  | 10, 20, 25, 30 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |

Table 6.3.x.1-2: Supported channel bandwidths per DC configuration for mixed intra-band contiguous and non-contiguous EN-DC

| E-UTRA – NR configuration /Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)3CA5,6 | DC_3A_n3A2 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 10, 20, 25, 30 |  | 50 | 0 |
|  |  |  | 10, 20, 25, 30 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)3CA7 | DC_(n)3AA2 DC_3A_n3A2 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 10, 20, 25, 30 |  | 50 | 0 |
|  |  |  | 10, 20, 25, 30 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| NOTE 2: Only single switched UL is supported. NOTE 5: The UE supporting these configurations indicates intraBandENDC-Support-UL = ‘non-contiguous’ with intraBandENDC-Support absent. NOTE 6: The minimum requirements also apply for the intra-band non-contiguous fallback resulting from releasing an Scell within the sub-block bandwidth of the downlink configuration. NOTE 7: The UE supporting these configurations indicates intraBandENDC-Support-UL = ‘both’ with intraBandENDC-Support absent. |  |  |  |  |  |  |

#### 6.3.x.2	Configuration for DC

Table 6.3.x.2-1:  Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_(n)3CA | DC_(n)3AA6 | Yes6 |
| NOTE 6: Only single switched UL is supported |  |  |

Table 6.3.x.2-2:  Intra-band EN-DC configurations for mixed intra-band contiguous and non-contiguous EN-DC

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_(n)3CA6,7 | DC_3A_n3A5 | Yes5 |
| DC_(n)3CA8 | DC_3A_n3A5 DC_(n)3AA5 | Yes5 |
| NOTE 5: Only single switched UL is supported. NOTE 6: The UE supporting these configurations indicates ‘non-contiguous’ by IE intraBandENDC-Support-UL with intraBandENDC-Support absent. NOTE 7: The minimum requirements also apply for the intra-band non-contiguous fallback resulting from releasing an Scell within the sub-block bandwidth of the downlink configuration. NOTE 8: The UE supporting these configurations indicates ‘both’ by IE intraBandENDC-Support-UL with intraBandENDC-Support absent. |  |  |

< Editor’s note: Please use the same NOTE numbering as in TS 38.101-3.>

#### 6.3.x.3	Maximum output power for DC

Omit, as PC3 for UL DC_(n)3AA and UL DC_3A_n3A have already been specified.

#### 6.3.x.4	Spurious emission band UE co-existence for DC

No additional requirements to be added.

#### 6.3.x.5	Analysis of MSD requirements

To be updated during the meeting

#### 6.3.x.6	MPR, A-MPR requirements

No additional requirements to be added.

<End of Text Proposal>