3GPP TSG-RAN WG4 Meeting #114bis     	     R4-2503377

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.18.2

Source: 	CMCC

Title: 			 Discussion on interference suppressing performance for 8Rx UE

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed test scope and test parameters about interference suppressing performance for 8Rx UE, the WF has been approved in [1].

In this contribution, we further discuss the remaining open test parameters.

- Discussion

  - General

Issue 1-1-1: Definition of MMSE-IRC receiver capability for 8Rx UE

Agreement:

- If agreed to change the receiver definition, it also needs to be changed in Rel-18 maintenance.

- If receiver definition is agreed to be changed, an LS will be sent to RAN2 for updating TS38.306.

Proposals:

- Option 1: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 2: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 3: 
Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE declares supporting SU-MIMO 8Rx receiver capability

Considering the baseline and simplified 8Rx MMSE-IRC receiver have the same implementation in both SU-MIMO and MU-MIMO scenario, we prefer to reuse the Rel-18 capabilities rather than defining new Rel-19 capabilities. To ensure accurate description of Rel-18 capabilities, the existing Rel-18 UE capability definition need to be updated like stated in Option 1.

Proposal 1: Update the existing Rel-18 UE capabilities definition as below in R18 FR1 RF enhancement WI maintenance:

- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Issue 1-1-2: 8Rx UE MMSE-IRC receiver coverage

Agreement:

- Criteria for decision if one or two sets of PDSCH requirements are to be introduced:
- Secure the span is <=2.5 dB for each receiver type by removal of outliers
- If difference in the average of each receiver type for a specific configuration is > [TBD:limit] then define two sets of requirements for the specific configuration.

- Further discuss the actual value for the [limit].

  - Proposals:

    - Option 1:
Set [limit] to 0.5dB

    - Option 2:
Set [limit] to 1.0dB

    - Option 3:
Set [limit] to 1.2dB

    - Other options not precluded.

Based on our simulation results shown in R4-2503376, for ICI scenario, the span between baseline 8Rx receiver and simplified 8Rx receiver is about 2.08-4.32 dB. For IUI scenario, the span between baseline 8Rx receiver and simplified 8Rx receiver is about 3.18 dB.

To define two sets of requirements, there should be valid SNR requirements and obvious SNR span between baseline receiver and simplified receiver. We think 1dB can be the SNR gap to differentiate two different receiver implementations.

- Proposal 2: If the difference in the average of each receiver type for a specific configuration is larger than 1dB, then define two sets of requirements for two receiver implementations.

  - PDSCH demodulation requirements with MMSE-IRC receiver for intra-cell inter-user interference scenario

Issue 1-3-2: Rank 4+4

Proposals:

- Option 1: 
Do not define Rank 4+4 requirements.

- Option 2: 
Define requirements for Rank 4+4 MCS 13 for baseline receiver.

From our simulation result 15.86dB and the current simulation results collection table, we can see that the SNR range for Rank 4+4 MCS 13 baseline receiver is testable. Therefore, we prefer to define requirements for Rank 4+4 for baseline receiver.

Proposal 3: Define requirements for Rank 4+4 MCS 13 for baseline receiver.

  - Applicability rule and release independent

Issue 1-5-1: Test applicability to skip 2/4Rx requirements

Proposals:

- Option 1: 
For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

- Option 2: 
For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.
TBD: Clarification of what “corresponding 8Rx requirements” covers.

For demodulation test of inter-cell interference scenario, the test configuration difference between 8Rx and 2/4Rx tests are as follows:

|  |  | 2Rx | 4Rx | 8Rx |
| --- | --- | --- | --- | --- |
| Rank | HomNet | 1 | 1 | 2 |
|  | HetNet | 1 | 1 | 4 |
| MCS | HomNet | MCS 13 | MCS 13 | MCS 17 |
|  | HetNet | MCS 13 | MCS 13 | MCS 13 |
| Propagation condition | HomNet | TDLC300-100 | TDLC300-100 | TDLA30-10 |
|  | HetNet | TDLA30-10 | TDLA30-10 | TDLA30-10 |

For demodulation test of intra-cell inter-user interference scenario, the test configuration difference between 8Rx and 2/4Rx tests are mainly as follows:

|  | 2Rx | 4Rx | 8Rx |
| --- | --- | --- | --- |
| Rank | 1+1 | 1+1 2+2 | 2+2 4+4 |
| MCS | MCS 13 | MCS 13 | MCS 17 MCS 13 |
| Propagation condition | TDLC300-100 | TDLC300-100 TDLA30-10 | TDLA30-10 |

From the above tables, we observed that except for propagation condition, other test configurations for 8Rx give a higher test requirement compared with 2Rx and 4Rx. With this observation, for demodulation test, we are fine to introduce a test applicability rule that if 8Rx tests are passed, then the test of 2/4Rx requirements can be skipped.

- Proposal 4: Introduce a test applicability rule that if 8Rx tests are passed, then the test of 2/4Rx requirements can be skipped.

Issue 1-5-3: Release independency

Proposals:

- Option 1: 
Discuss release independence requirements at a later stage in the WI.

- Option 2: 
The new Rel-19 requirements for 8Rx should be release independent from Rel-17.

- Option 3: 
The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

- Option 4: 
For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18.

- Option 5: 
The requirement should at least be released independent from Rel-18 if no new signalling or UE capability introduced.

As specified in TS 38.307, the Demodulation requirements for 8Rx UE is mandatory release independent from Rel-17.

Since no new signalling, UE capability, UE receiver implementation will be introduced, for the new demodulation requirements introduced from R19, we also support to release independent from R17.

- Proposal 5: The new Rel-19 requirements for 8Rx should be release independent from Rel-17.

- Conclusion

In this contribution, we provides some test parameter and test rule configuration for UE 8Rx interference suppressing requirement. The proposals are:

Proposal 1: Update the existing Rel-18 UE capabilities definition as below in R18 FR1 RF enhancement WI maintenance:

- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Proposal 2: If the difference in the average of each receiver type for a specific configuration is larger than 1dB, then define two sets of requirements for two receiver implementations.

- Proposal 3: Define requirements for Rank 4+4 MCS 13 for baseline receiver.

- Proposal 4: Introduce a test applicability rule that if 8Rx tests are passed, then the test of 2/4Rx requirements can be skipped.

- Proposal 5: The new Rel-19 requirements for 8Rx should be release independent from Rel-17.

- Reference

[1] R4-2502275, Way forward for [114][324] NR_demod_Ph5_Part2_UE, Moderator (Nokia)