3GPP TSG-RAN WG4 Meeting #114bis	    	R4-2503379

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.18.3

Source: 	CMCC

Title: 		    Discussion on BS demodulation requirements for interference suppressing with MMSE-IRC receiver

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed test scenario and simulation assumptions for BS with MMSE-IRC receiver, the WF has been approved in [1]. In this contribution, we further discuss the remaining open test parameters.

- Discussion

  - Interference modelling

Number of interferers for 2Rx

  - For HetNet scenario, introduce BS MMSE-IRC requirements with 2Rx.

  - For the number of interferers for 2Rx under HetNet, further decide between the following:

- Option 1: 2 interferers

- Option 2: 1 interferer

  - For HomNet scenario, further decide among the following:

- Option 1: 2 interferers

- Option 2: 1 interferer

- Option 3: Not to consider 2Rx requirements for HomNet scenario.

  - Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting.

Based on our simulation results, for 2Rx, both 1 or 2 interferes scenarios have performance gain between IRC and MRC.

| 1T2R FDD 5MHz | Gain of HomNet (dB) | Gain of HetNet (dB) |
| --- | --- | --- |
| with 1 interfere (CP-OFDM) | 1.42 - 1.8 | 3.22 - 3.62 |
| with 2 interferes (CP-OFDM) | 1.68 - 2.23 | 3.38 - 4.41 |

| 1T2R TDD 10MHz | Gain of HomNet (dB) | Gain of HetNet (dB) |
| --- | --- | --- |
| with 1 interfere (CP-OFDM) | 1.3 - 1.49 | 2.79 - 3.08 |
| with 2 interferes (CP-OFDM) | 1.62 - 1.64 | 2.74 - 3.2 |

From the point of decreasing the simulation and test effort and ensuring sufficient performance gain, we can compromise to consider the cases of 2 interferes for HomNet and 1 interfere for HetNet.

Proposal 1: For 2Rx, consider 2 interferes for HomNet and 1 interfere for HetNet.

  - Test parameters and simulation assumptions

Propagation condition for the serving cell

- Select between the following options:

  - Option 1: TDLA30-10 for HetNet and TDLB100-400 for HomNet

  - Option 2: TDLC300-100 for all cases

  - Option 3: TDLA30-10 for HetNet and TDLC300-100 for HomNet (Proposed during RAN4#114 ad-hoc session as a compromise between option 1 and option 2)

- Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting.

Based on our simulation results, for HetNet, both TDLA30-10 and TDLC300-100 have enough performance gain between MMSE-IRC and MRC, the gain from TDLA30-10 is little bit larger than TDLC300-100. Considering TDLA30-10 is more align with the propagation condition in HetNet scenario, we prefer TDLA30-10 for HetNet.

| 1T2R FDD 5MHz | Gain of HetNet (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1T2R 1 interference | 1T2R 2 interference | 1T4R | 1T8R |
| TDLA30-10 | 3.62 | 3.73 | 5.83 | 6.57 |
| TDLC300-100 | 3.45 | 3.38 | 5.65 | 6.46 |

| 1T2R TDD 10MHz | Gain of HetNet (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1T2R 1 interference | 1T2R 2 interference | 1T4R | 1T8R |
| TDLA30-10 | 3.08 | 3.2 | 5.53 | 6.42 |
| TDLC300-100 | 2.79 | 2.74 | 5.17 | 6.72 |

For HomNet, the performance gain between MMSE-IRC and MRC from TDLB100-400 is little bit larger than TDLC300-100. However, considering TDLC300-100 is more common in NR test cases, we slightly prefer TDLC300-100 for HomNet.

| 1T2R FDD 5MHz | Gain of HomNet (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1T2R 1 interference | 1T2R 2 interference | 1T4R | 1T8R |
| TDLB100-400 | 1.8 | 2.23 | 3.43 | 3.48 |
| TDLC300-100 | 1.42 | 1.68 | 3.13 | 3.17 |

| 1T2R TDD 10MHz | Gain of HomNet (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1T2R 1 interference | 1T2R 2 interference | 1T4R | 1T8R |
| TDLB100-400 | 1.49 | 1.64 | 3.02 | 3.3 |
| TDLC300-100 | 1.3 | 1.62 | 2.87 | 3.29 |

Proposal 2: Use TDLA30-10 for HetNet.

Proposal 3: Slightly prefer to use TDLC300-100 for HomNet.

  - Manufacturer declarations and applicability rules

Manufacturer declaration on MMSE-IRC receiver

- Candidate options:

  - Proposal 1: Introduce new manufactory declaration for BS MMSE-IRC receiver

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

  - Proposal 2: Do not need to introduce a new Manufacturer Declaration on MMSE-IRC

As we commented in last meeting, we think the MMSE-IRC receiver has been widely deployed, we can treat it as the baseline receiver. Therefore, we also think there is no need to introduce a new Manufacturer Declaration on MMSE-IRC.

Proposal 4: No need to introduce a new Manufacturer Declaration on MMSE-IRC.

Applicability

  - Candidate options on test requirement applicability for different BS classes and test scenarios:

- Proposal 1: HomNet requirements are not applicable for Local Area BS and HetNet requirements are not applicable for Wide and Medium Area BS

- Other options are not precluded.

  - Based on test requirement applicability, further discuss whether to consider additional applicability rule on how the test is performed:

- Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped

- Proposal 2: If a BS declared to support HomNet or HetNet and pass the tests for HomNet, can also consider the test for HetNet as passed

- Proposal 3: Do not define additional applicability rule for homogenous and heterogenous network scenarios

In LTE, the test requirements of Homogeneous network are not applicable for Local Area BS and Home BS. We think it is rational to follow similar approach in NR.

In 38.141-1 Clause 4.6, BS class manufactory declaration is as follows:

| D.2 | BS class | Declared as Wide Area BS, Medium Range BS, or Local Area BS. |
| --- | --- | --- |

For Wide Area BS, we think the test requirements of Homogeneous network shall be applicable, while for Medium Range BS and Local Area BS, the test of Homogeneous network can be skipped. In other words, the Wide Area BS has to pass both HomNet and HetNet test cases, and the Medium Range BS and Local Area BS have to pass HetNet test cases.

Proposal 5: Introduce the applicability rule that the test requirements of Homogeneous network are not applicable for Medium Range BS and Local Area BS.

- Conclusion

In this contribution, we provides some basic simulation assumptions for BS 8Rx interference suppressing requirement. The proposals are:

Proposal 1: For 2Rx, consider 2 interferes for HomNet and 1 interfere for HetNet.

Proposal 2: Use TDLA30-10 for HetNet.

Proposal 3: Slightly prefer to use TDLC300-100 for HomNet.

Proposal 4: No need to introduce a new Manufacturer Declaration on MMSE-IRC.

Proposal 5: Introduce the applicability rule that the test requirements of Homogeneous network are not applicable for Medium Range BS and Local Area BS.

- Reference

[1] R4-2502272, Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS, China Telecom