3GPP TSG-RAN WG4 Meeting # 114bis		R4-2503380

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.25.2.1

Source: 	CMCC

Title: 			  Discussion on RRM requirement of OD-SSB for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed measurement procedures and requirement definition about OD-SSB in NES enhancement WI. The WF has been approved in [1], the details of the open issues are captured in the moderator summary [2]. In this paper, we further discuss OD-SSB related open issues.

- OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

Sub-issue 1-1 – The length of FMW

Agreement:

- The time window is defined with Tuncertain plus actual measurement duration.

  - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows.

  - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time.

    - Treport is the first OD-SSB based L3 measurement reporting

    - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow.

      - A detected cell: OD-SSB based measurement period

      - A newly detectable cell: OD-SSB based cell identification period

- No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify).

Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs?

Agreement:

RAN4 to define measurement requirement based on T2.

- When NW re-activates the OD-SSB within the T2,

  - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

  - Option 2: UE is required to perform “fast measurement” without cell search.

    - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition.

  - Option 3: UE is required to perform FMW after T2.

- When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW

  - The UE is assumed to meet fast cell identification and measurement.

- The start point of T2 is based on

  - Option 1: the time point Tend

  - Option 2: the latter point of L3 measurement reporting for FMW and Tend

  - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend

  - Option 4: the time point of OD-SSB deactivation

  - Other options are not precluded

Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3.

- The duration is

  - Option 1: 5*max(measCycleSCell,  DRX cycles) [– OD-SSB periodicity]

  - Option 2: 5* measCycle when measCycle =160 ms

  - Other options are not precluded

- The activation and deactivation of OD-SSB is fully up to NW.

Agreement from RAN1

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

In last meeting, the length of FMW has been defined, leftovers are the additional processing/preparation time in Tuncertain and how to differentiate the detected cell and the newly detectable cell.

Regarding the additional processing/preparation time, in our point of view, it is for re-allocating the measurement searcher resource.

If OD-SSB carrier is always prioritized, i.e., CSSF=1, then UE should re-allocate it’s measurement searcher resource as long as OD-SSB is triggered.

If OD-SSB carrier has same priority as legacy SSB carriers, then UE should re-allocate it’s measurement searcher resource for following two cases, and for other cases, we think the additional processing time is not necessary.

- In Case 1 that no AO-SSB, due to the number of frequency carriers to be measured will be changed, UE should re-allocate it’s measurement searcher resource.

- In Case 2 that AO-SSB is transmitted, if OD-SSB and AO-SSB are within different frequencies and UE is expected to perform measurement on both OD-SSB and AO-SSB, then additional processing time may be needed.

- Based on the above agreement from RAN1, when AO-SSB and OD-SSB are on different frequencies, UE is not required to measure both AO-SSB and OD-SSB, so the second case is invalid.

Proposal 1: Only for the following cases if valid, additional processing/preparation time may be needed for UE to measure the OD-SSB:

- When OD-SSB carrier is always prioritized.

- When OD-SSB carrier has same priority as legacy SSB carriers and

  - No AO-SSB on the SCell (Case 1)

Regarding how long the additional processing time will take, considering the measurement resource re-allocation updating scheme is not complicated, we think 1ms can be the starting point.

Proposal 2: For the value of additional processing/preparation time, take 1ms as the starting point.

Regarding how to differentiate the detected cell and the newly detectable cell, similar as the legacy method, the definition can be based on T2, which starting point is the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend

- The cell is the detected cell if T2 is less than [5]s, and the subsequent OD-SSB is transmitted with the same spatial reception parameter, same transmission power and on the same frequency carrier as the OD-SSB used for the last valid measurement report, and the timing to this cell has not changed more than  3200/2µ Tc while L3 filtering has not been used, when L3 filtering is used, an additional delay can be expected.  The UE is assumed to perform fast mode measurement without cell identification period.

- The cell is the newly detectable cell if T2 is larger than [5]s. The UE is assumed to perform fast mode measurement with cell identification period.

Proposal 3: The starting point of T2 is the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend, and

- The cell is the detected cell if T2 is less than [5]s, and the subsequent OD-SSB is transmitted with the same spatial reception parameter, same transmission power and on the same frequency carrier as the OD-SSB used for the last valid measurement report, and the timing to this cell has not changed more than  3200/2µ Tc while L3 filtering has not been used, when L3 filtering is used, an additional delay can be expected.  The UE is assumed to perform fast mode measurement without cell identification period.

- The cell is the newly detectable cell if T2 is larger than [5]s. The UE is assumed to perform fast mode measurement with cell identification period.

We also understand there is concern from UE vendor than open FMW densely have power consumption issue. However, considering there is no reason to trigger multiple OD-SSB burst densely in order to guarantee network energy saving, and typically only limited OD-SSB will be transmitted before SCell activation, our proposal based on as long as network indicate/re-indicate OD-SSB transmission, UE need to start the fast mode measurement. We are open to further discuss if there is still strong concern from UE side.

Observation 1: In order to guarantee network energy saving, network has no reason to trigger multiple OD-SSB burst densely, and typically only limited OD-SSB will be transmitted before SCell activation.

- OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

Agreement:

When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2),

- Option 1: The same approach in Case 1 can be reused.

  - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell).

- Option 2:

  - If the serving cell is unknown, the same approach in Case 1 can be reused.

  - If the serving cell is known, UE follows legacy requirements based on measCycleSCell.

    - The known condition reuses the principle of legacy known condition.

    - This scenario applies at least when UE sends the meas. reporting.

  - FFS the case if the L3 meas. report cannot be triggered.

- The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

Since Case#2 and Scenario#2 has been identified as a valid scenario, then the OD-SSB shall be used by UE when triggered by network, even if the serving cell is in known state. One application scenario is network configure long measCycleSCell for UE to tracking AO-SSB with low power consumption, and configure OD-SSB for fast SCell activation determination.

Therefore, we support Option 1, that same approach in Case 1 shall be used for Case 2.

Proposal 4: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), the same approach in Case 1 should be reused.

Issue 2-2: Always-on SSB and OD-SSB time/frequency domain relation

Agreement:

RAN4 to start from the following scenarios:

- Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

- Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities.

  - At least the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity

- The scenarios can be updated based on other WGs’ progress.

Issue 2-3: Combine OD-SSB and AO-SSB in measurement

Agreement:

- In scenario 4, the measurement requirement is defined based on the effective measurement periodicity.

  - The requirement is only applied in the FMW which includes both OD-SSB and AO-SSB, and

  - The effective measurement periodicity is half of OD-SSB periodicity

  - AO-SSB and OD-SSB are on the same frequency with the same spatial relation

Agreement from RAN1

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB,

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - From RAN1 perspective,

    - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification).

    - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

  - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

  - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB

    - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB

- For the case when the center frequency locations of always-on SSB and on-demand SSB are different,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain.

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB

- NOTE: AO-SSB periodicity is not adapted

Based on the agreement from RAN1, besides Scenario 1 and Scenario 4 when the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity, Scenario 2,Scenario 3 and Scenario 4 when the offset between OD-SSB and AO-SSB doesn’t equal to the half of AO-SSB periodicity are also valid scenarios.

Observation 2: Following scenarios are also valid based on RAN1’s agreement

- Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities.

- Scenario 3: OD-SSB and always-on SSB are within different frequencies.

- Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities

  - The offset between OD-SSB and AO-SSB doesn’t equal to the half of AO-SSB periodicity

- Firstly, RAN1 has pointed out that when AO-SSB and OD-SSB are on different frequencies, UE is not required to measure both AO-SSB and OD-SSB.

- Secondly, for Scenario 2 and Scenario 4 when the offset between OD-SSB and AO-SSB doesn’t equal to the half of AO-SSB periodicity, the AO-SSB and OD-SSB are unevenly distributed in the time domain. It is hard for RAN4 to define a unified requirement for all cases. Therefore, we propose not defining related requirements.

Proposal 5: For the following scenarios, no need to define the requirement based on the combination of the OD-SSB and AO-SSB

- Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities.

- Scenario 3: OD-SSB and always-on SSB are within different frequencies.

- Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities

  - The offset between OD-SSB and AO-SSB doesn’t equal to the half of AO-SSB periodicity

- OD-SSB based deactivated SCell L3 measurement requirement in FMW (Case 1)

Sub-issue 3-1: Lower bound for measurement in FMW

Wayforward:

- Option 1: RAN4 not to define lower bound to measurement in FMW.

- Option 2: Define a lower bound with OD-SSB periodicity as [10ms].

  - The lower bound is only applied to measurement in FMW.

Considering UE is not required to continuously maintain the fast measurement mode, we don’t think the lower bound scheme can help UE with saving power. Besides, only extending the OD-SSB periodicity has limited benefit for ensuring the L3 filter gain, but increase the power consumption of network side. Therefore, not to define lower bound to OD-SSB periodicity in FMW is fine for us.

Proposal 6: Don’t define lower bound to OD-SSB periodicity in FMW.

Sub-issue 3-2: OD-SSB measurement in FMW(Case 1-1): single OD-SSB carrier with multiple legacy SSB carriers

Agreement:

When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW

- Option 1: The CSSF only counts OD-SSB layers

- Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers

In legacy, the searcher allocation takes frequency carrier priority into consideration, for example, the CSSF for PCC doesn’t count other SCC and inter-frequency carriers. In this aspect, we think the legacy scheme should be reused, that OD-SSB carrier shall have same priority as other SCCs and inter-frequency carriers. However, such scheme may cause large CSSF for OD-SSB carrier, furthermore, longer FMW and larger network power consumption. To find a balance between network power saving and frequency carrier priorities, we are fine to prioritize the OD-SSB carrier with the consideration of the fast measurement window is quite short with OD-SSB periodicity based requirement definition, due to no significant delay impact for other layers.

Proposal 7: The CSSF only counts OD-SSB layers provided that the length of fast measurement window is quite short, e.g., no lower bound to OD-SSB periodicity or Tidentify.

- OD-SSB general issues

Issue 4-2: L1 measurement after SCell activation

Agreement:

- RAN4 to support OD-SSB L1 measurement during and after SCell activation.

  - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB

- RAN4 to support OD-SSB L1 measurement during SCell activation.

  - The requirement when DRX is configured.

    - Based on OD-SSB periodicity

Wayforward:

- RAN4 to support OD-SSB L1 measurement after SCell activation.

  - FFS the requirement when DRX is configured.

    - Option 1: Follow legacy, based on DRX cycle (vivo, Apple, OPPO, Xiaomi)

    - Option 2: Based on OD-SSB periodicity (Nokia, ZTE, Samsung)

    - Option 3: FFS (HW, QC, Nokia, CMCC, Xiaomi, CATT)

    - Note: the OD-SSB is triggered before or together with the Scell activation command, the OD-SSB transmission kept after the SCell activation until the OD-SSB deactivation command received.

Since the Scenario 3B has not been involved based on the agreement of RAN1, here we are talking about Scenario 2 and Scenario 2A, that OD-SSB is triggered before or together with the SCell activation command, and keeps transmission after SCell activation.

Considering the OD-SSB for Scenario 2 and Scenario 2A is mainly for fast SCell activation, we don’t think UE shall keep fast measurement even after SCell activation. Therefore, we support Option 1.

Proposal 8: For Scenario 2 and Scenario 2a, when DRX is configured, the requirement of OD-SSB L1 measurement should follow the legacy form that based on DRX cycle. FFS for Scenario 3B.

- OD-SSB based SCell activation

Issue 1-4-1: The known cell condition

Agreement from RAN1

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2.

  - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

    - FFS: Whether this parameter can be absent for Case #2

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst)

    - FFS: Whether this parameter can be absent for Case #2

  - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity)

  - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing)

    - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB.

  - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId)

    - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

  - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower)

    - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx)

  - FFS: Which of the above parameters are a list

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

- Based on the agreement from RAN1, the relationship between AO-SSB and OD-SSB is like:

- When AO-SSB is NCD-SSB:

- The frequency location of AO-SSB and OD-SSB is same

- The spatial relation of AO-SSB and OD-SSB is same

  - the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported

  - FFS whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst

- The transmit power of AO-SSB and OD-SSB is same

- When AO-SSB is CD-SSB:

- The frequency location of AO-SSB and OD-SSB is different

- FFS additional conditions

- First of all, we think the known/unknown condition can be defined based on either AO-SSB or OD-SSB.

- Secondly, the SSB measured for known cell condition and the SSB used in SCell activation procedure can be same or different, and the SCell activation delay requirements shall be defined case by case.

- If the SSB measured for known cell condition and the SSB used in SCell activation procedure are same, then the current known SCell activation delay methodology can be reused.

- If the SSB measured for known cell condition is always-on SSB not on sync-raster, and the SSB used in SCell activation procedure is OD-SSB, the current known SCell activation delay methodology can be reused.

- If the SSB measured for known cell condition is OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, further study whether the current known SCell activation delay methodology can be reused or not, based on RAN1’s further input of whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

- If the SSB measured for known cell condition is always-on SSB on sync-raster, and the SSB used in SCell activation procedure is OD-SSB, due to the frequency location is different, at least the AGC adjustment time should be included in the known SCell activation delay requirement, and whether other components are needed or not shall wait for RAN1’s further agreements of additional conditions.

Proposal 9: The SSB measured for known cell condition and the SSB used in SCell activation procedure can be same or different:

- If the SSB measured for known cell condition and the SSB used in SCell activation procedure are same, then the current known SCell activation delay methodology can be reused.

- If the SSB measured for known cell condition is always-on SSB not on sync-raster, and the SSB used in SCell activation procedure is OD-SSB, the current known SCell activation delay methodology can be reused.

- If the SSB measured for known cell condition is OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, further study whether the current known SCell activation delay methodology can be reused or not, based on RAN1’s further input of whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

- If the SSB measured for known cell condition is always-on SSB on sync-raster, and the SSB used in SCell activation procedure is OD-SSB, due to the frequency location is different, at least the AGC adjustment time should be included in the known SCell activation delay requirement, and whether other components are needed or not shall wait for RAN1’s further agreements of additional conditions.

Issue 1-4-2: The known cell condition – period of known condition

About the period of the known/unknown condition, in legacy FR1, the known case requires UE to sent a valid measurement report during a period before the reception of the SCell activation command, the period equal to max(5*measCycleSCell,  5*DRX cycles), which related to the legacy L3 measurement requirement.

In other words, as long as UE reported the measurement report during max(5*measCycleSCell,  5*DRX cycles) before the SCell activation command, UE could maintain the known states of this deactivated SCell. From this point of view, we think this known/unknown condition can be reused even for OD-SSB case. We illustrate the cases in the figure below.

Proposal 10: Reuse the legacy known/unknown cell condition for both FR1 and FR2.

- Other OD-SSB measurement issues

Issue 7-3: OD-SSB based L3 reporting

Wayforward:

- RAN4 to discuss whether to introduce an OD-SSB based L3 measurement reporting.

Firstly, we should discuss whether the current event triggered measurement reporting mechanism can be reused for OD-SSB based measurement. If yes, then we don’t think an additional reporting mechanism is needed.

For Case#1 and Case#2 that AO-SSB is NCD-SSB, we think the current event triggered measurement reporting is still valid. Network configure the serving cell MO based on the OD-SSB/AO-SSB carrier frequency, and the Event A1 can be used for reporting with a relative low threshold.

For the Case#2 that AO-SSB is CD-SSB, how network configure the serving cell MO shall be further discussed. We suggest FFS for this case.

Proposal 11: Reuse current event triggered measurement reporting mechanism for Case#1 and Case#2 that AO-SSB is NCD-SSB. FFS for Case#2 that AO-SSB is CD-SSB.

- Conclusion

Based on the discussion above, the following proposals and observations are concluded.

Proposal 1: Only for the following cases if valid, additional processing/preparation time may be needed for UE to measure the OD-SSB:

- When OD-SSB carrier is always prioritized.

- When OD-SSB carrier has same priority as legacy SSB carriers and

  - No AO-SSB on the SCell (Case 1)

Proposal 2: For the value of additional processing/preparation time, take 1ms as the starting point.

Proposal 3: The starting point of T2 is the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend, and

- The cell is the detected cell if T2 is less than [5]s, and the subsequent OD-SSB is transmitted with the same spatial reception parameter, same transmission power and on the same frequency carrier as the OD-SSB used for the last valid measurement report, and the timing to this cell has not changed more than  3200/2µ Tc while L3 filtering has not been used, when L3 filtering is used, an additional delay can be expected.  The UE is assumed to perform fast mode measurement without cell identification period.

- The cell is the newly detectable cell if T2 is larger than [5]s. The UE is assumed to perform fast mode measurement with cell identification period.

Observation 1: In order to guarantee network energy saving, network has no reason to trigger multiple OD-SSB burst densely, and typically only limited OD-SSB will be transmitted before SCell activation.

Proposal 4: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), the same approach in Case 1 should be reused.

Observation 2: Following scenarios are also valid based on RAN1’s agreement

- Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities.

- Scenario 3: OD-SSB and always-on SSB are within different frequencies.

- Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities

  - The offset between OD-SSB and AO-SSB doesn’t equal to the half of AO-SSB periodicity

Proposal 5: For the following scenarios, no need to define the requirement based on the combination of the OD-SSB and AO-SSB

- Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities.

- Scenario 3: OD-SSB and always-on SSB are within different frequencies.

- Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities

  - The offset between OD-SSB and AO-SSB doesn’t equal to the half of AO-SSB periodicity

Proposal 6: Don’t define lower bound to OD-SSB periodicity in FMW.

Proposal 7: The CSSF only counts OD-SSB layers provided that the length of fast measurement window is quite short, e.g., no lower bound to OD-SSB periodicity or Tidentify.

Proposal 8: For Scenario 2 and Scenario 2a, when DRX is configured, the requirement of OD-SSB L1 measurement should follow the legacy form that based on DRX cycle. FFS for Scenario 3B.

Proposal 9: The SSB measured for known cell condition and the SSB used in SCell activation procedure can be same or different:

- If the SSB measured for known cell condition and the SSB used in SCell activation procedure are same, then the current known SCell activation delay methodology can be reused.

- If the SSB measured for known cell condition is always-on SSB not on sync-raster, and the SSB used in SCell activation procedure is OD-SSB, the current known SCell activation delay methodology can be reused.

- If the SSB measured for known cell condition is OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, further study whether the current known SCell activation delay methodology can be reused or not, based on RAN1’s further input of whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

- If the SSB measured for known cell condition is always-on SSB on sync-raster, and the SSB used in SCell activation procedure is OD-SSB, due to the frequency location is different, at least the AGC adjustment time should be included in the known SCell activation delay requirement, and whether other components are needed or not shall wait for RAN1’s further agreements of additional conditions.

Proposal 10: Reuse the legacy known/unknown cell condition for both FR1 and FR2.

Proposal 11: Reuse current event triggered measurement reporting mechanism for Case#1 and Case#2 that AO-SSB is NCD-SSB. FFS for Case#2 that AO-SSB is CD-SSB.

- Reference

- R4-2502598, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson

- R4-2500537, Topic summary for [114][223] Netw_Energy_NR_enh_Part1, Moderator (Ericsson)