3GPP TSG-RAN WG4 Meeting #114bis		R4-2503381

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.25.2.2

Source: 	CMCC

Title: 			  Discussion on RRM requirement of signal adaptation for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed SSB signal adaptation related issues, agreements and open issues have been approved in the WF [1]. In this paper, we further discuss open issues, and identify RAN4 impacts according to the progress of other working group.

- Discussion

Issue 1-3-1: L3 requirements for SSB adaptation – general

Agreement:

Deactivated SCell measurement:

- Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements.

  - At least for the case when the SSB periodicity is adapted to be shorter than before

- Option 2: Same as legacy

- Other options are not precluded.

The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

Reply LS from RAN2#129 [2]

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

Considering the SMTC framework will be reused, and SSB adaption usually is not a short-term adaptation, we don’t suggest to reuse the same conclusion from OD-SSB.

The legacy framework can be reused, that when SMTC configuration changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition.

Proposal 1: For L3 requirements, reuse the legacy transition framework, that when SMTC configuration changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition.

Issue 1-4-1: SCell activation requirement for SSB adaptation

Agreement:

For SCell activation:

- Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - For the case SSB adaptation command comes together with SCell activation command.

- The requirement can be updated based on further progress in other WG

Agreements from RAN1

For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell

- UE is configured with SSB burst periodicity using legacy signalling for the SCell

- UE is configured with X additional SSB burst periodicity for the SCell

  - FFS: Value of X

- SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity

  - FFS: Whether there is specification impact

  - Note: This does not impact the discussion on OD-SSB

- For switching the periodicity, down-select between

  - (MAC-CE)

    - MAC-CE details for SSB burst periodicity adaptation is up to RAN2.

  - (DCI)

    - Alt 1-3: DCI based signalling is used

Agreement

For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell

- Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI]

- FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1)

Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

Based on the RAN1’s input, the SSB adaptation signal can be RRC, MAC-CE or Group common DCI, therefore, it is hard to determine that the SSB adaptation command and SCell activation command come together or not. Therefore, we propose to refine the agreement as:

For SCell activation:

- Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - For the case SSB adaptation command comes not later than with SCell activation command

- Proposal 2: Refine the agreement from last meeting:

For SCell activation:

- Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - For the case SSB adaptation command comes not later than with SCell activation command

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: For L3 requirements, reuse the legacy transition framework, that when SMTC configuration changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition.

- Proposal 2: Refine the agreement from last meeting:

For SCell activation:

- Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - For the case SSB adaptation command comes not later than with SCell activation command

- Reference

- R4-2502689, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei

- R2-2501484, Reply LS on SSB adaptation, RAN2