3GPP TSG-RAN WG4 Meeting # 114bis		R4-2503383

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.26.4.1

Source: 	CMCC

Title: 			  Simulation assumption and results for LP-WUR

Document for:	Discussion

- Introduction

In RAN4#114 meeting, RAN4 provided the updated RRM simulation assumption. The WF has been approved in [1].

In this contribution, we give our simulation results based on RAN4 simulation assumption.

- Simulation result

Our simulation results are provided below by using the simulation assumption in Annex.1.

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  | SINR(dB) | channel | Interference Source | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | X = 2 |  |  |  |  |  |  |  |
|  |  | mean | std | ΔRSRP | ΔRSRQ |  |  |  |  |
| {1，4} | S: [0101] I: [0110] | -0.25 | 0.73 | 0.9 | 0.46 | -3 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {1，6} | S: [101010] I: [010101] | -0.78 | 0.65 | -0.11 | -0.18 | -3 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {1，8} | S:[10100101] I:[10101001] | 0.76 | 0.51 | 1.69 | 1.05 | -3 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {2，8} | S:[01011001] I: [01100101] | -0.31 | 0.41 | 0.3 | 0.24 | -3 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {2，16} | S:[1001010110011001] I:[1001100101100101] | -0.25 | 0.23 | 0.11 | 0.1 | -3 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {4，16} | S:[0110100110101010] I:[0110101010011010] | 0.6 | 0.56 | 1.6 | 1.03 | -3 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {4，32} | S:[01011010101010011010011001100101] I:[01100101011001011001101010100101] | 0.12 | 0.15 | 0.36 | 0.34 | -3 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {1，4} | S: [0101] I: [0110] | 0.07 | 0.55 | 0.91 | 0.65 | -6 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {1，6} | S: [101010] I: [010101] | -2.73 | 3.02 | -0.19 | -0.27 | -6 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {1，8} | S: [10100101] I:[10101001] | 0.51 | 0.36 | 1.14 | 0.76 | -6 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {2，8} | S:[01011001] I: [01100101] | 0.07 | 0.34 | 0.61 | 0.47 | -6 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {2，16} | S:[1001010110011001] I:[1001100101100101] | -0.03 | 0.21 | 0.3 | 0.23 | -6 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {4，16} | S:[0110100110101010] I:[0110101010011010] | 0.6 | 0.34 | 1.19 | 0.87 | -6 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {4，32} | S:[01011010101010011010011001100101] I:[01100101011001011001101010100101] | 0.02 | 0.39 | 0.63 | 0.48 | -6 | TDL-C 300ns | 6 dB lower compared with cell 1 |  |
| {1，4} | S: [0101] I: [0110] | -0.05 | 0.24 | 0.33 | 0.15 | -3 | AWGN | 6 dB lower compared with cell 1 |  |
| {1，6} | S: [101010] I: [010101] | -1.31 | 0.26 | -0.9 | -1 | -3 | AWGN | 6 dB lower compared with cell 1 |  |
| {1，8} | S: [10100101] I:[10101001] | 0.39 | 0.14 | 0.64 | 0.51 | -3 | AWGN | 6 dB lower compared with cell 1 |  |
| {2，8} | S:[01011001] I: [01100101] | -0.6 | 0.3 | -0.12 | -0.29 | -3 | AWGN | 6 dB lower compared with cell 1 |  |
| {2，16} | S:[1001010110011001] I:[1001100101100101] | -0.45 | 0.2 | -0.13 | -0.25 | -3 | AWGN | 6 dB lower compared with cell 1 |  |
| {4，16} | S:[0110100110101010] I:[0110101010011010] | 0.59 | 0.21 | 0.93 | 0.77 | -3 | AWGN | 6 dB lower compared with cell 1 |  |
| {4，32} | S:[01011010101010011010011001100101] I:[01100101011001011001101010100101] | 0.05 | 0.18 | 0.33 | 0.21 | -3 | AWGN | 6 dB lower compared with cell 1 |  |

- Discussion

After updating the ideal RSRP/RSRQ calculation methodology, from our simulation results, 2 samples can fulfill the ± 3.5dB accuracy requirement.

Observation 1: For OOK based LP-WUR, 2 samples can fulfill the accuracy requirement of ±3.5dB for LP-RSRP and ±3.5dB for LP-RSRQ.

- Conclusion

Based on the discussion above, the following observations are concluded.

Observation 1: For OOK based LP-WUR, 2 samples can fulfill the accuracy requirement of ±3.5dB for LP-RSRP and ±3.5dB for LP-RSRQ.

- Reference

- R4-2502600, WF on RRM requirements for NR_LPWUS, vivo

- Annex.A

Simulation assumption

Note: Highlight part is updated base on the WF of last meeting

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 2.6 GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | Data, SSB and LP-SS have the same SCS |
| Subcarrier spacing | 30kHz |
| Measurement period (in number of measurement samples) | - [2, 3, 4, 5] |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS initially |
| SSS | 30KHz for SSS |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-4 with M = [1, 2,4] | when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 or NR signal is used for Cell 2 |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [10] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: T=5us for OOK-4 with M=1 T=2us for OOK-4 with M=2 T=1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
|  |  | When Ês/Iot = -6 dB - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-5.7] | When Ês/Iot = -6 dB - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-11.7] |
|  |  | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
| Ês/Iot | dB | -3; -0.5dB; 2dB ; -6dB | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [3th/5th Order Butterworth with 3.96MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4 or 8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [7.68MHz] |