3GPP TSG-RAN WG4 Meeting # 114bis		R4-2503385

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.29.4.1

Source: 	CMCC

Title: 			   Discussion on the RRM requirement of (e)Redcap UE for NTN phase3

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed (e)RedCap UEs over NR FR1-NTN. The WF has been approved in [1].

In this contribution, we further discuss the RRM impact and specific requirements.

- General consideration on RRM requirements

  - What RRM requirements are defined for (e)RedCap UE with FR1-NTN

Issue 1-1-3: MDT requirements

Agreement:

- For MDT requirements in RRC_IDLE and RRC_INACTIVE state,

- Deprioritize to define Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN.

We think it is beneficial to support MDT capability for Redcap UE over NTN.

Firstly, for NTN network with large/moving cell coverage, it is barely impossible to use the traditional drive test, which requires support from both operation and testing equipment, the difficulty and cost of network optimization will increase a lot. While MDT could help network to optimize the scheduling with low cost.

Secondly, for large cell coverage, especially for lonely area, it is hard to assume that network can acquire MDT report from handheld UE, the Redcap UE can serve as a supplement in this case.

Thirdly, the link performance seen between handheld UE and Redcap UE may be different, especially for 1Rx Redcap and HD-FDD Redcap. Therefore, although NTN network has already have MDT report from handheld UE, the MDT report from Redcap UE can still provide more information to network.

Lastly, we understand there is power consumption concern for Redcap UE. However, considering it is up to UE whether to support this capability or not, and UE who support this feature will stop the memory when the cached data over the threshold, and MDT will be started only after receiving the network configuration, we think all these approach can guarantee that the feature will be applied for UE without power consumption issue and only when necessary from network side.

- Proposal 1: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN.

  - Features to be considered for (e)RedCap UE with FR1-NTN

Issue 1-2-3-1: Feature 25-1(parallelSMTC)

<Way Forward>:

Options for further discussion:

If not differentiate 1Rx and 2Rx RedCap UEs, the baseline for UE capability is:

- Option 1: [1 SMTC or] 2 SMTCs not in parallel

- Option 2: 2 parallel SMTCs

  - ‘Parallel’ means the SMTCs are TDM with proximity distance.

Issue 1-2-3-1a: The proximity distance between two SMTCs

Agreement:

- RAN4 to define the proximity distance between two SMTCs for (e)RedCap UEs in NTN.

- The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

- For whether to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier,

  - Option 1: No. RAN4 to reuse the legacy collision condition.

  - Option 2: Yes. RAN4 to consider relaxation for proximity, i.e.,

    - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

      - the two SMTC occasions are fully or partially overlapping in time domain, or

      - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)), x can be FFS. (1ms as starting point)

In R17 NTN, according to the UE capability parallelSMTC-r17, UE is capable of performing RRM measurements in parallel on at least 2 SMTCs with different offsets and same periodicity, optionally on 4 SMTCs, which configured by SSB-MTC4List-r17, which means UE is able to sample in ‘2 or optional 4 SMTCs’ in parallel during one SMTC periodicity.

For Redcap over NTN, since the network scenario is unchanged, RAN4 still need to define RRM requirements for multiple SMTC-s for a single frequency carrier to guarantee at least 2 target satellites including serving LEO satellite and neighbour LEO satellite(s) will be monitored. With enough distance between SMTCs, we think it is feasible for UE to treat them in TDM manner (also called parallel), even for Redcap UE. Therefore, we prefer to follow the legacy that UE is mandatory to support parallel measurements on 2 SMTC-s for a single frequency carrier.

- Proposal 2: For both 1Rx and 2Rx Redcap UE, the baseline of UE capability is that UE is mandatory to support parallel measurements on 2 SMTC-s for a single frequency carrier.

If the SMTCs are overlapped with each other, or within the proximity distance, UE can not support parallel measurements, due to additional processing time for processing the different doppler in each SMTC is needed. Currently, we don’t think this processing time is needed to be relaxed for RedCap UE, legacy collision condition is preferred.

- Proposal 3: Reuse the legacy collision conditions for parallel measurements on multiple SMTCs.

Issue 1-2-3-3: The number of cells/SSBs/supported satellites

<Way Forward>:

- For the number of supported satellites,

  - Proposal 1: For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for RedCap UE.

  - Proposal 2:

    - RedCap UEs shall support at least one NGSO satellite per SMTC configured in a frequency layer in addition to the serving satellite.

    - The total number of supported satellites for measurement in a frequency layer is equal to N+1 where N is the number of configured SMTCs, if the serving satellite is to be measured in this frequency layer and N if the serving satellite is not to be measured in this frequency layer.

- For the number of cells and SSB,

  - Proposal 1: RedCap NTN reuses the number of cells and SSB for NR NTN, i.e.,

    - Proposal 1a: For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

      - 8 identified cells, and

      - 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

      - 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment.

      - In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

- Since the network scenario is unchanged, it is still important to guarantee at least 2 target satellites including serving LEO satellite and neighbour LEO satellite(s) will be monitored.  Therefore, the baseline capability should be 2 satellites per carrier, same as legacy UE capability. For the UE capability of number of cells and SSB, we also think the NR NTN requirement can be reused.

- Proposal 4: For both intra-frequency measurement and inter-frequency measurement, the number of cells and SSBs can be reused, the number of supported satellites and related UE capability can be reused.

Issue 1-2-3-5: Feature 25-3(parallelMeasurementGap)

<Way Forward>:

- For whether to define RRM requirements for Feature 25-3(parallelMeasurementGap-r17) for (e)RedCap UEs,

  - Proposal 1: Support to define.

    - Proposal 1a: The UE capability is applicable, and the support of the optional features is left to UE implementation following the same principle of the existing non-RedCap requirements.

    - Proposal 1b: The capability can be optionally supported by (e)RedCap UEs with 2Rx, can not be supported by (e)RedCap UEs with 1Rx.

- Similar as other UE capabilities, since it is optional feature, whether to support it or not can be left to UE implementation.

- Proposal 5: Optional feature 25-3(parallelMeasurementGap-r17) is applicable for (e)Redcap UE, the requirement definition can follow the same principle of the existing NR NTN requirements.

Issue 1-2-6: NCD-SSB

<Way Forward>:

- Proposal 1:

  - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause that the requirements are only applicable to CD-SSB.

- Proposal 2:

  - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, no need to add additional requirement applicability about CD-SSB/NCD-SSB.

In our perspective, both Proposal 1 and Proposal 2 means that the requirements are defined with the assumption that SSB is CD-SSB. We can go either way.

Proposal 6: No strong view about whether to add additional requirement applicability about CD-SSB/NCD-SSB.

Issue 1-2-7: Other features

<Way Forward>:

- For SMTC adjustment in idle/inactive,

  - Option 1: Keep mandatory for RedCap UE.

  - Option 2: Keep optional for RedCap UE.

- For Location-based measurement trigger,

  - Option 1: Keep optional for RedCap UE.

  - Option 2: RAN4 not to further discuss applicability of location-based measurement report trigger.

For information, the UE capability of SMTC adjustment in IDLE/Inactive is optional.

| 34-11 | SMTC adjustment in idle/inactive | It's optional for UE to perform SMTC adjustment in RRC_IDLE/RRC_INACTIVE | Optional without capability signalling |
| --- | --- | --- | --- |

For the optional NTN features below which have impact on RAN4 requirements, we think they can also be optionally supported by Redcap UE.

- SMTC adjustment in idle/inactive

- Location-based measurement report trigger

Proposal 7: For the optional NTN features which have impact on RAN4 requirements, they can also be optionally supported by Redcap UE.

- SMTC adjustment in idle/inactive

- Location-based measurement report trigger

- Reduction in the number of UE Rx branches

  - RRC_IDLE/RRC_INACTIVE mode mobility

Issue 2-1-1: Enhanced cell reselection measurement requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN

<Way Forward>:

- Proposal 1: Enhanced RRM requirements for measurements in IDLE and INACTIVE modes are kept optional in Redcap over NTN.

  - Proposal 1a: Define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX_IDLE cycle, the requirement for normal UEs can be reused.

  - Proposal 1b: To define enhanced cell reselection requirements for RedCap UE,

    - Re-use the requirements for normal UE except that Tdetect is extended by [2] samples

    - Same delay requirements for 1Rx and 2Rx UEs.

  - Proposal 1c:

    - For 2 Rx (e)RedCap NTN UE, RAN4 to reuse the existing enhanced cell reselection measurement requirements

    - For 1 Rx (e)RedCap NTN UE, RAN4 to

      - Option 1: Reuse the existing enhanced cell reselection measurement requirements

      - Option 2: Consider increasing measurement samples based on the current cell reselection measurement enhanced requirements. The actual number of samples can be further determined.

  - Proposal 1d: For enhanced cell re-selection for (e)RedCap UE with FR1-NTN bands,

    - For 2Rx/1Rx, the Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh are reused the legacy FR1-NTN requirements. In addition, The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.

Firstly, we think the enhanced cell reselection measurement will be defined for RedCap UE not configured with eDRX_IDLE cycle, the requirement will be applied based on UE capability and network signalling. Then, for the delay requirements, we think the enhanced cell reselection measurement requirement for normal UEs in NR NTN can be reused.

Proposal 8: Define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX_IDLE cycle, the requirement for normal UEs can be reused.

Issue 2-1-2: Cell re-selection requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN

<Way Forward>:

- Proposal 1: The existing requirements for measurements of inter-RAT E-UTRAN cells with TN carrier can be used as baseline.

- Proposal 2: For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements, reuse the legacy requirements defined for normal UEs for (e)Redcap UE not configured with eDRX_IDLE cycle, reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2 for (e)Redcap UE configured with eDRX_IDLE cycle.

- Proposal 3: For cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for (e)RedCap UE with FR1-NTN bands,

  - For 2Rx UE, the existing cell reselection requirements in Rel-18 (e)NTN can be reused for (e)RedCap 2Rx UE with FR1-NTN bands.

  - For 1RX UE, reuse the same margin of RSRP/RSRQ as for 2Rx.

For inter-RAT E-UTRAN cells with TN carrier cell re-selection measurement requirements, the legacy requirements defined for normal UEs can be reused for (e)Redcap UE not configured with eDRX_IDLE cycle. For (e)Redcap UE configured with eDRX_IDLE cycle, the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2 can be reused.

Proposal 9: For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements:

- For (e)Redcap UE not configured with eDRX_IDLE cycle, reuse the legacy requirements defined for normal UEs.

- For (e)Redcap UE configured with eDRX_IDLE cycle, reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2.

  - L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Issue 2-4-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands,

  - Proposal 1: RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN.

- For 1Rx (e)RedCap UEs with FR1-NTN bands,

  - Proposal 1: RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

    - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

      - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

      - L1-RSRP: relax by 3dB.

For measurement accuracy requirement, the way forward from RAN4#114 meeting exactly follow the same principle for (e)RedCap UE in TN network, we support all of them.

Proposal 10: For measurement accuracy requirement, support the way forward from RAN4#114 meeting.

- The impact of HD-FDD

  - General HD-FDD related impact

Issue 3-1-1: General HD-FDD related impact

<Way Forward>:

- Proposal 1: The legacy requirements and applicable conditions can be as the baseline for defining requirements of RedCap UE in NTN scenario.

  - Proposal 1a: To following the TN, the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN:

    - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

    - Handover interruption time

    - Random access

    - SA: RRC Connection Release with Redirection

    - Minimum requirement for L1 indication for RLM and LR

    - MAC-CE/DCI/RRC based uplink spatial relation switch delay

    - Scheduling availability of UE performing intra/inter measurements

    - Scheduling availability of UE during L1-RSRP measurement

  - Proposal 1b: For scheduling availability, need further RAN1 progress for detailed requirements of HD-FDD UEs.

  - Proposal 1c:

    - Adopt TN requirements as baseline for clarifying the requirements listed in P1a for HD-FDD RedCap UE except for

      - MAC-CE/DCI/RRC based uplink spatial relation switch delay

      - Scheduling availability of UE during L1-RSRP measurement

    - Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period.

  - Proposal 1d:RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reach further conclusions.

- Proposal 2: For NTN UEs with HD-FDD, the requirements are met provided that the SSB of at least one of the satellites to be measured in this SMTC period is available at the UE side.

- Proposal 3: For HD RedCap and eRedCap UEs, RAN4 to adopt a high level statement that the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification.

- Proposal 4: RAN4 should further discuss measurement behavior for collision between SMTC and UL when network indicates UL overriding DL in HD-FDD.

For NTN specific aspects, the DL and UL collision error due to mismatch between actual TA used by UE and assumed TA at the gNB may need to further considered. RAN1 has achieved several solutions listed as below:

Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13. Agreement When network indicates UL overriding DL in RRC-Connected mode for collision case 3, one UE-specific RRC parameter is used to indicate overriding for all the applicable other use cases except for collisions with Type-0/0A/1/2-PDCCH CSS. Working assumption For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A[/1][/2]-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - FFS: handling of PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that [DL or UL] is prioritized. - Network is allowed to indicate [UL or DL] overriding [DL or UL] for all cases - This is signaled by one UE specific RRC parameter - Note: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13. Agreement The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode. - FFS whether to prioritize UL or prioritize DL

Firstly, we think the same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline.

Proposal 11: Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline.

Besides, for HD-FDD Redcap UE, due to UL transmission may override DL transmission by network indication or UE implementation, it is rational to clarify that the measurement requirements apply provided that no RS is overrode by UL transmission during the measurement period. Otherwise, longer delays can be expected.

Proposal 12: Clarify that the measurement requirements apply provided that no RS is overrode by UL transmission during the measurement period. Otherwise, longer delays can be expected.

Then we go through the scheduling availability which may impacted by collision cases.

For scheduling availability of UE performing intra/inter measurements, the main rule is that UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols/SMTCs to be measured. This ‘to be measured’ restriction has already excluded the SSB symbols/SMTCs  which overrode by UL transmission. Therefore, the existing principle of scheduling availability requirement can be reused.

For scheduling availability of UE during L1-RSRP measurement, the main rule is that scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. However, with RAN1’s design, the CBD evaluation may not always prioritized over UL transmission. Therefore, we propose to update the side condition as: scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, when CBD evaluation is prioritized over UL transmission.

Proposal 13: For scheduling availability of UE performing intra/inter measurements, the existing principle can be reused, that UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols/SMTCs to be measured.

Proposal 14: For scheduling availability of UE during L1-RSRP measurement, update the principle as scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, when CBD evaluation is prioritized over UL transmission.

  - The specific impact of HD-FDD on NR Handover

Issue 3-2-1: The impact on NR Handover due to HD-FDD

<Way Forward>:

- Proposal 1:

  - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

    - For RedCap NTN UE with HD-FDD, the requirements are met provided that

      - SSB is available at the UE once every SMTC period during Tsearch

      - One SSB is available during T∆

      - One SSB is available during TIU.

  - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

    - For RedCap NTN UE with HD-FDD, the requirements are met provided that

      - One SSB is available during T∆

      - One SSB is available during TIU

- For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - SSB is available at the UE once every SMTC period during Tsearch

        - One SSB is available during T∆

        - One SSB is available during TIU.

- For Time/location-based CHO with L3 measurement, due to the target cell must be the known cell, no Tserach needed in the interruption, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - One SSB is available during T∆

        - One SSB is available during TIU.

Proposal 15: For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - SSB is available at the UE once every SMTC period during Tsearch

        - One SSB is available during T∆

        - One SSB is available during TIU.

Proposal 16: For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - One SSB is available during T∆

        - One SSB is available during TIU

- Others

  - UE capabilities for (e)RedCap UE with FR1-NTN bands

Issue 4-2-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

<Way Forward>:

- Proposal 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Proposal 2: Introduce new capability to declare support over RedCap over NTN.

- Proposal 3: Both proposals, i.e., a new capability declaring (e) RedCap for NTN or combining (e)RedCap and NTN, are fine with us. It may be determined later.

For (e)Redcap UE capability declaring, both Proposal 1 and Proposal 2 are workable. Considering the principle of no need to introduce new capability if existing capability can be used, we slightly prefer the implicit way to save the workload of RAN2.

Proposal 17: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

- Proposal 1: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN.

- Proposal 2: For both 1Rx and 2Rx Redcap UE, the baseline of UE capability is that UE is mandatory to support parallel measurements on 2 SMTC-s for a single frequency carrier.

- Proposal 3: Reuse the legacy collision conditions for parallel measurements on multiple SMTCs.

- Proposal 4: For both intra-frequency measurement and inter-frequency measurement, the number of cells and SSBs can be reused, the number of supported satellites and related UE capability can be reused.

- Proposal 5: Optional feature 25-3(parallelMeasurementGap-r17) is applicable for (e)Redcap UE, the requirement definition can follow the same principle of the existing NR NTN requirements.

Proposal 6: No strong view about whether to add additional requirement applicability about CD-SSB/NCD-SSB.

Proposal 7: For the optional NTN features which have impact on RAN4 requirements, they can also be optionally supported by Redcap UE.

- SMTC adjustment in idle/inactive

- Location-based measurement report trigger

Proposal 8: Define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX_IDLE cycle, the requirement for normal UEs can be reused.

Proposal 9: For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements:

- For (e)Redcap UE not configured with eDRX_IDLE cycle, reuse the legacy requirements defined for normal UEs.

- For (e)Redcap UE configured with eDRX_IDLE cycle, reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2.

Proposal 10: For measurement accuracy requirement, support the way forward from RAN4#114 meeting.

Proposal 11: Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline.

Proposal 12: Clarify that the measurement requirements apply provided that no RS is overrode by UL transmission during the measurement period. Otherwise, longer delays can be expected.

Proposal 13: For scheduling availability of UE performing intra/inter measurements, the existing principle can be reused, that UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols/SMTCs to be measured.

Proposal 14: For scheduling availability of UE during L1-RSRP measurement, update the principle as scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, when CBD evaluation is prioritized over UL transmission.

Proposal 15: For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - SSB is available at the UE once every SMTC period during Tsearch

        - One SSB is available during T∆

        - One SSB is available during TIU.

Proposal 16: For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - One SSB is available during T∆

        - One SSB is available during TIU

Proposal 17: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Reference

- R4-2502687, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)