3GPP TSG-RAN WG4 Meeting #114bis			R4-2503387

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.30.2

Source: 	CMCC

Title: 			   Discussion on the LS on TDM DMRS for 3.75kHz SCS OCC

Document for:	Discussion

- Introduction

RAN1 sent a LS to RAN4 about TDM DMRS for 3.75kHz SCS OCC scheme, we copy the content as follows for information.

# 1. Overall Description: RAN1 has made the following working assumption on the DMRS pattern for OCC for 3.75kHz SCS OCC for NPUSCH format 1: For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.. Question: From a RAN4 perspective, is it feasible to introduce support of TDM DMRS, as per the description above, in Rel-19?

In this contribution, we discuss the feasibility of introducing TDM DMRS for 3.75kHz NB-IOT NTN.

- Discussion

In last RAN4 meeting, when OCC schemes would cross gap in UL transmission, RAN4 has concluded that there is no RF impact for <1ms gap length [1].

- Issue 2-2: Whether there would be UE RF requirement impact when OCC schemes would cross gap in UL transmission.

- Agreement:

  - No RF impact for <1ms gap length.

  - FFS for longer gaps.

For the TDM DMRS case mentioned by RAN1, obviously the UL gap is symbol level and shorter than 1ms. The phase continuity can be maintained by UE. Further, from Demodulation performance perspective, although the DMRSs are blanked in the two slots out of every four slots, the channel estimation can still be achieved through interpolation. Therefore, the feasibility question from RAN1 can be confirmed from RAN4 perspective.

Proposal 1: It is feasible to introduce support of TDM DMRS for 3.75kHz SCS OCC for NPUSCH format 1.

- Conclusion

Based on the discussion above, the following proposal is concluded.

Proposal 1: It is feasible to introduce support of TDM DMRS for 3.75kHz SCS OCC for NPUSCH format 1.

- Reference

- R4-2502373, Way Forward for [114][315] IoT_NTN_Ph3, MediaTek