3GPP TSG-RAN WG4 Meeting #114bis				R4-2503390

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.24.2.1

Source: 	CMCC

Title: 	Discussion on A-IoT BS RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI [1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the RF requirements for Type 1-C Ambient-IOT BS.

- Discussion

2.1 A-IoT BS Tx requirement

The conclusion on BS Tx requirements in study item are captured in following table

| RF Requirement for A-IoT BS- TX requirements |  | Potentially needed or not |
| --- | --- | --- |
| Base station output power |  | YES |
| Output power dynamic | RE power control dynamic range | TBD in WI phase |
|  | Total power dynamic range | TBD in WI phase |
|  | Power boosting | NO |
| Some requirements like Transmit ON/OFF power | Transmitter OFF power | TBD in WI phase |
|  | Transmitter transient period | TBD in WI phase |
| Transmit signal quality | Frequency error | YES |
|  | Some requirements like EVM (Modulation quality) | YES |
|  | TAE | NA |
| Unwanted emissions | Occupied bandwidth | YES |
|  | ACLR | YES |
|  | IBE | NO |
|  | Operating band unwanted emissions | YES |
|  | Transmitter spurious emissions | YES |
| Transmitter intermodulation |  | TBD in WI phase |

- Output power

In SI phase, micro BS is considered and used for co-existence evaluation. Hence, use NR Micro BS type 1-C as the starting point for Rel-19 ambient IOT BS.

The rated carrier output power of the BS type 1-C shall be as specified in table 6.2.1-1.

Table 6.2.1-1: BS type 1-C rated output power limits for BS classes

| BS class | Prated,c,AC |
| --- | --- |
| Medium Range BS | ≤ 38 dBm |

Proposal 1: For reader output power, use NR Medium range BS type 1-C as baseline, i.e delete the square brackets in last approved WF.

Legacy medium range BS use 53dB MCL assumption with minimum 5m horizontal distance between BS and UE. But during the simulation, the minimum distance is assumed as 1m. this MCL will impact max input power level definition of device and is deployment related. It’s suggested to take care of the definition of this MCL for micro cell scenario and taken the device allowed max input power level and deployment scenario into consideration.

Proposal 2: It’s suggested to take care of the definition of MCL for micro cell scenario and taken the device allowed max input power level and deployment scenario into consideration.

- Power related requirements

For power dynamic range related requirement,

- RE power control dynamic range: according to the WID, R2D support OOK-4 modulation scheme. The reader will still use OFDM based modulator to generate OOK-4 signal according to RAN1 agreements. The basic operation unit for RAN1 is at least 1 PRB, so there is no need to define RE power control dynamic range requirements.

- Proposal 3: RE power control dynamic range is not needed because the operation unit for A-IoT is at least one PRB.

- Total power dynamic range: for NR, the basic operation unit in frequency domain is per PRB and total power dynamic range is only related to SCS and max transmission bandwidth configuration. For OOK-4, the basic operation unit is variant as M value. For example, when M =32, the basic operation unit is 4PRB. So the definition should be updated based on M value. RAN4 should further discuss whether total power dynamic range shoud be updated based on M value or not. If RAN4 allow wider bandwidth besides minimum bandwidth, it’s suggested to define new total power dynamic range requirements based on different M value.

Proposal 4: total power dynamic range requirement is needed if wider bandwidth is defined besides minimum transmission bandwidth and the requirements are M value relevant. Otherwise, there is no need for total power dynamic range.

- OFF power requirements

Last meeting approve to define OFF power requirements. One motivation of OFF power is to avoid the interference from closed reader transmitter to reader receiver, especially when the received SNR is much lower. From coverage point of view, the R2D is bottleneck compared with D2R, so there is still margin at D2R receiver and the impact of OFF power interference could be negligible. Therefore, there is no need to define more strict requirements than legacy BS requirements.

Proposal 5: legacy OFF power requirements could be reused.

- Transient period

- IoT reader wok in HD-FDD mode, so the transient period is needed.

RFID also define Tx time related requirement, e.g. rise time, settling time, fall time.

- Rise time: the time periodicity of power-up envelop from 10% point rising monotonically to 90% point.

- Fall time: the time periodicity of power-down envelop from 90% point falling monotonically to 10% point.

- Settling time: the time periodicity of power-up envelop rise from 90% point until at least the ripple limit MI. the 90% point is rising monotonically from 10% point. During the whole settling time, envelop should not fall below the 90% max filed strength.

BS spec defines transient period which is much like rise time and fall time requirements as in RFID spec. BS transient period is much stricter than RFID rise and fall time. It’s reasonable to reuse BS transient period requirements.

Proposal 6: it’s reasonable to reuse legacy BS transient period requirements.

Besides, RFID also defines settling time to evaluate signal ripple characteristics. reader support OOK signal which is much like the ASK modulated signal as in RFID spec. It’s suggested to further check whether the settling time as in RFID spec is needed or not.

Proposal 7: further consider whether settling time as defined in RFID spec is needed or not to evaluate RF envelop ripple characteristics.

- Tx signal quality related requirement

For signal quality related requirements, gNB spec define EVM requirement which is used to evaluate phase distance. A-IOT reader only supports OOK-4 modulation. So current gNB quality related requirement is not applicable anymore. RFID[1] use three types ASK modulation types, i.e. DSB-ASK, SSB-ASK or PR-ASK and define RF envelop requirement including modulation depth, RF envelop ripple, RF pluswidth etc which can be referred for OOK modulation. Details illustration of RFID RF envelop requirement is listed as below for information.

In theory, The baseband signal for OOK-4 is much like that of the ASK signal. But now RAN1 has introduced the CP for OOK-4 to orthogonal with NR sub-carrier which will impact the modulation signal quality. It’s suggested to wait for RAN1 agreements of CP related conclusion before discussing whether to reuse RFID signal quality.

Proposal 8: RAN4 define modulation depth, RF envelop ripple, RF plusewidth requirements and further check whether to reuse the same value as RFID or not based on RAN1 conclusion of CP discussion.

- Unwanted emission requirements

For unwanted emission requirements, legacy NB-IOT requirements are used for co-existence evaluation in SI. The conclusion of evaluation shows that legacy requirements can fulfil the co-existence when NR macro BS is outdoor and A-IOT reader is indoor. One scenario that causes severe interference is when legacy NR UE is indoor, which require some existing scheduling mechanism to avoid adjacent interference or enhanced mechanism to mitigate interference (as captured in TR 38.769 clause 6.6.5.10). Consider enhanced mechanism to mitigate interference is not within Rel-19 WI scope, then legacy NB-IOT requirements can be reused for A-IOT reader.

For NB-IoT, 100kHz offset is defined. Some background of this 100kHz offset is listed as below:

During the discussion of NB-IoT BS UEM requirements[2][3][4], standalone operation is divided into two sub-cases according to WID description:[2]

a)NB-IoT only operation --- start from GSM mask----capture in TS 36.104

b)NB-IoT with other RATs mixed operation ----- start from MSR mask----capture in TS 37.104

[3] compares the modulation spectrum mask between GSM and MSR, results show the mask is almost the same in the 0-150kHz offset range from RF channel bandwidth edge when Foffset is 100kHz. so finally the Foffset from MSR spec is reused and only one set of requirements for both above sub-cases, i.e. the BC2 MSR requirement applies.

Compared with GSM spectrum mask, OFDM based reader has higher PAPR, so under the same output power, A-IoT needs more linear PA to meet GSM mask and A-IoT will reach the lower power to meet GSM mask than GSM. Therefore, 100kHz offset seems still applies for A-IoT.

Proposal 9: legacy NB-IOT ACLR, unwanted emission can be reused for A-IOT reader. 100kHz offset is still applicable.

2.2 A-IoT BS Rx requirement

The conclusion on BS Rx requirements in study item are captured in following table

| A-IoT BS- RX RF Requirement |  | Potentially needed or not |
| --- | --- | --- |
| Reference sensitivity level |  | YES |
| Dynamic range |  | TBD in WI phase |
| Adjacent Channel Selectivity |  | YES |
| In-band blocking | general blocking | YES |
|  | narrowband blocking | YES |
| Out-of-band blocking |  | YES |
| Receiver spurious emission |  | YES |
| Receiver intermodulation | General intermodulation | Wait for RAN1 conclusion. |
|  | Narrowband intermodulation | YES |
| In-channel selectivity |  | TBD in WI phase |

- Reference Sensitivity

Based on RAN1 link budget analysis as below, R2D link is bottleneck for D1T1-B.

Table D1T1-96bit-10%BLER-6kbps-BPSK-6dBi BS antenna gain

| Scenario | CW case | R2D pathloss (dB) | D2R pathloss (dB) | min pathloss (dB) | coverage(m) |
| --- | --- | --- | --- | --- | --- |
| Device 1 |  |  |  |  |  |
| D1T1-B | 1-4 | 67.1 | 84.54 | 67.10 | 36.59 |

But it’s noted, RAN1 use 33dBm as reader max output power, but based on RAN4 latest agreements, reader output power could be increased to 38dBm per antenna connector. If the assume 2Tx of reader, then the coverage margin between R2D and D2R link could be negligible (from 7.44 to -0.56dB). besides, if CW is distant from device, then the D2R link will be the coverage bottleneck. Therefore, REFSENSE is significant.

Observation 1: R2D link is not the coverage bottleneck anymore if reader is assumed as medium range BS with 2Tx. And the D2R link will be the bottleneck if we assume CW is much distant from device.

To conclude final SNR, it’s suggested to converge and align simulation assumptions. Following lists our simulation assumption to conclude final SNR at reader side.

| Parameters | Assumptions | Note |
| --- | --- | --- |
| Carrier frequency | 900 MHz |  |
| SCS | 15 kHz as baseline |  |
| Block structure | Depend on RAN1 |  |
| Channel model | AWGN |  |
| Number of Tx/Rx chains for Ambient IoT device | 1 |  |
| Number of BS antenna elements | 1 |  |
| Reference data rate | 1 kbps (M) 5 - 7 kbps (M) | Data rate is very import for final SNR, should be carefully be taken care of. |
| Message size | 96 bits | 96bits is suggested. RAN1 allow message segmentation, so 400bits is not mandatory. |
| BLER target | 10% |  |
| Sampling frequency（SFO） | No need | SFO: Randomly select a value from the range of [104 ppm~105 ppm] for device 1 Reader could use preamble and midamble to avoid the SFO impact. From reader demode point of view, residual SFO rather than SFO is more important. How about update it as residual SFO which is assumed as max 500ppm in RAN1 |
| Residual SFO | Max 500ppm |  |
| Transmission bandwidth（DSB） | The value is for two sidebands. 2SB=round up (4*data rate) in the units of 15kHz | Last meeting agreements: Option 1： X kHz is considered for D2R transmission bandwidth. The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz X = {[15 (M)], [180 (O)]} Option 2: 15kHz |
| OOK/BPSK chip rate | Data rate *2 | For Manchester encoding, chip rate=data rate, |
| Receiver bandwidth | It’s suggested the same as transmission bandwidth. |  |
| Waveform (CW) | unmodulated single tone |  |
| Modulation | OOK |  |
| Line code | Manchester encoding |  |
| FEC | 1/3 Convolutional Code | Option 1：No FEC Option 2：1/3 Convolutional Code For coverage, option 2 is suggested |
| ADC bit width | Ideal |  |
| D2R receiver | coherent receiver |  |
| Repetition | None |  |

Proposal 10: above list the simulation assumption which could be used as baseline for final SNR definition.

- ACS and blocking related requirements

If there is no indoor UE, then the ACS and blocking seems not that significant. But it’s relatively easier for BS to achieve ACS requirements using digital filter. There is no need to define much relaxed requirements since BS could easily achieve it.

Proposal 11: For ACS requirements, reuse legacy NB-IOT requirements.

NB-IoT services are deployed using band 8, it’s better to consider the narrowband blocking. Legacy NB-IoT narrow-band blocking requirements would be the baseline but further update the interference signal level.

Proposal 12: define narrow-band blocking requirements for reader.

- Inter-modulation related requirements

Inter-modulation requirements is used to evaluate receiver non-linearity requirements. Besides, there is NB devices using band n8, it’s suggested to define the narrow-band inter-modulation requirements.

Proposal 13: it’s suggested to at least define the narrow-band inter-modulation requirements.

- ICS

ICS requirement is used to evaluate receiver performance when the wanted signal is allocated to certain PRB while there is other UL signals. Current scope support device FDMA operation, which is similar like the use case for ICS requirements. So such requirement is suggested. Reader know each devices’ transmission bandwidth and it could use per device digital filter to achieve better ICS compared with legacy NR BS which can’t use per UE filter.

As for the wanted signal and interference signal for ICS, since there is no UL power control, the max interference signal could be assumed from the nearest device and the wanted signal comes from the farthest device.

Proposal 14: ICS requirement is needed with assuming wanted signal coming from the nearest device and interference signal coming from the farthest device.

# 3. Conclusions

In this contribution, RF requirements for gNB like reader are discussed with following observations and proposals.

Proposal 1: For reader output power, use NR Medium range BS type 1-C as baseline, i.e delete the square brackets in last approved WF.

Proposal 2: It’s suggested to take care of the definition of MCL for micro cell scenario and taken the device allowed max input power level and deployment scenario into consideration.

- Proposal 3: RE power control dynamic range is not needed because the operation unit for A-IoT is at least one PRB.

Proposal 4: total power dynamic range requirement is needed if wider bandwidth is defined besides minimum transmission bandwidth and the requirements are M value relevant. Otherwise, there is no need for total power dynamic range.

Proposal 5: legacy OFF power requirements could be reused.

Proposal 6: it’s reasonable to reuse legacy BS transient period requirements.

Proposal 7: further consider whether settling time as defined in RFID spec is needed or not to evaluate RF envelop ripple characteristics.

Proposal 8: RAN4 define modulation depth, RF envelop ripple, RF plusewidth requirements and further check whether to reuse the same value as RFID or not based on RAN1 conclusion of CP discussion.

Proposal 9: legacy NB-IOT ACLR, unwanted emission can be reused for A-IOT reader. 100kHz offset is still applicable.

Observation 1: R2D link is not the coverage bottleneck anymore if reader is assumed as medium range BS with 2Tx. And the D2R link will be the bottleneck if we assume CW is much distant from device.

| Parameters | Assumptions | Note |
| --- | --- | --- |
| Carrier frequency | 900 MHz |  |
| SCS | 15 kHz as baseline |  |
| Block structure | Depend on RAN1 |  |
| Channel model | AWGN |  |
| Number of Tx/Rx chains for Ambient IoT device | 1 |  |
| Number of BS antenna elements | 1 |  |
| Reference data rate | 1 kbps (M) 5 - 7 kbps (M) | Data rate is very import for final SNR, should be carefully be taken care of. |
| Message size | 96 bits | 96bits is suggested. RAN1 allow message segmentation, so 400bits is not mandatory. |
| BLER target | 10% |  |
| Sampling frequency（SFO） | No need | SFO: Randomly select a value from the range of [104 ppm~105 ppm] for device 1 Reader could use preamble and midamble to avoid the SFO impact. From reader demode point of view, residual SFO rather than SFO is more important. How about update it as residual SFO which is assumed as max 500ppm in RAN1 |
| Residual SFO | Max 500ppm |  |
| Transmission bandwidth（DSB） | The value is for two sidebands. 2SB=round up (4*data rate) in the units of 15kHz | Last meeting agreements: Option 1： X kHz is considered for D2R transmission bandwidth. The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz X = {[15 (M)], [180 (O)]} Option 2: 15kHz |
| OOK/BPSK chip rate | Data rate *2 | For Manchester encoding, chip rate=data rate, |
| Receiver bandwidth | It’s suggested the same as transmission bandwidth. |  |
| Waveform (CW) | unmodulated single tone |  |
| Modulation | OOK |  |
| Line code | Manchester encoding |  |
| FEC | 1/3 Convolutional Code | Option 1：No FEC Option 2：1/3 Convolutional Code For coverage, option 2 is suggested |
| ADC bit width | Ideal |  |
| D2R receiver | coherent receiver |  |
| Repetition | None |  |

Proposal 10: above list the simulation assumption which could be used as baseline for final SNR definition.

Proposal 11: For ACS requirements, reuse legacy NB-IOT requirements.

Proposal 12: define narrow-band blocking requirements for reader.

Proposal 13: it’s suggested to at least define the narrow-band inter-modulation requirements.

Proposal 14: ICS requirement is needed with assuming wanted signal coming from the nearest device and interference signal coming from the farthest device.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-77AH-IoT-0018, On BS emission requirement in standalone operation for NB-IoT, Huawei

- R4-160610, On BS emission mask in stand-alone operation for NB-IoT, Huawei

- R4-161289, Way forward on BS unwanted emission requirement for NB-IoT

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).

- R4-2417088, WF on RF requirement impact of A-IoT, Huawei.

- R4-2414305, WF on RF requirements for A-IoT, Huawei

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID, Specification for RFID Air Interface Protocol for Communications at 860 MHz - 960 MHz, Version 2.0.0 Ratified