3GPP TSG-RAN WG4 Meeting #114bis				R4-2503391

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.24.2.2

Source: 	CMCC

Title: 	Discussion on A-IoT device RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI[1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the RF requirements A-IOT devices.

- Discussion

2.1 Tx requirements

Agreement in SI phase:

- Whether requirements potentially needed or not is summarized in the following table:

| A-IoT device TX (D2R) RF Requirement |  |
| --- | --- |
|  |  | Device 1 |
| Transmit output power | Maximum output power /output power | NO for maximum output power YES for backscattering power or power backscattering loss |
| Output power dynamic | Transmit OFF power | No |
|  | Transmit ON/OFF time mask | NO, as RAN4#112 agreed |
|  | Minimum output power | N/A |
|  | Power control | NO, as RAN4#112 agreed |
| Transmit signal quality | Frequency error | NO, as RAN4#112 agreed |
|  | Modulation quality requirements | TBD |
|  | In band emissions | NO, as RAN4#112 agreed |
|  | Carrier leakage | NO, as RAN4#112 agreed |
| Output RF spectrum emissions | Occupied bandwidth | YES |
|  | SEM | TBD |
|  | ACLR | TBD |
|  | Spurious emissions | TBD |
| Unwanted emissions |  | TBD |
| Transmit intermodulation |  | NO |

For output power, backscattering power can be defined for device 1. Due to only outside topology is considered in Rel-17, the location of CW node will also impact the backscattering power. UE like max output power definition could guarantee D2R coverage. Besides, this requirement will also reflect device allowed max input power level from CW to guide CW location deployment, i.e. device should still work under the max input power. Typical testing condition is needed to make sure max input power from CW.

Proposal 1: from coverage point of view, suggest to define max backscattering power under the max input power from CW and all devices should achieve such max power.

Device support OOK and BPSK modulation scheme. For NR BPSK modulation, 17.5% EVM is defined which equals to 15dB SNR. In theory this is the max SNR at receiver side. For A-IoT, from D2R coverage point of view, reader may receive such high SNR signal, so this EVM requirement is still significant and should be reused.

Proposal 2: for BPSK modulation, use legacy UE EVM requirement as starting point.

For OOK modulation, reader may detect the rising and falling edge to demode OOK signal. So the legacy RF plusewidth of RFID could be referred. Besides, the modulation depth will also impact final BER.

Proposal 3: for OOK signal, further check whether can reuse legacy RFID plusewidth and modulation depth requirements.

Based on the co-existence evaluation, no extra requirement is needed. So the simulation assumption for device leakage could be used as baseline. Note the simulation assumption is derived only from baseband signal without considering RF impact. Certain relaxation is also allowed to consider the RF impacts

Proposal 4: for SEM, use device leakage simulation assumption as starting point and further consider whether to allow certain relaxation considering RF impacts.

For spurious emission, this is mainly used to evaluate the interference to other systems, since D2R using UL spectrum, the same spurious emission requirement as UE could be reused.as for the applicable frequency range, it could be outside the band edge, i.e. the same applicable range as legacy NR bands.

Proposal 5: use legacy NR UE spurious emission requirements as starting point.

2.1 Rx requirements

Agreement in SI phase:

- Whether requirements potentially needed or not is summarized in the following table:

| A-IoT device- RX Requirement |
| --- |
|  | Device 1 |
| Reference sensitivity | YES, as RAN4#112 agreed |
| Maximum input power | TBD |
| ACS | NO |
| ACSC | NO |
| In-band blocking | NO |
| Out-of-band blocking | NO |
| Receiver intermodulation | NO |
| Rx spurious emission | NO |
| Spurious response | NO, as RAN4#112 agreed |
| Interference rejection | TBD |

For device 1, considering its low cost, only define reference sensitivity for Rx requirements. In SI phase, -36dBm is used for co-existence evaluation. For the methodology, legacy SNR+N method is not applicable because device’s sensitivity is much high than the value calculated from SNR+N.

Proposal 6: legacy SNR+N methodology is not applicable since actual devices’ sensitivity is much high than that calculated from SNR+N.

In our view, device sensitivity depends on the device capability. So the legacy approach to use NF+target SNR is not appropriate to define device reference sensitivity. Similar as backscattering power, device declaration on reference sensitivity can also be considered.

Proposal 7: REFSENSE requirement can be defined considering device capability or based on device declaration.

The max input level is used to evaluate receiver performance under large input level. So such requirement is applicable for all kinds of signal. This should be applicable for both CW and R2D signal. But during testing, we can only use one signal. Our suggestion is that device max output power could already test device receiver performance under max input power from CW node. So for max input power, we can only use R2D signal.

Proposal 8: max input power is defined from R2D signal.

For max input power, now we assume medium range BS cell, the MCL is 53dB with 5m assumption. But in the real deployment scenario, it seems the minimum distance from reader to device would be much smaller than 5m. besides, during the simulation 1m is assumed. Therefore, it’s suggested to use 1m assumption for max input power and update the definition of reader BS class.

Proposal 9: use 1m distance assumption from reader to device. Define the max input power requirements and update the definition of reader BS class based on above assumption.

Regarding for testing, throughput is not the proper testing metric anymore. RAN4 further check whether max input power could be tested as lone as device could response inventory command.

Proposal 10: max input power is tested with the target that device could response inventory command.

# 3. Conclusions

In this contribution, RF requirements for device 1 are discussed with following observations and proposals.

Proposal 1: from coverage point of view, suggest to define max backscattering power under the max input power from CW and all devices should achieve such max power.

Proposal 2: for BPSK modulation, use legacy UE EVM requirement as starting point.

Proposal 3: for OOK signal, further check whether can reuse legacy RFID plusewidth and modulation depth requirements.

Proposal 4: for SEM, use device leakage simulation assumption as starting point and further consider whether to allow certain relaxation considering RF impacts.

Proposal 5: use legacy NR UE spurious emission requirements as starting point.

Proposal 6: legacy SNR+N methodology is not applicable since actual devices’ sensitivity is much high than that calculated from SNR+N.

Proposal 7: REFSENSE requirement can be defined considering device capability or based on device declaration.

Proposal 8: max input power is defined from R2D signal.

Proposal 9: use 1m distance assumption from reader to device. Define the max input power requirements and update the definition of reader BS class based on above assumption.

Proposal 10: max input power is tested with the target that device could response inventory command.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).