3GPP TSG-RAN WG4 Meeting #114bis				R4-2503392

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.24.1

Source: 	CMCC

Title: 	Discussion on A-IoT System parameters

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI [1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the system parameters for A-IOT.

- Discussion

The conclusion on system parameters in study item are captured in following table.

| System parameter for A-IoT |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | A-IoT BS | A-IoT device | Intermediate UE |
| Operating band |  | YES |  |  |
| Channel bandwidth | Transmission bandwidth | YES the value needs be discussed in WI phase depending on other WG conclusion. |  |  |
|  | channel bandwidth | YES |  |  |
| Channel arrangement | Channel spacing | TBD in WI phase for standalone NA for in-band/Guard-band |  |  |
|  | Channel raster | IB/SA:YES GB:TBD | GB:NO IB/SA:TBD | IB/SA:YES GB:TBD |
|  | Synchronization raster | NO for device 1/2a |  |  |

- R2D Transmission bandwidth

Last meeting agreement is listed as below:

Issue 2-1-4: Values of R2D bandwidth Agreement: - Define R2D transmission bandwidth configuration: - 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS - FFS on other RB numbers - Further discuss the R2D channel bandwidth considering additional guard band. - If RAN1 has the agreement different than above, RAN4 will revisit the agreements

According to the conclusion of SI, the minimum transmission bandwidth for R2D is 1PRB. R2D transmission bandwidth is related to the M value of OOK-4. The max M value is 32, the max minimum R2D transmission bandwidth is 4PRB. Larger transmission bandwidth will allow higher data rate, but legacy outdoor 900MHz BS also needs to mute  corresponding PRB to avoid co-channel interference. Usually the data rate of reader device is not high, it’s better to limit the max transmission bandwidth configuration, i.e. 4PRB is still enough. If so, then there is no total dynamic range requirements.

Observation 1: larger R2D transmission bandwidth require outdoor 900MHz BS to reserve more muted PRB to avoid co-channel interference.

Proposal 1: For R2D transmission bandwidth, limit the max transmission bandwidth to 4PRB and there is no dynamic range requirements.

One motivation of channel bandwidth is to use this CBW as applicable frequency edge to define other requirements, i.e. unwanted emission requirements. From this point of view, the guard band is necessary. Reader use OFDMA based OOK-4 signal, the shaping filter would be much similar like NR, so the similar spectrum efficiency as NR could be reused. For NR 3MHz CBW, 90% spectrum efficiency is used. if reusing such spectrum efficiency, 200kHz, 400kHz, 600kHz, 800kHz CBW are assumed for 1PRB, 2PRB, 3PRB, 4PRB.

Proposal 2: for R2D, consider 200kHz, 400kHz, 600kHz, 800kHz CBW with 90% spectrum efficiency assumption for 1PRB, 2PRB, 3PRB, 4PRB.

- D2R channel bandwidth

Last meeting agreement is listed as below:

Issue 2-1-6: D2R bandwidth Agreement: - D2R channel bandwidth for A-IoT device - Define D2R channel bandwidth for A-IoT device considering the following aspects: - [1SB or] 2SB - D2R maximum data rate (depending on RAN1 conclusion) - SFO - FDMA, i.e., considering the different device 1 has different frequency shift - Reference Filter performance - Other aspects are not precluded

One motivation of CBW at NR spec is to identify its filter passband bandwidth to filter out wanted signal. Based on RAN1 design, reader could know the CBW+SFO based on indicated chip rate and data rate and filter D2R signal out. Next, reader could use preamble, midamble to identify the SFO. So reader don’t need the CBW concept.

Another motivation for channel bandwidth is to define applicable frequency range for unwanted emission. Based on co-existence analysis and device baseband leakage performance, the interference to outdoor legacy BS is negligible. If finally RAN4 approve to define unwanted emission requirements, we can use transmission bandwidth as the frequency edge to define unwanted emission requirements.

Besides, Value of D2R transmission bandwidth is related to the uplink data rate and corresponding chip rate, which can be very flexible. According to the SI conclusion, D2R transmission bandwidth of 2SB range is from 15KHz to 2400KHz, and it’s integer multiple(s) of 15KHz. It’s very hard to define limited CBW for device. Therefore, no need to define CBW requirements unless clear and strong motivation is identified for CBW requirements.

Proposal 3: there is no need for D2R CBW requirements unless strong motivation is identified for CBW definition.

- D2R transmission bandwidth

D2R only supports DSB in Rel-19, we first need to discuss whether to define bandwidth based on 1SB or 2SB. The advantage to define bandwidth based on 1SB is that when considering small frequency shift, the definition of bandwidth can be unified. Otherwise, after small frequency shift, how to define the gap between 2SBs will be a problem, as illustrated in the following figure.

Except for the transmission bandwidth of 1SB, which is the baseband chip rate F, SFO impact needs to be considered for the channel bandwidth of 1SB. Consider the worst SFO 10%, the channel bandwidth of 1SB would become F*(1+10%).

Proposal 4: it’s suggested to define D2R transmission bandwidth of 1SB.

We can consider to define transmission bandwidth with an equation, and define requirements based on some typical transmission bandwidth, e.g. 15KHz, 180KHz, 2400KHz.

According to the WID, small frequency shift is supported in Rel-19. As illustrated in the above figure, the frequency will be shifted by R*F, R is codeword repetition number and can be configured within a very large range, F is the transmission bandwidth of 1SB. Due to the shift, the 10% SFO will have impact on R*F. Hence, the channel bandwidth after small frequency shift will become F+20%*R*F. Also FDMA operation among different devices is in the scope. If we use the transmission bandwidth edge to define UEM requirements, then this method applies for both small frequency shift and FDMA cases.

Proposal 5: RAN4 define SEM related requirements from the edge of some typical transmission bandwidth which is an equaltion. This definition applies for both small frequency shift and FDMA cases. For example

- D2R transmission bandwidth of 1SB=baseband chip rate

- Channel raster

Last meeting agreement is listed as below

Issue 2-1-5: R2D channel raster and channel spacing Agreement: - Do not define channel spacing for R2D transmission - Define channel raster for R2D transmission. - Details need further discussion.

For channel raster, in SI phase, it was agreed that for in-band and standalone spectrum operation mode, channel raster is potentially needed. Considering the R2D and D2R transmission are at least integer multiples if 15KHz, it is suggested to define channel raster as 15KHz. The channel raster is mainly from the reader side, for device side, there is no need to define channel raster.

Proposal 7: Define channel raster as 15KHz for reader.

# 3. Conclusions

In this contribution, system parameters are discussed with following proposals.

Observation 1: larger R2D transmission bandwidth require outdoor 900MHz BS to reserve more muted PRB to avoid co-channel interference.

Proposal 1: For R2D transmission bandwidth, limit the max transmission bandwidth to 4PRB and there is no total power dynamic range requirements.

Proposal 2: for R2D, consider 200kHz, 400kHz, 600kHz, 800kHz CBW with 90% spectrum efficiency assumption for 1PRB, 2PRB, 3PRB, 4PRB.

Proposal 3: there is no need for D2R CBW requirements unless strong motivation is identified for CBW definition.

Proposal 4: it’s suggested to define D2R transmission bandwidth of 1SB.

Proposal 5: RAN4 define SEM related requirements from the edge of some typical transmission bandwidth which is an equaltion. This definition applies for both small frequency shift and FDMA cases. For example

- D2R transmission bandwidth of 1SB=baseband chip rate

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).

- R4-2417088, WF on RF requirement impact of A-IoT, Huawei.

- R4-2414305, WF on RF requirements for A-IoT, Huawei

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID, Specification for RFID Air Interface Protocol for Communications at 860 MHz - 960 MHz, Version 2.0.0 Ratified