3GPP TSG-RAN WG4 Meeting # 114-bis                              R4-2503398

Wuhan, China, 7th –11th April, 2025

Agenda item:	7.26.3

Source: 	CMCC

Title: 	Discussion on LP-WUS BS RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #105 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In RAN4#114, the LP-WUS BS RF requirements had been discussed and the WF was approved in [1]. This paper will focus on some issue about LP-WUS BS RF requirements.

- Discussion

2.1 Core requirement for LP-WUS power boosting

WF: - Adopt following content as the concept of LP-WUS power boosting. - The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). - Clarify the declaration on the support of LP-WUS and support of LP-WUS power boosting should be separate. - The minimum power boost level is included in the BS manufacturer declaration table, but not defined as a minimum requirement. - FFS on the detail of the minimum power boosting level, e.g. - Whether it can be different for different BS CBW

From our perspective, it is challenging to establish a minimum requirement specific to the network. Given that different operators employ this technology across a range of scenarios, setting a uniform minimum requirement is complex. The same applies to the declaration tables of BS manufacturer. However, we believe that different BS CBW scenarios can be considered. Under the current definition of power boosting requirements, NR signals with smaller bandwidths are more significantly impacted by the same power boosting level. Therefore, we suggest that the power boosting level could be different for different BS CBWs.

Proposal 1: the minimum power boosting level could be different for different BS CBW.

### 2.2 BS Tx requirements for LP-WUS

WF: - For RAN4 simulation campaign and discussion in next meeting, the latest RAN1 agreements on waveform design are expected to be used. - FFS on the following options, others are not precluded. - Option 1: No need to specify transmit signal quality requirement for LP-WUS. - Option 2: Instead of defining transmit signal quality requirement for LP-WUS, just do the verification e.g. utilizing a higher-order EVM test (64QAM) for NR with embedded LP-WUS.

For BS RF requirements, in order to regulate the use of LP-WUS and its performance, we suggest to define a set of the BS Tx requirements. Unwanted emissions should also to be defined.

Proposal 2: Define a set of the BS Tx requirements for LP-WUS.

# 3. Conclusions

Proposal 1: the minimum power boosting level could be different for different BS CBW.

Proposal 2: Define a set of the BS Tx requirements for LP-WUS.

# 4. Reference

- R4-2502305, Way Forward for [114][308] NR_LPWUS