3GPP TSG-RAN WG4 Meeting # 114-bis                                R4-2503402

Wuhan, China, 7th –11th April, 2025

Agenda item:	7.23.2.3

Source: 	CMCC

Title: 	Discussion on existing Rx requirements for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [1]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 ACS requirements

Following capture the agreements from TR 38.858 for ACS requirements:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

Last meeting, we have the following requirements.

Agreement: - FFS on the followings: - The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

Similar to ACLR requirements, adjacent-channel co-existence simulation only focuses on not less than 10% grid shift values, and 0% grid shift is avoided. Considering it is hard to define the co-location ACS requirement, it is better to prioritize discussing the scenarios under non-colocation case first.

The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

Proposal 1: Prioritize discussing the scenarios under non-colocation case first.

Proposal 2: The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

2.2 In-band blocking

Last meeting we have approved the WF below:

WF: - For FR2-1 WA SBFD BS, re-use the current FR2-1 WA BS in-band blocking requirements. - For FR1 WA BS, try to define the in-band blocking requirement, FFS on following options. - Option 1: Define the requirement by selecting one of the following grid-shift values. Additional emission issue is not included in this release. If regulators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario. - Option A: 20% - Option B: 50% - Option 2: Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario. - Option 3: Define in-band blocking requirements based on co-existence simulation results with GS 5%. - Option 4: Define in-band blocking requirements based on co-existence simulation results with grid-shift 100%. - For FR1 MR SBFD BS, FFS.

For Option 3 and Option 4, we believe that the scenarios are too extreme. For Option 3, according to the simulation results from various companies, the blocking requirements is almost close to 0, which is extremely challenging for equipment implementation. Similarly, for the 100% grid shift scenario mentioned in Option 4, it is also difficult to implement from a deployment perspective. Therefore, considering practical deployment and taking into accounting both the deployment scenarios and the feasibility of achieving the metrics, we believe that the results obtained with the parameters of 20% and 50% can serve as references.

Proposal 3: The results obtained with the parameters of 20% and 50% grid shift can serve as references.

2.3 Receiver intermodulation

The introduction of the two new RX intermodulation requirements is primarily driven by the need to address BS2BS cross-link interference. For the general in-band blocking scenario has already accounted for the new Rx cross-link interference. So there is no need to define RX intermodulation requirement with 1 interfering signal. For the scenario for 2 input signals, it could be regarded as the worst case when setting requirement on receiver linearity. It is better to defined this requirement.

Proposal 4: there is no need to define new RX intermodulation requirement with 1 interfering signal.

Proposal 5: It is better to define new RX intermodulation requirement with 2 interfering signals.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: Prioritize discussing the scenarios under non-colocation case first.

Proposal 2: The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

Proposal 3: The results obtained with the parameters of 20% and 50% grid shift can serve as references.

Proposal 4: there is no need to define new RX intermodulation requirement with 1 interfering signal.

Proposal 5: It is better to define new RX intermodulation requirement with 2 interfering signals.

# 4. Reference

[1] TR 38.858,

[2] R4-2502310, Way Forward for [114][306] NR_duplex_evo_General