3GPP TSG-RAN WG4 Meeting # 114-bis                               R4-2503404

Wuhan, China, 7th –11th April, 2025

Agenda item:	7.23.2.1

Source: 	CMCC

Title: 	Discussion on new RF requirement for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [2]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 Location of transient period between SBFD and non-SBFD

Agreement: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols - FFS guard symbol(s) in this case is SBFD symbol or not. - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission: - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design. - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall - either send LS to RAN1 to ask for clarification, - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact. - Case-D: in TA_offset in non-SBFD UL symbol(s). - Note: The illustrative figure for Case A, B, C and D is provided below:

We think set transient period between SBFD and non-SBFD within the guard period is superior alternative. On the one hand, the duration of the guard period is longer than that of transient periods. On the other hand, it can avoid impacts on traditional TDD and SBFD operations. Given the critical role that TDD and SBFD play in maintaining the seamless functioning of contemporary communication networks, preserving their stability and efficiency is of paramount importance. However, this issue needs to wait for further progress from RAN1.

Proposal 1: Transient period between SBFD and non-SBFD could be set in guard symbol(s), and wait for the conclusion for the location of the guard period from RAN1.

2.2 In-channel adjacent subband leakage ratio requirements

In last meeting, agreements are listed as below:

Agreement: - New requirement for In-channel adjacent subband leakage ratio: - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104) - FFS the value for absolute PSD limit - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit.

The existing ACLR absolute basic limit requirements in TS 38.104 are shown as below.

Table 6.6.3.2-2: Base station ACLR absolute basic limit

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

Table 9.7.3.3-2: BS type 2-O ACLR absolute limit

| BS class | ACLR absolute limit |
| --- | --- |
| Wide area BS | -13 dBm/MHz |
| Medium range BS | -20 dBm/MHz |
| Local area BS | -20 dBm/MHz |

When co-exist with legacy NR network, interference may still occur even with infinitely increased SBFD performance. But when co-exist with SBFD of different configuration, interference could be avoided by increased SBFD performance. We could identify the assumption of adjacent network performance with legacy ACLR absolute basic limit requirement.

Proposal 2: Identify the assumption of adjacent network performance before defining sub-band Tx leakage.

Proposal 3: Legacy ACLR requirements could be used as starting point.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: Transient period between SBFD and non-SBFD could be set in guard symbol(s), and wait for the conclusion for the location of the guard period from RAN1.

Proposal 2: Identify the assumption of adjacent network performance before defining sub-band Tx leakage.

Proposal 3: Legacy ACLR requirements could be used as starting point.

# 4. Reference

[1] TR 38.858

[1] R4-2502310, Way Forward for [114][306] NR_duplex_evo_General