3GPP TSG-RAN WG4 Meeting # 114-bis                                R4-2503405

Wuhan, China, 7th –11th April, 2025

Agenda item:	7.23.1

Source: 	CMCC

Title: 	Discussion on SBFD general part

Document for:	Discussion

# 1. Introduction

During the previous session, the WF on SBFD BS requirements was approved with multiple consensus points [2]. In this submission, we proceed with the discussion on the general aspects, building upon the WF that was agreed upon in the last meeting.

- Discussion

  - DL/UL subband configurations support for SBFD

In RAN4 113 meeting, agreements are listed as below.

Issue 1-3-1: Clarification on declaration and flexibility Agreement: - The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - FFS on how to capture this in the specification. - This restriction shall at least be applied to FR1 WA BS - FFS other BS classes - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations. - FFS how to capture this in the specifications Issue 1-3-2: DL/UL subband configurations support for SBFD Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

The conclusions reached at the last meeting were all related to the restrictions on SBFD subband configurations. After simulation results of the study item, for MR BS and LA BS, the vast majority of companies believe that coexistence can be achieved. Therefore, there is no need to impose restrictions on the subband configurations for MR BS and LA BS

Proposal 1: There is no need to set restrictions on subband configurations for MR and LA BS.

Regarding the discussion on restrictions for macro base stations, the conclusion from the last meeting was that a maximum of [2 or 3] specific Uplink (UL) subband sizes are allowed to be declared and should be selected from the channel bandwidth sizes listed in Table 5.3.2-1 of TS38.104. In our view, one of the key benefits of SBFD is its flexible frame structure. To retain this advantage, we advocate for no restrictions on the size of subbands. However, considering the concerns of some companies, as well as preserving the feasibility of future base station implementations, we believe that allowing three types of UL subband configurations could already effectively restrict the WA BS.

Proposal 2: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared for WA BS.

  - Restriction on guardband size for SBFD

Agreement: - Restriction on guardband size for SBFD: - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband) - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications. - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband:

Since the sub-band size is not fixed in the spec, the guard band also should not be fixed. The maximum value of the guard band size should be specified in the spec, and some guard RB could be used as DL or UL in actual operation.

Proposal 3: The maximum value of the guard band size should be specified in the spec.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: There is no need to set restrictions on subband configurations for MR and LA BS.

Proposal 2: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared for WA BS.

Proposal 3: The maximum value of the guard band size should be specified in the spec.

# 4. Reference

[1] TR 38.858,

[2] R4-2502310, Way Forward for [114][306] NR_duplex_evo_General